WK3500 Casio Keyboard Info

Delete

22/4/04

Hi, I saved several SMF files on the internal memory of my wk-3000 and a couple of them were in the wrong midi format, I can't find in the manual how to delete these or overwrite them.

You can delete SMF files from the Casio....using the 'Transpose' button, cursor down to 'DelSMF', answer 'yes' then right cursor to the one you want to remove. Then down cursor to do the deed.

20/5/04

To clear song memory:

1. Press the 'song memory' button...the display shows 'SG (0 thru 4) Play. Use the 'yes/no' buttons...to select which song (0 thru 4) you want to access.

2. Press the 'down cursor'...the display shows 'SGx SongDel?'

3. Press the 'yes' button...display shows 'SGx Sure?'...press 'yes' again. The display says 'please wait' then returns to the normal display. Note that if you record to a song that already has notes in it, you record to the next track (starting on track 11...thru track 16). Each time you stop...you advance one track on the next 'record'.

5/7/04

Track deletion

I am assumming that you are recording a 'song'....using the song memory button.

1. Press the song button...the display shows 'SGx Play' (the x being song 0 thru 4)

2. Select the song to delete..using the 'yes/no' buttons

3.Use the down cursor to select 'SGx Song Del?'...answer 'yes'

4. You get a chance to change your mind ...display..'Sure?'...answer 'yes'

To remove 1 track..at step 3..click another down cursor to 'SGx Tr. Del?'...and use the little blue channel button to choose the track you want cleared (11 thru 16)

28/7/04

How do you delete a file from keyboard?

To delete Selected items ...using the Casio internal program:

1. Press the 'Transpose' button.

2. Down cursor to the 'delSMF?' display. That will remove SMF files, by pressing 'yes' ...then using the 'yes/no' buttons to select the actual SMF to delete.

3. If you right cursor...you will see the display show 'DelRhy?'...another right cursor shows 'DelTone?' (this gives you access to delete the tones in memory slots 700 to 803....which are all the 'wave' sample tones). Again, you use the 'yes/no' buttons to select the thing to delete.

To wipe all the memory clean...press the 'Transpose' button...then, cursor down to
'InitMix?'. Right cursor to 'InitSys?'...press yes...and you will have cleaned the memory (factory reset...all settings cleared)

You can also use the Casio Data Management software to remove any file. 
1. Select the keyboard tab (right side)...and in the file type prompt (upper left side..the white box drop down)

2. Select the type of file to display. Press the 'refresh...blue circle arrow..right side'

3.Select the actual file to remove (by highlighting it)...and press the 'red X'

4. The Casio software will remove the file (If you are connected to the computer with midi cables)