WK3500 Casio Keyboard Info

MIDI CABLE/COMPUTER

30/10/03

MIDI Cable Setup

Usually each program needs to be set separately, as far as what driver device you are using with that program. There are two settings that must be set to allow things to work. You want to choose the 'in device' driver to let the Casio send information to the computer, and the 'out device' to let the computer send data to the Casio. Both directions must be enabled to let the Casio program work. The Casio software also has a place to set the drivers (under File/ Preference/ Midi ), just go thru the options to find your correct drivers that will enable the Casio program.  Remember... the midi 'out' cable from the computer connects to the midi 'in' plug on the Casio.

Ok, so you can play a midi file on the computer, and send the midi data to your Casio (by selecting the proper driver). You need to make sure you can play a note on your Casio, and have noteworthy record the note. That shows that you know the correct drivers to use for the Casio software ( it will be the same ones).The Casio software also has a place to set the drivers (under File/ Preference/ Midi ), just go thru the options to find your correct drivers that will enable the Casio program to be able to communicate with your computer.

To use the Casio software, you first need to have your computer screen display set to at least to 1024 X 768 ( to have the display show up correctly). Then select the file you want to transfer (by using the File/ preference/ directory) to point to where the file is located on your computer, and set the 'file type' on the computer side of the display to the type of file you are transferring ( like a user tone or user rhythm, etc.) Click on the blue arrow circles on the computer side.... and your file should show up. Then click on the blue arrows on the keyboard side, and the user memory slots will show up on the keyboard side. Then click on the file you are loading (to highlight it), and also click on the Casio memory slot you are loading to (you can choose any one that shows up on the list). Finally, click on the arrow that is pointing to the Casio side (in the middle of the screen).... and the file will load. The important thing is to select the correct type of file you are transferring.

1/2/04

When I try to convert files using the "CASIO SMF Converter Software" I choose the correct "in" and "out" ports and click "ok" when a box comes up saying "There is no Response from a keyboard". My keyboard is switched on and the cables are secured.

You may have your cables reversed. The Casio out connects to the Computer midi in cable. The next thing to check...do you have the drivers selected in the Casio program. This is done under the File/preference/midi.