.HM A 1 1 1 1 1 1

.H 1 "Appendix for as6809 Frankenstein Assembler"

.H 2 "Pseudo Operations"

.H 3 "Standard Pseudo Operation Mnemonics"

.VL 40 5 1

.LI "End"

END

.LI "File Inclusion"

INCL

INCLUDE

.LI "If"

IF

.LI "Else"

ELSE

.LI "End If"

ENDI

.LI "Equate"

EQU

.LI "Set"

SET

.LI "Org"

ORG

.LI "Reserve Memory"

RESERVE

RMB

.LI "Define Byte Data"

BYTE

DB

FCB

.LI "Define Word Data"

DW

FDB

WORD

.LI "Define String Data"

FCC

STRING

.LI "Define Character Set Translation"

CHARSET

.LI "Define Character Value"

CHARDEF

CHD

.LI "Use Character Translation"

CHARUSE

.LE

.H 2 "Instructions"

.H 3 "Instruction List"

.TS H

;

l l l.

Opcode	Syntax	Selection Criteria

.sp

.TH

.sp

ABX

.sp

ADCA	 '#' expr

ADCA	 expr	DIRECT

ADCA	 expr	EXTENDED

ADCA	 indexed

.sp

ADCB	 '#' expr

ADCB	 expr	DIRECT

ADCB	 expr	EXTENDED

ADCB	 indexed

.sp

ADDA	 '#' expr

ADDA	 expr	DIRECT

ADDA	 expr	EXTENDED

ADDA	 indexed

.sp

ADDB	 '#' expr

ADDB	 expr	DIRECT

ADDB	 expr	EXTENDED

ADDB	 indexed

.sp

ADDD	 '#' expr

ADDD	 expr	DIRECT

ADDD	 expr	EXTENDED

ADDD	 indexed

.sp

ANDA	 '#' expr

ANDA	 expr	DIRECT

ANDA	 expr	EXTENDED

ANDA	 indexed

.sp

ANDB	 '#' expr

ANDB	 expr	DIRECT

ANDB	 expr	EXTENDED

ANDB	 indexed

.sp

ANDCC	 '#' expr

.sp

ASL	 expr	DIRECT

ASL	 expr	EXTENDED

ASL	 indexed

.sp

ASLA

.sp

ASLB

.sp

ASR	 expr	DIRECT

ASR	 expr	EXTENDED

ASR	 indexed

.sp

ASRA

.sp

ASRB

.sp

BCC	 expr

.sp

BCS	 expr

.sp

BEQ	 expr

.sp

BGE	 expr

.sp

BGT	 expr

.sp

BHI	 expr

.sp

BHS	 expr

.sp

BITA	 '#' expr

BITA	 expr	DIRECT

BITA	 expr	EXTENDED

BITA	 indexed

.sp

BITB	 '#' expr

BITB	 expr	DIRECT

BITB	 expr	EXTENDED

BITB	 indexed

.sp

BLE	 expr

.sp

BLO	 expr

.sp

BLS	 expr

.sp

BLT	 expr

.sp

BMI	 expr

.sp

BNE	 expr

.sp

BPL	 expr

.sp

BRA	 expr

.sp

BRN	 expr

.sp

BSR	 expr

.sp

BVC	 expr

.sp

BVS	 expr

.sp

CLR	 expr	DIRECT

CLR	 expr	EXTENDED

CLR	 indexed

.sp

CLRA

.sp

CLRB

.sp

CMPA	 '#' expr

CMPA	 expr	DIRECT

CMPA	 expr	EXTENDED

CMPA	 indexed

.sp

CMPB	 '#' expr

CMPB	 expr	DIRECT

CMPB	 expr	EXTENDED

CMPB	 indexed

.sp

CMPD	 '#' expr

CMPD	 expr	DIRECT

CMPD	 expr	EXTENDED

CMPD	 indexed

.sp

CMPS	 '#' expr

CMPS	 expr	DIRECT

CMPS	 expr	EXTENDED

CMPS	 indexed

.sp

CMPU	 '#' expr

CMPU	 expr	DIRECT

CMPU	 expr	EXTENDED

CMPU	 indexed

.sp

CMPX	 '#' expr

CMPX	 expr	DIRECT

CMPX	 expr	EXTENDED

CMPX	 indexed

.sp

CMPY	 '#' expr

CMPY	 expr	DIRECT

CMPY	 expr	EXTENDED

CMPY	 indexed

.sp

COM	 expr	DIRECT

COM	 expr	EXTENDED

COM	 indexed

.sp

COMA

.sp

COMB

.sp

CWAI	 '#' expr

.sp

DAA

.sp

DEC	 expr	DIRECT

DEC	 expr	EXTENDED

DEC	 indexed

.sp

DECA

.sp

DECB

.sp

EORA	 '#' expr

EORA	 expr	DIRECT

EORA	 expr	EXTENDED

EORA	 indexed

.sp

EORB	 '#' expr

EORB	 expr	DIRECT

EORB	 expr	EXTENDED

EORB	 indexed

.sp

EXG	 register ',' register

.sp

INC	 expr	DIRECT

INC	 expr	EXTENDED

INC	 indexed

.sp

INCA

.sp

INCB

.sp

JMP	 expr	DIRECT

JMP	 expr	EXTENDED

JMP	 indexed

.sp

JSR	 expr	DIRECT

JSR	 expr	EXTENDED

JSR	 indexed

.sp

LBCC	 expr

.sp

LBCS	 expr

.sp

LBEQ	 expr

.sp

LBGE	 expr

.sp

LBGT	 expr

.sp

LBHI	 expr

.sp

LBHS	 expr

.sp

LBLE	 expr

.sp

LBLO	 expr

.sp

LBLS	 expr

.sp

LBLT	 expr

.sp

LBMI	 expr

.sp

LBNE	 expr

.sp

LBPL	 expr

.sp

LBRA	 expr

.sp

LBRN	 expr

.sp

LBSR	 expr

.sp

LBVC	 expr

.sp

LBVS	 expr

.sp

LDA	 '#' expr

LDA	 expr	DIRECT

LDA	 expr	EXTENDED

LDA	 indexed

.sp

LDB	 '#' expr

LDB	 expr	DIRECT

LDB	 expr	EXTENDED

LDB	 indexed

.sp

LDD	 '#' expr

LDD	 expr	DIRECT

LDD	 expr	EXTENDED

LDD	 indexed

.sp

LDS	 '#' expr

LDS	 expr	DIRECT

LDS	 expr	EXTENDED

LDS	 indexed

.sp

LDU	 '#' expr

LDU	 expr	DIRECT

LDU	 expr	EXTENDED

LDU	 indexed

.sp

LDX	 '#' expr

LDX	 expr	DIRECT

LDX	 expr	EXTENDED

LDX	 indexed

.sp

LDY	 '#' expr

LDY	 expr	DIRECT

LDY	 expr	EXTENDED

LDY	 indexed

.sp

LEAS	 indexed

.sp

LEAU	 indexed

.sp

LEAX	 indexed

.sp

LEAY	 indexed

.sp

LSL	 expr	DIRECT

LSL	 expr	EXTENDED

LSL	 indexed

.sp

LSLA

.sp

LSLB

.sp

LSR	 expr	DIRECT

LSR	 expr	EXTENDED

LSR	 indexed

.sp

LSRA

.sp

LSRB

.sp

MUL

.sp

NEG	 expr	DIRECT

NEG	 expr	EXTENDED

NEG	 indexed

.sp

NEGA

.sp

NEGB

.sp

NOP

.sp

ORA	 '#' expr

ORA	 expr	DIRECT

ORA	 expr	EXTENDED

ORA	 indexed

.sp

ORB	 '#' expr

ORB	 expr	DIRECT

ORB	 expr	EXTENDED

ORB	 indexed

.sp

ORCC	 '#' expr

.sp

PSHS	 register list

.sp

PSHU	 register list

.sp

PULS	 register list

.sp

PULU	 register list

.sp

ROL	 expr	DIRECT

ROL	 expr	EXTENDED

ROL	 indexed

.sp

ROLA

.sp

ROLB

.sp

ROR	 expr	DIRECT

ROR	 expr	EXTENDED

ROR	 indexed

.sp

RORA

.sp

RORB

.sp

RTI

.sp

RTS

.sp

SBCA	 '#' expr

SBCA	 expr	DIRECT

SBCA	 expr	EXTENDED

SBCA	 indexed

.sp

SBCB	 '#' expr

SBCB	 expr	DIRECT

SBCB	 expr	EXTENDED

SBCB	 indexed

.sp

SEX

.sp

STA	 expr	DIRECT

STA	 expr	EXTENDED

STA	 indexed

.sp

STB	 expr	DIRECT

STB	 expr	EXTENDED

STB	 indexed

.sp

STD	 expr	DIRECT

STD	 expr	EXTENDED

STD	 indexed

.sp

STS	 expr	DIRECT

STS	 expr	EXTENDED

STS	 indexed

.sp

STU	 expr	DIRECT

STU	 expr	EXTENDED

STU	 indexed

.sp

STX	 expr	DIRECT

STX	 expr	EXTENDED

STX	 indexed

.sp

STY	 expr	DIRECT

STY	 expr	EXTENDED

STY	 indexed

.sp

SUBA	 '#' expr

SUBA	 expr	DIRECT

SUBA	 expr	EXTENDED

SUBA	 indexed

.sp

SUBB	 '#' expr

SUBB	 expr	DIRECT

SUBB	 expr	EXTENDED

SUBB	 indexed

.sp

SUBD	 '#' expr

SUBD	 expr	DIRECT

SUBD	 expr	EXTENDED

SUBD	 indexed

.sp

SWI2

.sp

SWI3

.sp

SWI

.sp

SYNC

.sp

TFR	 register ',' register

.sp

TST	 expr	DIRECT

TST	 expr	EXTENDED

TST	 indexed

.sp

TSTA

.sp

TSTB

.TE

.H 3 "Operand Types"

.H 4 "register list"

The operands for the push and pull operations consist of a list of

one or more registers seperated by commas.

The stack pointer associated with an operator will give an error if

it occurs in the list.

The registers can be D, X, Y, U, S, PC, A, B, CC, DP and their lower case

equivalents.

For the push and pull operations, the D register is an alias for the

A and B registers, and will result in two selection bits set in the

postbyte.

.H 4 "indexed"

The indexed mode operands are in the forms.

.VL 25 5 1

.LI ", index"

.LI "expr , index"

.LI "accumulator , index"

.LI ", index +"

.LI ", index ++"

.LI ", - index"

.LI ", -- index"

.LI "expr , PCR"

.LI "[ , index ]"

.LI "[ expr , index ]"

.LI "[ accumulator , index ]"

.LI "[ , index ++ ]"

.LI "[ , -- index ]"

.LI "[ expr , PCR ]"

.LI "[ expr ]"

.LE

.P

Index can be any of X, Y, S, U (or their lower case equivalents).

Accumulator can be any of A, B, D (or their lower case equivalents).

.P

The short offset forms will only be used if the expression in the operand

has a value defined at the first pass, and the value is small enough to fit.

Expressions that have an as yet undefined value will use the 16 bit offset

form.

.H 3 "Selection Criteria Keywords"

.VL 25 5

.LI DIRECT

The instruction can use the direct addressing mode.

The expression must have a value between 0 and 255 that is defined when the

instruction is processed in the input pass.

.LI EXTENDED

The instruction can use the extended addressing mode.

.LE

.H 3 "Apostrophes"

The apostrophes in the syntax field are a notation used for the

parser generator and are not put in the assembler source statement.

.H 2 "Notes"

.H 3 "Reserved Symbols"

.H 4 "Machine Dependent Reserved Symbols"

A

B

CC

D

DP

PC

PCR

S

U

X

Y

a

b

cc

d

dp

pc

pcr

s

u

x

y

.H 4 "Standard Reserved Symbols"

AND

DEFINED

EQ

GE

GT

HIGH

LE

LOW

LT

MOD

NE

NOT

OR

SHL

SHR

XOR

and

defined

eq

ge

gt

high

le

low

lt

mod

ne

not

or

shl

shr

xor

.TC 1 1 7