K 0 it G

FROM THE INSIDE

A

PROGRAMMER'S

GUIDE	TO

THE

,TWENTY-THR E!

Most of the Ol /W owners that I've met use their instrument as a sonic workhorse, and team only enough basic features to get around. But there are a lot of cool tricks waiting for you under the Ol /W's hood. Hopefully the tips in this article will motivate you to

really tap into its power.

This clinic assumes that you have a general familiarity wish your instrument. Most of the items are self-explanatory, but if you're not clear about a certain area, there's always the owners manual. (You have read it, haven't you?) Note that the information presented here can be applied to all of the instruments in the Ol /W line.

OPERATIONAL TIPS

Get Up to Date. Turn on the 01 /W while holding down the Reset button to display your instrument's operating system version. (Look for it in the left-hand side of the LCD.) Versions #62 and higher add some important new features, including support for Standard MIDI Files, a sequencer track solo feature, and

MARCH 1994/KEYBOARD	81