edit to another location in memory, hit 9 on the keypad to go to the Write/Rename page.

THE SEQUENCER

Most on-board sequencers get panned before anyone has taken a good look to see just how powerful they actually are. The sequencer in the 01 /W has many of the same features found in today's software-based sequencers, and is definitely worth checking out.

Direct Data Entry. The techniques described above for data entry from the keypad also work with the sequencer. For example, select VOL in the Edit: field, cursor up to the Volume field for the track you wish to edit, and directly input values to your heart's desire. Remember, too, that when selecting the program for the track, repeatedly pressing

10's Hold/- cycles among the banks.

All About Panning. For some extra animation in a sequence, it's great to have a track or two moving around in the stereo field (though I admit I got a little carried away on the 01AN Dance PCM card sequences using this type of trick). The 0 1 /W sequencer can record MIDI controller commands for volume and panning in real time using either OVDB (overdub) or Loop Record mode. (Note that these controller messages cannot be recorded over MIDI, but must come from the O1/W itself using the data slider and the appropriate Edit: command.)

In the sequencer, panning moves show up as controller 10 data. Values between 000 and 100 determine the sound's left/right position in the stereo field. Values 110 through 115 assign the track's sound to output C; values 116 through 119 assign the sound to output D; values 120 through 124 assign the sound to outputs C+D; values 125 through 127 assign the sound to ALL.

Comparing Notes. When you record in OVWR (overwrite) mode, use the Compare button to toggle between the original and new takes. When the Compare light is lit, you are listening to the first take; when the light is off, you are listening to the most recent take. The same is true when you've edited

a track using Event Edit (page 5, Edit Song); use Compare to see if you like the results of your edit.

Recording Tempo Changes. It's easy to record tempo changes in real time. First, press the Rec/Write button. Next, change the field immediately to the right of the bpm display so that it reads REC (normally, this field is set to MAN). Cursor to the BPM field to highlight it, then start the sequence playing. As it plays, you can change the tempo using the data slider, the value up/down buttons, or directly from the keypad (don't forget to keep the A softkey pressed when entering values directly from the keypad).

You can also edit the tempo track in Event Edit. Select page 5, cursor to highlight Event Edit, then use the value up/down buttons or slider to select Tempo Track in the track selection field at the bottom of the page. (Tempo Track is the last option after track 16.) Next, press Rec/Write and Start/Stop simultaneously to enter Event Edit mode. To change the tempo, either edit an existing tempo value or press the [INSI softkey to add a new one.

While in Event Edit mode you can easily change the time signature as well. Simply highlight the Time Signature field and enter the desired value.

Sequencing

Performer

Vision

Cubase

Cakewalk

Metro Mastertracks

Pro

Computers

Notalor Logic Musicator Drummer

Are you confused about MIDI and music software?

4 5 4

MIDI Editors

Galaxy

Unisyn

EditOne

MAX	3

a

Digital Recording

Audiomedia II

Sound Tools ll

Session 8

NuMedia

Samplecell II

Digital

Performer

Cubase Audio

Studiovision

Deck

DINR

Infinity

Time Bandit

Turbosynth

Hyperprism

Notation		We aren't.		General MIDI

Nightingale	Educational		MIDI Interfaces	Emu SoundEngine

MIDISCAN	Song Works

Translator 	Roland SC7

Mosaic	Rhythm Ace			PC MIDI Card	Roland Rap IO

Musicshop	Play it by Ear			MIDI Time Piece	Roland SCCI

Music Printer Plus	Note Play			Studio 2-3-4-5	Roland SC33

~			_ 7 67 _ 6161

Finale	Listen			MIDI Express	General MIDI Files

Encore	Practica Musica			MQX	Band in a Box

Musictime	Music Lessons			Midiator	Jammer Pro

Allegro	Jazz Improvisation			Dual Port SE	Pianist

Guitarist

647 Mission St

Serving Performers, Educators,	San Francisco

Composers, Programmers,

and Sound Designers since 1982	CA 94105	Send for our 88page Catalos

84	KEYBOARD/MARCH 1994

Hip-Hop i tensity functu G) allowsyo retaining as li feel as desires hop groove i! Quantize Re quantizethe, set to a sixtee tization Inter

Simple Tr way to assign ranges, and ! use the Copy 9), which let! tion into the i fact, you can, - one for us the other for

16-Way h you're doing an external c MIDI chann rather than it can have the nets simultar>t to eight chap

Tuning p drum kits are range of t12