KORGLOAD.PRG INSTRUCTIONS  (Ver 1.0)

COMPUTER REQUIREMENT : Any Atari STE, STFM, STF or MEGA.

Mono(Hi Res) or Colour(Med Res) monitor.

Double sided disc drive only.

(Single sided version available on request).

TO RELOAD DATA

==============

1. Connect the MIDI OUT socket on the Atari ST to the MIDI IN

socket on the instrument.

2. Switch on instrument and set basic MIDI channel to 1

and ensure that SYS-EX RECEIVE is enabled.

3. Switch on Atari ST. If you are using either a colour monitor

or a colour T.V. set the Atari ST to Med resolution mode.

4. Load KORGLOAD.PRG by double clicking on its icon.

5. When loaded the program will display a Dialogue box.

6. Click on load to display the GEM file selector.

7. Load the desired SYS-EX file by double clicking on it.

M1PRELD .BNK - M1 Factory preload (All data)

M1PRGCOM.BNK - M1 Factory preload (Prog/Combi and Global only)

M1REXFPD.BNK - M1REX Factory preload (All data)

M1REXPCG.BNK - M1REX Factory preload (Prog/Combi and Global only)

8. To download the data to the instrument click on transfer

and wait while the MIDI SYS-EX data is transferred.

9. If all goes well click on the EXIT button to return to the

desktop.

10. If you are experiencing a problem try clicking on MIDI TEST.

11. When the MIDI TEST dialogue is displayed click on TEST.

This sends 2 notes a major 3rd apart on channel 1, and 2 notes

a minor 2nd apart on channels 2 to 16.

12. You can also click on the channel numbers to momentarily

output a middle C on that channel.