******************************

SENDI3/RECVI3 v1.7

(DOS with MPU, SOUNDBLASTER, Windows 95)

send and receive korg i2/i3 dumps

by Guenter Nagler

1996/1997

(gnagler@iicm.tu-graz.ac.at)

******************************

[1] BACKGROUND

KORG i2/i3 keyboards store can their data on 720KB floppy disks in

files with extensions

*.pcg  program data (64 programs + 2 drum kits)

*.sty  style data  (4 styles)

*.arr  arrangement data (64 programs)

*.sng  song data (10 songs)

*.bsq  backing sequence data (10 backing sequences)

I defined additional data files:

*.glb  global data

*.dkt  drumkit data

The keyboard supports data dump exchange via midi cable.

These utilities communicate with the keyboard and send and

request/receive such dumps.

send:

The sendi3 utility reads korg files and convert them to korg dumps and send

them to the keyboard. After sending the keyboard usually sends

back a reply (data load completed or data format error).

The sendi3 utility tries to catch these message.

receive:

The recvi3 utility listens to the keyboard and requests a dump. Then it

waits for reaction of the keyboard which should send the dump immediately.

The utility catches the dump decodes it and converts it into a korg file.

[2] FILES DESCRIPTION

SENDI3.EXE..........MSDOS executable for sending korg i3 files

RECVI3.EXE..........MSDOS executable for receiving korg i3 files

SENDI3W.EXE.........Windows95 executable for sending korg i3 files

RECVI3W.EXE.........Windows95 executable for receiving korg i3 files

MIDIDLL.DLL.........Windows95 dynamic library for midi communication

EXCHI3.DOC..........this file, showing usage of SENDI3.EXE and RECVI3.EXE

MIDIDLL.DLL should be moved to directory: windows\system

MIDIDLL.DLL is necessary to run the Windows95 executables

[3] COPYRIGHT

SENDI3 and RECVI3 (c) 1996 were created by Guenter Nagler.

SENDI3W and RECVI3W (c) 1997 were created by Guenter Nagler.

These programs are and may be used as you wish with this one exception:

You may NOT charge any fee or derive any profit for distribution

of SENDI3 and RECVI3.  Thus, you may NOT sell or bundle SENDI3/RECVI3

with any product in a retail environment (shareware disk distribution,

CD-ROM, etc.) without permission of the author.

You may give SENDI3 and RECVI3 to your friends, upload it to a BBS, or ftp it to

another internet site, as long as you don't charge anything for it.

[4] DISCLAIMER

SENDI3 and RECVI3 were designed to read and write original KORG i3 files.

Communications are done using original Roland MPU-401 (tm) and

Creative Labs Soundblaster (tm) midi interfaces or a Windows 95 (tm)

compatible midi interface (mmsystem).

Use SENDI3 at your own risk.  Anything you do with SENDI3 is your

responsibility, and not the author's.  Any damage caused to any person,

computer, software, hardware, company, or business by running SENDI3

is your responsibility, and the author will not be liable.

If you don't understand these terms, or are not sure of something, or

are afraid something bad might come of using SENDI3, don't  use  it!

You are here forewarned.

[5] INSTALLATION

[DOS]

Simply copy SENDI3.EXE and RECVI3.EXE into a directory that is in your path.

C:\> SENDI3

C:\> RECVI3 -h

[WINDOWS 95]

Simply copy SENDI3W.EXE and RECVI3W.EXE into a directory that is in your

Windows 95 path.

MIDIDLL.DLL should be moved to directory: windows\system

C:\> SENDI3W

C:\> RECVI3W -h

in both versions you should get the usage text (see next section)

[6] USAGE

usage is same for MSDOS and Windows95 versions of the programs (use

the program with same commandline).

Under Windows95 it is best to start an Msdos shell and then can use

the windows95 or/and the msdos versions. It is also possible to start

them with run command.

usage: sendi3 [-prog xxx] filename ...

files are korg i3 files (pcg, sty, arr, bsq, sng) or glb, drk

prog xxx  send only program xxx of given pcg

pcg	korg i3 programs

sty	korg i3 styles

arr	korg i3 arrangements

bsq	korg i3 backing sequences

sng	korg i3 songs

glb	korg i3 global settings

drk	korg i3 drumkit parameters

dos wildcards (*, ?) can be used

usage: recvi3 [-identify] [filename]

-identify	request identification of KORG i3 keyboard

filename	should have extension *,PCG,STY,ARR,SNG,BSQ,GLB,DKT

When no filename specified then dump must be sent manually

Important:

To exchange data dumps with the keyboard the keyboard must be turned on

and global data parameter EXCL set to ON.

Important: (MSDOS)

This program works only with a MPU-401 or Soundblaster compatible

midi interface. For soundblaster cards the environment variable BLASTER

must be set to select the correct soundcard port number (A220,A240,A260,A280).

The MPU midi port must be 300 or 330 or specified in the environment variable

MPU (hexadezimal port number).

e.g.

set BLASTER=A240 I5 D0 H5 P300 T6

set MPU=210

Important:  (Windows 95)

By default this program only uses default midi devices (#0) for input/output.

If you want to use the programs with other input/output devices then

you can set environment variables WINMIDIIN, WINMIDIOUT with other device numbers:

e.g.

set WINMIDIIN=1

set WINMIDIOUT=2

Warning:

sendi3 will change most settings of the keyboard.

Backup your current settings to a floppy disk (Disk - Save all data),

so that you can restore this state.

Warning:

korg i3 data dumps do not allow to receive all parameters that are

stored in an original PCG file (e.g. Protect Program, EXCL ON/OFF, ...)

recvi3 will save a PCG that contains only program data.

This should not be a big disadvantage because usually the missing settings

are set manually only once and should stay forever.

Warning:

This program can only transfer files using global channel 1.

Korg i3 global channel also needs to be set to 1 (factory default).

Warning: (only Windows95 version)

Always change to the directory containing the korg files or

use full filename path  (e.g. c:\windows\korg\styles\macarena.* )

Warning: (only Windows95 version)

recvi3w gets all data dump from korg and then saves individual parts.

While windows is transfering the data dump the program gets no

notification and therefore can't show how far it is already.

Hint:

The Windows95 dump transfer seems to be quicker than the MSDOS transfer.

Hint:

Press RESET button on the keyboard that all settings are really applied.

[7] SUGGESTIONS / COMMENTS / BUG REPORTS / QUESTIONS

WWW:    http://hgiicm.tu-graz.ac.at/Cpub

contains all my dos/unix midi programs

EMAIL:  gnagler@iicm.tu-graz.ac.at

[8] EXAMPLES

Use sendi3w instead of sendi3 for Windows95 version.

Use recvi3w instead of recvi3 for Windows95 version.

First backup your current keyboard settings manually to a floppy disk!

Ask KORG i2/i3 to identify itself:

C:\> recvi3 -identify

KORG responded

15 bytes received: identification:  F0 7E 00 06 02 42 39 00 00 00 17 00 01 00 F7

Manufacturer id: 42 Korg

Family code: 39h  i2/i3

Member code: 00h  i3

ROM No: 00h (0)

Software Version: 17h (23)

Get all current settings (all that Korg i2/i3 can send through dump):

C:\> recvi3 backup.*

KORG responded

requesting dump from KORG

saved global data to BACKUP.GLB

saved drumkit data to BACKUP.DKT

saved program data to BACKUP.PRG

saved style data to BACKUP.STY

saved arrangement data to BACKUP.ARR

saved backing sequence data to BACKUP.BSQ

saved song sequence data to BACKUP.BSQ

Receive an "all program dump" that will be sent manually from the keyboard:

C:\> recvi3 manually

KORG responded.

Waiting for receiving KORG dumps. Abort with <esc>

7 bytes received: mode changed to GLOBAL

11 bytes received: drumkit parameter changed in Dr1 Param 0 to Value 0

11 bytes received: drumkit parameter changed in Dr2 Param 0 to Value 0

12377 bytes received: all program data dump

saved program data to MANUALLY.PRG

This will save all dumps sent manually and shows all EXCL messages sent by

the keyboard.

Load a full KORG environment:

C:\> sendi3 macarena.*

KORG responded

file MACARENA.TXT ignored

sending MACARENA.PCG to KORG ...

korg answered: data load completed

sending MACARENA.STY to KORG ...

korg answered: data load completed

sending MACARENA.ARR to KORG ...

korg answered: data load completed

sending MACARENA.BSQ to KORG ...

korg answered: data load completed

set new programs only

C:\> sendi3 8_beat.pcg

KORG responded

sending 8_BEAT.PCG to KORG ...

korg answered: data load completed

Hint: The programs will be available at Program Bank D and drumkits

at user drumkits Drum7 and Drum8

More than one file per type cannot be sent:

C:\> sendi3 8*.*

cannot send more than one arrangement data file (80THGOLD.ARR,8_BEAT.ARR)

Send single program and save it in a certain user program

goto program mode and select a user program place that will be used

Hint: The program place will be used temporarily and will not be saved

automatically. You can use any place to test a program and save it with

Write Program Key.

Use findi3 to find the program wanted (*.pcg).

You need the pcg filename and program place (D11-D88,Dr7,Dr8) or program name.

C:> findi3 mini

DoubleMini   D37  PRELOAD.PCG

C:\> sendi3 -prog D37 preload.pcg

or

C:\> sendi3 -prog doublemini preload.pcg

Hint: If the program name contains spaces or other not DOS compatible characters

then use the program place instead or use "..."

The program will change to program edit mode and transfer the program.

You can play it immediately after successful notification.

The program will be forgotten if any other program is selected.

Use Write Program Key or Write Program function (edit program/utility)

to save the program.

[9] CHANGES

v1.0 to v1.1:

* allow MPU port other than 300 or 330 (environment variable MPU)

v1.1 to v1.2:

* better MPU support

* option -prog in sendi3 to send single programs

v1.2 to v1.3:

* identifying iX300

v1.4 to v1.5:

* windows version added sendi3w, recvi3w

user interface remained same (commandline)

v1.5 to v1.6:

* sending user drumkits from in .pcg

(successful decoded tune into dump format)

v1.6 to v1.7:

* receiving global data and user drumkits and encoding them into .pcg

v1.7 to v1.8:

* added serial port midi interface at COM1

environment variable COM must be set to 1,38400,n,8,1 to activate