i30 MIDI IMPLEMENTATION

1.Transmitted Data

1-1.Channel Messages

+-----------------------------------------------------------------------------------------------------------------+

| Status         | Second             | Third          | Description [i30]                                    |   |

|   [Bin(Hex)]   |   [Bin(Hex,Dec)]   |   [Bin(Hex)]   |                                                      |   |

|-----------------------------------------------------------------------------------------------------------------|

| 1000 nnnn (8n) | 0kkk kkkk (kk,kkk) | 0100 0000 (40) | Note Off                 (*1)                        | A |

|                |                    |                | k=001~127   [61Keys+Octave+Transpose] (Seq:000~127)  |   |

| 1001 nnnn (9n) | 0kkk kkkk (kk,kkk) | 0VVV VVVV (VV) | Note On                  (*1)                        | A |

|                |                    |                | k=001~127   [61Keys+Octave+Transpose] (Seq:000~127)  |   |

|                |                    |                | V=001~127   [Velocity] (Seq:002~126 even number)     |   |

| 1010 nnnn (An) | 0kkk kkkk (kk,kkk) | 0vvv vvvv (vv) | Poly Key Pressure[Recorded Sequence Data]            |T,Q|

| 1011 nnnn (Bn) | 0000 0000 (00,000) | 0MMM MMMM (MM) | Bank Selecet MSB [Program Bank Key,etc] (*2)         | B |

| 1011 nnnn (Bn) | 0010 0000 (20,032) | 0LLL LLLL (LL) | Bank Selecet LSB [Program Bank Key,etc] (*2)         | B |

| 1011 nnnn (Bn) | 0000 0001 (01,001) | 0vvv vvvv (vv) | Modulation 1     [Joystick (+Y)]                     | C |

| 1011 nnnn (Bn) | 0000 0010 (02,002) | 0vvv vvvv (vv) | Modulation 2     [Joystick (-Y)]                     | C |

| 1011 nnnn (Bn) | 0000 0100 (04,004) | 0111 1111 (7F) | Foot Controller  [KBD(Special)Scale On]              | C |

|                |                    | 0000 0000 (00) | Foot Controller  [KBD(Special)Scale Off=Normal Scale]| C |

| 1011 nnnn (Bn) | 0000 0111 (07,007) | 0vvv vvvv (vv) | Volume           [Volume]                            | C |

| 1011 nnnn (Bn) | 0000 1010 (0A,010) | 0vvv vvvv (vv) | Panpot           [Pan]                               | C |

| 1011 nnnn (Bn) | 0000 1011 (0B,011) | 0vvv vvvv (vv) | Expression       [Assignable Pedal:KBD Expression]   | C |

| 1011 nnnn (Bn) | 0000 1100 (0C,012) | 0vvv vvvv (vv) | Effect Control 1 [Assign:FX Control 1(CC#12)]        | C |

| 1011 nnnn (Bn) | 0000 1101 (0D,013) | 0vvv vvvv (vv) | Effect Control 2 [Assign:FX Control 2(CC#13)]        | C |

| 1011 nnnn (Bn) | 0100 0000 (40,064) | 0111 1111 (7F) | Damper Pedal On  [Damper Pedal]                      | C |

|                |                    | 0000 0000 (00) | Damper Pedal Off [Damper Pedal]                      | C |

| 1011 nnnn (Bn) | 0100 1010 (4A,074) | 0vvv vvvv (vv) | Brightness       [Assignable Pedal:VDF Cutoff]       | C |

| 1011 nnnn (Bn) | 0101 1011 (5B,091) | 0vvv vvvv (vv) | Effect 1 Depth   [C Send Level]                      | C |

| 1011 nnnn (Bn) | 0101 1101 (5D,093) | 0vvv vvvv (vv) | Effect 3 Depth   [D Send Level]                      | C |

| 1011 nnnn (Bn) | 0ccc cccc (cc,ccc) | 0vvv vvvv (vv) | Control Change   [Recorded Sequence Data]ccc=000~127 |C,Q|

| 1100 nnnn (Cn) | 0ppp pppp (pp,ppp) | ---- ----      | Program Change   [Program Number]                    | P |

| 1101 nnnn (Dn) | 0vvv vvvv (vv,vvv) | ---- ----      | Channel Pressure [Aftertouch]                        | T |

| 1110 nnnn (En) | 0bbb bbbb (bb,bbb) | 0bbb bbbb (bb) | Pitch Bend       [Joystcik (X)]                      | C |

+-----------------------------------------------------------------------------------------------------------------+

nnnn:MIDI Channel Number 0~15 [1~16],Program Mode - Global Channel

VVVV:Velocity

vvvv:Value  (000~127)

A :Always Enabled

C :Enabled when "Enable Control Change" in Global Mode is Checked

P :Enabled when "Enable Program Change" in Global Mode is Checked

B :Enabled when "Enable Bank Change" in Global Mode is Checked

T :Enabled when "Enable Aftertouch" in Global Mode is Checked

Q :Enabled when Sequencer is Playing(Transmit),Recording(Receive)

(*1)----------------------------------+

| Key=[Hex,Dec] |                     |

|-------------------------------------|

| G 9 = 7F,127  | Oct.+2 Transpose+7  |

| C 9 = 78,120  | Octave +2           |

| C 8 = 6C,108  | Octave +1           |

| C 7 = 60,096  | 61Key               |

| C 6 = 54,084  |   |                 |

| C 5 = 48,072  |   |                 |

| C 4 = 3C,060  |   |                 |

| C 3 = 30,048  |   |                 |

| C 2 = 24,036  | 61Key               |

| C 1 = 18,024  | Octave -1           |

| C 0 = 0C,012  | Octave -2           |

| C#-1= 01,001  | Oct.-2 Transpose-11 |

| C-1 = 00,000  | Seq. Recorded Data  |

+-------------------------------------+

(*2)----------------------------------------------------+

|i30    :               MIDI Out[Hex(Dec)]              |

|-------------------------------------------------------|

| A11~A88: MM , LL , pp =38(056),00(000),00~3F(000~063) |

| (When Discard KORG GM)=00(000),00(000),00~3F(000~063) |

| B11~B88: MM , LL , pp =38(056),00(000),40~7F(064~127) |

| (When Discard KORG GM)=00(000),00(000),40~7F(064~127) |

| C11~C88: MM , LL , pp =00(000),01(001),00~3F(000~063) |

| D11~D88: MM , LL , pp =00(000),01(001),40~7F(064~127) |

| E11~E88: MM , LL , pp =00(000),02(002),00~3F(000~063) |

| F11~F88: MM , LL , pp =00(000),03(002),00~3F(000~063) |

| G11~G88: MM , LL , pp =00(000),03(002),40~7F(064~127) |

| R Bank : MM , LL , pp =3E(062),00(000),pp             |

| (When Discard KORG GM)=00(000),04(004),pp             |+-------------------------------------------------------+

| R11    :                              ,00(000)        || R35    :                              ,64(100)        |

| R12    :                              ,01(001)        || R36    :                              ,65(101)        |

| R13    :                              ,08(008)        || R37    :                              ,66(102)        |

| R14    :                              ,10(016)        || R38    :                              ,67(103)        |

| R15    :                              ,18(024)        || R41    :                              ,68(104)        |

| R16    :                              ,19(025)        || R42    :                              ,69(105)        |

| R17    :                              ,1A(026)        || R43    :                              ,6A(106)        |

| R18    :                              ,20(032)        || R44    :                              ,6B(107)        |

| R21    :                              ,28(040)        || R45    :                              ,6C(108)        |

| R22    :                              ,30(048)        || R46    :                              ,6D(109)        |

| R23    :                              ,40(064)        || R47    :                              ,6E(110)        |

| R24    :                              ,41(065)        || R48    :                              ,6F(111)        |

| R25    :                              ,74(116)        || R51    :                              ,48(072)        |

| R26    :                              ,38(056)        || R52    :                              ,49(073)        |

| R27    :                              ,5E(094)        || R53    :                              ,4A(074)        |

| R28    :                              ,5F(095)        || R54    :                              ,4B(075)        |

| R31    :                              ,60(096)        || R55    :                              ,4C(076)        |

| R32    :                              ,61(097)        || R56    :                              ,4D(077)        |

| R33    :                              ,62(098)        || R57    :                              ,4E(078)        |

| R34    :                              ,63(099)        || R58    :                              ,4F(079)        |

+-------------------------------------------------------++-------------------------------------------------------+

1-2.System Common Messages

+--------------------------------------------------------------------------------------------+

| Status         | Second         | Third          | Description                             |

|   [Bin(Hex)]   |   [Bin(Hex)]   |   [Bin(Hex)]   |                                         |

|--------------------------------------------------------------------------------------------|

| 1111 0010 (F2) | 0sss ssss (ss) | 0ttt tttt (tt) | Song Position Pointer                   |

|                |                |                |     sss ssss : Least significant (LSB)  |

|                |                |                |     ttt tttt : Most significant (MSB)   |

| 1111 0011 (F3) | 0000 0000 (00) | 0000 ssss (ss) | Song Select                             |

|                |                |                |          ssss:Song Number = 0~9         |

+--------------------------------------------------------------------------------------------+

Transmited when in Song mode (Internal Clock)

1-3.System Realtime Messages

+----------------------------------------------------------+

| Status         | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 1000 (F8) | Timing Clock                            |

| 1111 1010 (FA) | Start                                   |

| 1111 1011 (FB) | Continue                                |

| 1111 1100 (FC) | Stop                                    |

| 1111 1110 (FE) | Active Sensing                          |

+----------------------------------------------------------+

Transmited when in ARR PLAY/BSEQ/SONG/SONG PLAY/E.STYLE mode when "Clock Source" is Internal.

1-4.Universal System Exclusive Messages (Device Inquiry Reply)

+----------------------------------------------------------+

| Byte           | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 0000 (F0) | Exclusive Status                        |

| 0111 1110 (7E) | Non Realtime Message                    |

| 0000 gggg (0g) | Global MIDI Channel                     |

| 0000 0110 (06) | Inquiry Message                         |

| 0000 0010 (02) | Identity Reply                          |

| 0100 0010 (42) | KORG ID                                 |

| 0100 1001 (49) | i30 (Family Code LSB)                   |

| 0000 0000 (00) | i30 (Family Code MSB)                   |

| 0000 0000 (00) | MemberCode (LSB)                        |

| 0000 0000 (00) | MemberCode (MSB)                        |

| 0??? ???? (??) | Minor Version (LSB)                     |

| 0??? ???? (??) | Minor Version (MSB)                     |

| 0??? ???? (??) | Major Version (LSB)                     |

| 0??? ???? (??) | Major Version (MSB)                     |

| 1111 0111 (F7) | End of Exclusive                        |

+----------------------------------------------------------+

1-5.Structure of KORG System Exclusive Messages

1st Byte = 1111 0000 (F0) : Exclusive Status

2nd Byte = 0100 0010 (42) : KORG

3rd Byte = 0011 gggg (3g) : g:Global Ch.

4th Byte = 0100 1001 (49) : i30

5th Byte = 0fff ffff (ff) : Function Code

6th Byte = 0ddd dddd (dd) : Data

|

LastByte = 1111 0111 (F7) : End of Exclusive

1-6.KORG System Exclusive Message Transmitted Function Code List

+----------------------------------------------------------+

| 5th Byte       | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 0100 0010 (42) | Mode Data                           | R |

| 0100 1110 (4E) | Mode Change                         | C |

| 0100 0000 (40) | Current Program Parameter Dump      | R |

| 0101 0010 (52) | Current Drum Program Parameter Dump | R |

| 0110 0100 (64) | ARR(All Arrangement) Dump           |R,D|

| 0110 1000 (68) | ARG(Arrangement Global) Dump        |R,D|

| 0110 1001 (69) | ARK(All KBD Set) Dump               |R,D|

| 0100 1100 (4C) | PRG(All Program) Dump               |R,D|

| 0101 0001 (51) | GBL(Global) Dump                    |R,D|

| 0110 0101 (65) | STY(Style Block) Dump               |R,D|

| 0110 0110 (66) | BSQ(All Backing Sequence) Dump      |R,D|

| 0100 1000 (48) | SNG(All Song) Dump                  |R,D|

| 0010 0101 (26) | Received Message Format Error       |R,E|

| 0010 0001 (21) | Write Completed                     | E |

| 0010 0010 (22) | Write Error                         | E |

| 0010 0011 (23) | Data Load Completed(ACK)            | E |

| 0010 0100 (24) | Data Load Error(NAK)                | E |

| 0110 0111 (67) | Chord                               |   |

+----------------------------------------------------------+

Transmitted when

R : Request message is received

C : Mode is Changed by switch

D : Data dump from Global mode

E : Exclusive message is received

2.Recognized Receive Data

2-1.Channel Messages

+-----------------------------------------------------------------------------------------------------------------+

| Status         | Second             | Third          | Description [i30]                                    |   |

|   [Bin(Hex)]   |   [Bin(Hex,Dec)]   |   [Bin(Hex)]   |                                                      |   |

|-----------------------------------------------------------------------------------------------------------------|

| 1000 nnnn (8n) | 0kkk kkkk (kk,kkk) | 0100 0000 (40) | Note Off                 (*1)                        | A |

| 1001 nnnn (9n) | 0kkk kkkk (kk,kkk) | 0000 0000 (00) | Note Off                 (*1)                        | A |

| 1001 nnnn (9n) | 0kkk kkkk (kk,kkk) | 0VVV VVVV (VV) | Note On                  (*1)                        | A |

|                |                    |                |  [Velocity]v=001~127(Seq.:002~126 even number)       |   |

| 1010 nnnn (An) | 0kkk kkkk (kk,kkk) | 0vvv vvvv (vv) | Poly Key Pressure[Recorded Sequence Data]            |T,Q|

| 1011 nnnn (Bn) | 0000 0000 (00,000) | 0MMM MMMM (MM) | Bank Selecet MSB [for Program/Arrangement Change](*2)| B |

| 1011 nnnn (Bn) | 0010 0000 (20,032) | 0LLL LLLL (LL) | Bank Selecet LSB [for Program/Arrangement Change](*2)| B |

| 1011 nnnn (Bn) | 0000 0001 (01,001) | 0vvv vvvv (vv) | Modulation 1     [Joystick (+Y)]                     | C |

| 1011 nnnn (Bn) | 0000 0010 (02,002) | 0vvv vvvv (vv) | Modulation 2     [Joystick (-Y)]                     | C |

| 1011 nnnn (Bn) | 0000 0100 (04,004) | 01xx xxxx (xx) | Foot Controller  [KBD(Special)Scale On]   xx=40~7F   | C |

|                |                    | 00xx xxxx (xx) | Foot Controller  [KBD(Special)Scale Off=Normal Scale]| C |

| 1011 nnnn (Bn) | 0000 0110 (06,006) | 0vvv vvvv (vv) | Data Entry MSB   [for RPN Parameter Edit](*3)        | C |

| 1011 nnnn (Bn) | 0010 0110 (26,038) | 0vvv vvvv (vv) | Data Entry LSB   [for RPN Parameter Edit](*3)        | C |

| 1011 nnnn (Bn) | 0000 0111 (07,007) | 0vvv vvvv (vv) | Volume           [Volume]                            | C |

| 1011 nnnn (Bn) | 0000 1010 (0A,010) | 0vvv vvvv (vv) | Panpot           [Pan]                               | C |

| 1011 nnnn (Bn) | 0000 1011 (0B,011) | 0vvv vvvv (vv) | Expression        [Expression (invisible)]           | C |

| 1011 gggg (Bg) | 0000 1100 (0C,012) | 0vvv vvvv (vv) | Effect Control 1  [Dynamic Modulation Source=CC#12]  | C |

| 1011 gggg (Bg) | 0000 1101 (0D,013) | 0vvv vvvv (vv) | Effect Control 2  [Dynamic Modulation Source=CC#13]  | C |

| 1011 nnnn (Bn) | 0100 0000 (40,064) | 01xx xxxx (xx) | Damper Pedal On   [Damper Pedal]          xx=40~7F   | C |

|                |                    | 00xx xxxx (xx) | Damper Pedal Off  [Damper Pedal]          xx=00~3F   | C |

| 1011 nnnn (Bn) | 0100 1000 (48,072) | 0vvv vvvv (vv) | Release Time      [Release time (invisible)]   (*4)  | C |

| 1011 nnnn (Bn) | 0100 1001 (49,073) | 0vvv vvvv (vv) | Attack Time       [Attack time (invisible)]    (*4)  | C |

| 1011 nnnn (Bn) | 0100 1010 (4A,074) | 0vvv vvvv (vv) | Brightness        [Brightness (invisible)]     (*4)  | C |

| 1011 nnnn (Bn) | 0101 1011 (5B,091) | 0vvv vvvv (vv) | Effect 1 Depth    [C Send Level]                     | C |

| 1011 nnnn (Bg) | 0101 1100 (5C,092) | 0xxx xxxx (xx) | Effect 2 Depth    [FX 1 (ACC FX 1)On]     xx=01~7F   | C |

|                |                    | 0000 0000 (00) | Effect 2 Depth    [FX 1 (ACC FX 1)Off]               | C |

| 1011 nnnn (Bn) | 0101 1101 (5D,093) | 0vvv vvvv (vv) | Effect 3 Depth    [D Send Level]                     | C |

| 1011 nnnn (Bg) | 0101 1110 (5E,094) | 0xxx xxxx (xx) | Effect 4 Depth    [FX 2 (ACC FX 2)On]     xx=01~7F   | C |

|                |                    | 0000 0000 (00) | Effect 4 Depth    [FX 2 (ACC FX 2)Off]               | C |

| 1011 nnnn (Bn) | 0110 0100 (64,100) | 0000 00rr (rr) | RPN Parameter Number(LSB)(*3)                        | C |

| 1011 nnnn (Bn) | 0110 0101 (65,101) | 0000 0000 (00) | RPN Parameter Number(MSB)(*3)                        | C |

| 1011 nnnn (Bn) | 0111 1000 (78,120) | 0000 0000 (00) | All Sound Off                                        | C |

| 1011 nnnn (Bn) | 0111 1001 (79,121) | 0000 0000 (00) | Reset All Controllers                                | C |

| 1011 nnnn (Bn) | 0ccc cccc (cc,ccc) | 0vvv vvvv (vv) | Control Data [for Seq.Recording]   ccc=000~127       | C |

| 1011 nnnn (Bg) | 0111 1010 (7A,122) | 0111 1111 (7F) | Local Control On                                     | A |

|                |                    | 0000 0000 (00) | Local Control Off                                    |   |

| 1011 nnnn (Bn) | 0111 1011 (7B,123) | 0000 0000 (00) | All Notes Off                                        | A |

| 1011 nnnn (Bn) | 0111 1100 (7C,124) | 0000 0000 (00) | Omni Mode Off     [All Notes Off]                    | A |

| 1011 nnnn (Bn) | 0111 1101 (7D,125) | 0000 0000 (00) | Omni Mode On      [All Notes Off]                    | A |

| 1011 nnnn (Bn) | 0111 1110 (7E,126) | 000m mmmm (mm) | Mono Mode On      [All Notes Off] mm=00~10           | A |

| 1011 nnnn (Bn) | 0111 1111 (7F,127) | 0000 0000 (00) | Poly Mode On      [All Notes Off]                    | A |

| 1100 nnnn (Cn) | 0ppp pppp (pp,ppp) | ---- ----      | Program Change    [Program/Arrangement Number](*2)   | P |

| 1101 nnnn (Dn) | 0vvv vvvv (vv,vvv) | ---- ----      | Channel Pressure  [Aftertouch]                       | T |

| 1110 nnnn (En) | 0bbb bbbb (bb,bbb) | 0bbb bbbb (bb) | Pitch Bend        [Joystcik (X)]                     | C |

+-----------------------------------------------------------------------------------------------------------------+

nnnn:MIDI Channel Number 0-15 [1~16],Program Mode - Global Channel

gggg:Always Global Channel

vvvv:Value      x:Don't care

A :Always Enabled

C :Enabled when "Enable Control Change" in Global Mode is Checked

P :Enabled when "Enable Program Chanbe" in Global Mode is Checked

B :Enabled when "Enable Bank Chanbe" in Global Mode is Checked

T :Enabled when "Enable Aftertouch" in Global Mode is Checked

Q :Enabled when Sequencer is Playing(Transmit),Recording(Receive)

(*1)------------+

| Key=[Hex,Dec] |                          (*2)-----------------------------------------------------+

|---------------|                          |  MIDI IN      [Dec(Hex)]                      :i30     |

| G 9 = 7F,127  |                          |--------------------------------------------------------|

| C 9 = 78,120  |                          | MM , LL , pp =000(00),000(00),000~063(00~3F) : A11~A88 |

| C 8 = 6C,108  |                          |              =056(38),000(00),000~063(00~3F)           |

| C 7 = 60,096  |                          | MM , LL , pp =000(00),000(00),064~127(40~7F) : B11~B88 |

| C 6 = 54,084  |                          |              =056(38),000(00),064~127(40~7F)           |

| C 5 = 48,072  |                          | MM , LL , pp =000(00),001(01),000~063(00~3F) : C11~C88 |

| C 4 = 3C,060  |                          | MM , LL , pp =000(00),001(01),064~127(40~7F) : D11~D88 |

| C 3 = 30,048  |                          | MM , LL , pp =000(00),002(02),000~063(00~3F) : E11~E88 |

| C 2 = 24,036  |                          | MM , LL , pp =000(00),003(03),000~063(00~3F) : F11~F88 |

| C 1 = 18,024  |                          | MM , LL , pp =000(00),003(03),064~127(40~7F) : G11~G88 |

| C 0 = 0C,012  |                          | MM , LL , pp =000(00),004(04),ppp(pp)        : R Bank  |

| C#-1= 01,001  |                          |              =062(3E),000(00),ppp(pp)                  |

| C-1 = 00,000  |                          |  (GM Kit 1)                  ,000    (00)    : R11     |

+---------------+                          |  (GM Kit 2)                  ,001    (01)    : R12     |

|                              ,002~007(02~07) : R11     |

(*3)--------------------------------------+|  (Room Kit)                  ,008~015(08~0F) : R13     |

|         RPN                             ||  (Power Kit)                 ,016~023(10~17) : R14     |

|-----------------------------------------||  (Electronic Kit)            ,024    (18)    : R15     |

| PitchBend Sens. [PitchBend Range]       ||  (Analog Kit)                ,025    (19)    : R16     |

|   CC#101=000 (Bn 65 00)                 ||  (Dance Kit)                 ,026    (1A)    : R17     |

|   CC#100=000 (Bn 64 00)                 ||                              ,027~031(1B~1F) : R15     |

|   CC#006=000~127 (Bn 06 00~7F)          ||  (Jazz Kit)                  ,032~039(20~27) : R18     |

|     (*net=000~012 : +00~+12)            ||  (Brush Kit)                 ,040~047(28~2F) : R21     |

|   CC#038=ignore                         ||  (Orchestra Kit)             ,048~055(30~37) : R22     |

|-----------------------------------------||  (SFX Kit)                   ,056~063(38~3F) : R26     |

| Fine Tune [Detune]                      ||  (Percussion Kit)            ,064    (40)    : R23     |

|   CC#101=000 (Bn 65 00)                 ||  (Latin Perc. Kit)           ,065    (41)    : R24     |

|   CC#100=001 (Bn 64 01)                 ||                              ,066~071(42~47) : R23     |

|   CC#006=000~127 (Bn 06 00~7F)          ||  (User Kit 1)                ,072    (48)    : R51     |

|   CC#038=000~127 (Bn 26 00~7F)          ||  (User Kit 2)                ,073    (49)    : R52     |

|     (*net=032,081~064,000~096,000       ||  (User Kit 3)                ,074    (4A)    : R53     |

|           : -50  ~  +00  ~  +50   )     ||  (User Kit 4)                ,075    (4B)    : R54     |

|   ARR PLAY/B.SEQ/SONG/SONG PLAY/E.STYLE ||  (User Kit 5)                ,076    (4C)    : R55     |

|       act as Chennel's Transpose.       ||                              ,077    (4D)    : R56     |

|   GLOBAL                   (User Kit 6) ||  (User Kit 7)                ,078    (4E)    : R57     |

|    When received Global MIDI Channel,   ||  (User Kit 8)                ,079    (4F)    : R58     |

|       act as "Master Tune"(Global Mode) ||                              ,080~093(50~5D) : R11~R26 |

|-----------------------------------------||  (Techno Kit)                ,094    (5E)    : R27     |

| Coarse Tune [Transpose]                 ||  (Rock Kit)                  ,095    (5F)    : R28     |

|   CC#101=000 (Bn 65 00)                 ||  (i30 Perc. Kit)             ,096    (60)    : R31     |

|   CC#100=002 (Bn 64 02)                 ||  (i30 Latin Perc.)           ,097    (61)    : R32     |

|   CC#006=000~127 (Bn 06 00~7F)          ||  (GM Kit 3)                  ,098    (62)    : R33     |

|     (*net=040~064~088                   ||  (Ballad Kit)                ,099    (63)    : R34     |

|         : +24~+00~+24)                  ||  (Jazz Kit 2)                ,100    (64)    : R35     |

|   CC#038=ignore                         ||  (R & B Kit)                 ,101    (65)    : R36     |

|       at every Note On                  ||  (Techno Kit2)               ,102    (66)    : R37     |

+-----------------------------------------+|  (Brush Kit 2)               ,103    (67)    : R38     |

|  (Lo-Fi Kit)                 ,104    (68)    : R41     |

|  (Pop Kit)                   ,105    (69)    : R42     |

|  (i30 Rock Kit)              ,106    (6A)    : R43     |

|  (Defness Kit)               ,107    (6B)    : R44     |

(*4)----------------------------------+    |  (Drum'n'Bass Kit)           ,108    (6C)    : R45     |

| vv=000~063(00~3F) : Fast or Dark    |    |  (Analog Kit 2)              ,109    (6D)    : R46     |

| vv=064(40)        : No Change       |    |  (Kicks & Snares)            ,110    (6E)    : R47     |

| vv=065~127(41~7F) : Slow or Bright  |    |  (Tom & HH & Sym)            ,111    (6F)    : R48     |

+-------------------------------------+    |                              ,112~115(70~73) : R11     |

|  (Arabian Kit)               ,116    (74)    : R25     |

|                              ,117~127(75~7F) : R11     |

+--------------------------------------------------------+

2-2.System Common Messages

+--------------------------------------------------------------------------------------------+

| Status         | Second         | Third          | Description                             |

|   [Bin(Hex)]   |   [Bin(Hex)]   |   [Bin(Hex)]   |                                         |

|--------------------------------------------------------------------------------------------|

| 1111 0010 (F2) | 0sss ssss (ss) | 0ttt tttt (tt) | Song Position Pointer                   |

|                |                |                |     sss ssss : Least significant (LSB)  |

|                |                |                |     ttt tttt : Most significant (MSB)   |

| 1111 0011 (F3) | 0000 0000 (00) | 0000 ssss (ss) | Song Select                             |

|                |                |                |          ssss:Song Number = 0~9         |

+--------------------------------------------------------------------------------------------+

These messages are received when "Clock Source" is External and Sequencer stop.

(ex.)If all measure's "Meter" are 4/4.

F2 ss tt : Measure.Beat.Tick

F2 00 00 : 001.1.00

F2 01 00 : 001.1.24  (6 MIDI Clock)

F2 10 00 : 002.1.00  (96 MIDI Clock)

F2 20 00 : 003.1.00

|

F2 00 01 : 009.1.00

|

F2 60 7C : 999.1.00

2-3.System Realtime Messages

+----------------------------------------------------------+

| Status         | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 1000 (F8) | Timing Clock                            |

| 1111 1010 (FA) | Start                                   |

| 1111 1011 (FB) | Continue                                |

| 1111 1100 (FC) | Stop                                    |

| 1111 1110 (FE) | Active Sensing                          |

+----------------------------------------------------------+

Received in ARR PLAY/BSEQ/SONG/E.STYLE mode when "Clock Source" is External.

2-4.Universal System Exclusive Message (Non Realtime)

Inquiry Message Request

+----------------------------------------------------------+

| Byte           | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 0000 (F0) | Exclusive Status                        |

| 0111 1110 (7E) | Non Realtime Message                    |

| 0ggg gggg (gg) | Global MIDI Channel                     |

| 0000 0110 (06) | General Information (Sub ID 1)          |

| 0000 0001 (01) | Identity Request (Sub ID 2)             |

| 1111 0111 (F7) | End of Exclusive (EOX)                  |

+----------------------------------------------------------+

gg = 00~0F :Received if Global Channel

7F    :Received on any Channel

General MIDI Mode ON

+----------------------------------------------------------+

| Byte           | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 0000 (F0) | Exclusive Status                        |

| 0111 1110 (7E) | Non Realtime Message                    |

| 0ggg gggg (gg) | Global MIDI Channel                     |

| 0000 1001 (09) | General MIDI Message (Sub ID 1)         |

| 0000 0001 (01) | General MIDI On (Sub ID 2)              |

| 1111 0111 (F7) | End of Exclusive (EOX)                  |

+----------------------------------------------------------+

Received in SONG PLAY mode except when Sequencer playing.

2-5.Universal System Exclusive Message (Realtime)

Master Volume

+----------------------------------------------------------+

| Byte           | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 0000 (F0) | Exclusive Status                        |

| 0111 1111 (7F) | Realtime Message                        |

| 0ggg gggg (gg) | Global MIDI Channel                     |

| 0000 0100 (04) | Device Control (Sub ID 1)               |

| 0000 0001 (01) | Master Volume  (Sub ID 2)               |

| 0vvv vvvv (vv) | Value(LSB)                              |

| 0mmm mmmm (mm) | Value(MSB)                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                  |

+----------------------------------------------------------+

mm,vv=00,00 ~ 7F,7F : Min ~ Max

Master Balance

+----------------------------------------------------------+

| Byte           | Description                             |

|   [Bin(Hex)]   |                                         |

|----------------------------------------------------------|

| 1111 0000 (F0) | Exclusive Status                        |

| 0111 1111 (7F) | Realtime Message                        |

| 0ggg gggg (gg) | Global MIDI Channel                     |

| 0000 0100 (04) | Device Control (Sub ID 1)               |

| 0000 0010 (02) | Master Balance (Sub ID 2)               |

| 0vvv vvvv (vv) | Value(LSB)                              |

| 0mmm mmmm (mm) | Value(MSB)                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                  |

+----------------------------------------------------------+

mm,vv=00,00 ~ 40,00 ~ 7F,7F : Left ~ Center ~ Right

2-6.KORG System Exclusive Message Received Function Code List

+-------------------------------------------------------------------+

| 5th Byte       | Description                             |        |

|   [Bin(Hex)]   |                                         |        |

|-------------------------------------------------------------------|

| 0100 0010 (12) | Mode Request                            | A,r-42 |

| 0100 1110 (4E) | Mode Change                             | A,r-23 |

| 0001 0000 (10) | Program Parameter Dump Request          | P,r-40 |

| 0000 1101 (0D) | Drum Program Parameter Dump Request     | P,r-52 |

| 0011 0000 (30) | ARR(All Arrangement) Dump Request       | M,r-64 |

| 0011 0011 (33) | ARG(Arrangement Global) Dump Request    | M,r-68 |

| 0011 0100 (34) | ARK(All KBD Set) Dump Request           | M,r-69 |

| 0001 1100 (1C) | PRG(All Program) Dump Request           | M,r-4C |

| 0000 1110 (0E) | GBL(Global) Dump Request                | M,r-51 |

| 0011 0001 (31) | STY(Style Block) Dump Request           | M,r-65 |

| 0011 0010 (32) | BSQ(All Backinh Sequence) Dump Request  | M,r-66 |

| 0001 1000 (18) | SNG(All Song) Dump Request              | M,r-48 |

| 0100 0000 (40) | Current Program Parameter Dump          | P,r-23 |

| 0101 0010 (52) | Current Drum Program Parameter Dump     | P,r-23 |

| 0110 0100 (64) | ARR(All Arrangement) Dump               | G,r-23 |

| 0110 1000 (68) | ARG(Arrangement Global) Dump            | G,r-23 |

| 0110 1001 (69) | ARK(All KBD Set) Dump                   | G,r-23 |

| 0100 1100 (4C) | PRG(All Program) Dump                   | G,r-23 |

| 0101 0001 (51) | GBL(Global) Dump                        | G,r-23 |

| 0110 0101 (65) | STY(Style Block) Dump                   | G,r-23 |

| 0110 0110 (66) | BSQ(All Backing Sequence) Dump          | G,r-23 |

| 0100 1000 (48) | SNG(All Song) Dump                      | G,r-23 |

| 0100 0001 (41) | Program Parameter Change                | P,r-23 |

| 0101 0011 (53) | Drum Program Parameter Change           | P,r-23 |

| 0001 0001 (11) | Program & Drum Program  Write Request   | P,r-21 |

| 0110 0111 (67) | Chord                                   | A      |

+-------------------------------------------------------------------+

Received when

G    : "Receive Dumped Data"dialog is opened.

M    : "Filter,MIDI"page is opened.

P    : in Program mode.

A    : in All mode.

r-XX : Reply Function Number

(Transmitted after the message has been received.)

3.MIDI Exclusive Format (R:Receive, T:Transmit)

Please see 1-5 "Structure of KORG System Exclusive Messages"

(1) Mode Request                                              R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0001 0010 (12) | MODE REQUEST                                  |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits MODE DATA(42) message.

(2) Mode Data                                                 -,T

+----------------------------------------------------------------+

| Byte           | Description                                   | 0m=0A :ARR PLAY

|----------------------------------------------------------------|   =0B :BACKING SEQ

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |   =06 :SONG

| 0100 0010 (42) | MODE DATA                                     |   =05 :SONG PLAY

| 0000 mmmm (0m) | Mode                                          |   =02 :PROGRAM

| 0000 0000 (00) |                                               |   =09 :E.STYLE

| 1111 0111 (F7) | End of Exclusive (EOX)                        |   =04 :GLOBAL

+----------------------------------------------------------------+   =08 :DISK

Receives MODE REQUEST(12) message, and transmits this message and data.

(3) Mode Change                                               R,T

+----------------------------------------------------------------+

| Byte           | Description                                   | 0m=0A :ARR PLAY

|----------------------------------------------------------------|   =0B :BACKING SEQ

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |   =06 :SONG

| 0100 1110 (4E) | MODE CHANGE                                   |   =05 :SONG PLAY

| 0000 mmmm (0m) | Mode                                          |   =02 :PROGRAM

| 1111 0111 (F7) | End of Exclusive (EOX)                        |   =09 :E.STYLE

+----------------------------------------------------------------+   =04 :GLOBAL

Receives this message and data, changes the Mode.  When the        =08 :DISK

mode is changed by switch, this message and data is transmitted.

At the same time "i-series" ID(39) is transmitted for "ih".

(4) ARR(All Arrangement) Dump Request                         R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0011 0000 (30) | ARR DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits ARR DUMP(64) message.

(5) ARG(Arrangement Global) Dump Request                      R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0011 0011 (33) | ARG DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits ARG DUMP(64) message.

(6) ARK(All KBD Set) Dump Request                             R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0011 0100 (34) | ARK DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits ARK DUMP(69) message.

(7) PRG(All Program) Dump Request                             R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0001 1100 (1C) | PRG DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits PRG DUMP(4C) message.

(8) GBL(Global) Dump Request                                  R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0000 1110 (0E) | GBL DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits GBL DUMP(51) message.

(9) STY(Style Block) Dump Request                             R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0011 0001 (31) | STY DUMP REQUEST                              |

| 0000 bbbb (0b) | Style Block Number : 1~12  0b:00~0B           |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits STY DUMP(65) message.

(10) BSQ(All Backing Sequence) Dump Request                   R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0011 0010 (32) | BSQ DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits BSQ DUMP(66) message.

(11) SNG(All Song) Dump Request                               R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0001 1000 (18) | SNG DUMP REQUEST                              |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits SNG DUMP(48) message.

(12) Program Parameter Dump Request                           R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0001 0000 (10) | PROGRAM PARAMETER DUMP REQUEST                |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits CURRENT PROGRAM PARAMETER(40).

(13) Drum Program Parameter Dump Request                      R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0000 1101 (0D) | DRUM PROGRAM PARAMETER DUMP REQUEST           |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives this message, and transmits CURRENT DRUM PROGRAM PARAMETER(52).

(14) ARR(All Arrangement) Dump                                R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 0100 (64) | ARR DUMP                                      |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives ARR DUMP REQUEST(30) message, and transmits this message & data.

Transmits this message & data when "Dump Arrangements" is executed.

NOTE 1:DUMP DATA CONVERT

Data <-> MIDI Data

*** Data (1set = 8bit x 7Byte) **************************************************

*   b7            b0   b7            b0   b7            b0   b7            b0   *

*   |Y|X|X|X|X|X|X|X|  |Y|X|X|X|X|X|X|X|  |Y|X|X|...|X|X|X|  |Y|X|X|X|X|X|X|X|  *

*        7n+0               7n+1            7n+2 ... 7n+5         7n+6          *

*********************************************************************************

*** MIDI Data (1 Set = 7bit x 8Byte) ********************************************

*     b7b7b7b7b7b7b7     b6          b0     b6          b0     b6          b0   *

*   |0|Y|Y|Y|Y|Y|Y|Y|  |0|X|X|X|X|X|X|X|  |0|X|X|...|X|X|X|  |0|X|X|X|X|X|X|X|  *

*   7n+6,5,4,3,2,1,0        7n+0            7n+1 ... 7n+5         7n+6          *

*********************************************************************************

NOTE 2:ARR DUMP FORMAT

[Arrangement C11],...,[Arrangement D88]

Data: 164 x 128 = 20992[Byte]

MIDI: 7x2998+6=20992 -> 8*2998+(1+6)=23991[Byte]

(15) ARG(Arrangement Global) Dump                             R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 1000 (68) | ARG DUMP                                      |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives ARG DUMP REQUEST(33) message, and transmits this message & data.

Transmits this message & data when "Dump Arrangement Global" is executed.

NOTE 3:ARG DUMP FORMAT

Data: 48[Byte]

MIDI: 7x6+6=48 -> 8x6+(1+6)=55[Byte]

(16) ARK(All KBD Set) Dump                                    R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 1001 (69) | ARK DUMP                                      |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives ARK DUMP REQUEST(34) message, and transmits this message & data.

Transmits this message & data when "Dump KBD Sets" is executed.

NOTE 4:ARK DUMP FORMAT

[KBD SET 11],...,[KBD SET 88]

Data: 56 x 64 = 3584[Byte]

MIDI: 7x512=3584 -> 8x512=4096[Byte]

(17) PRG(All Program) Dump                                    R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0100 1100 (4C) | PRG DUMP                                      |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives PRG DUMP REQUEST(1C) message, and transmits this message & data.

Transmits this message & data when "Dump Programs" is executed.

NOTE 5:PRG DUMP FORMAT

[Program F11],...,[Program G88]

,[Program R51],...,[Program R58]

,[Drum Kit R51],...,[Drum Kit R58]

Data: 172 x 128 + 172 x 8 + 880 x 8 = 30432[Byte]

MIDI: 7x4347+3=30432 -> 8x4347+(1+3)=34780

(18) GBL(Global) Dump                                         R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0101 0001 (51) | GBL DUMP                                      |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives GBL DUMP REQUEST(0E) message, and transmits this message & data.

Transmits this message & data when "Dump Global Setting" is executed.

NOTE 6:GBL DUMP FORMAT

Data: 416[Byte]

MIDI: 7x59+3=416 -> 8x59+(1+3)=476

(19) STY(Style Block) Dump                                    R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 0101 (65) | STY DUMP                                      |

| 0000 bbbb (0b) | Style Block Number : 1~12  0b:00~0B           |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives STY DUMP REQUEST(31) message, and transmits this message & data.

Transmits this message & data when "Dump Styles" is executed.

NOTE 7:STY DUMP FORMAT

[Style C11],...,[Style C68]

Style Header (Block 1:C11~14...32[Byte])

Style Data   (Block 1:C11~14...1176 x 4 + Seq.Data[Byte])

Min Data: 32+1176x4 + Seq.Data =  4704[Byte]

MIDI: 7x672=4704 -> 8x672=5376

Max Data: 32+1176x4 + Seq.Data = 65504[Byte]

MIDI: 7x9357+5=65504 -> 8x9357+(1+5)=74862

Style Header (Block 2:C15~18...32[Byte])

Style Data   (Block 2:C15~18...1176 x 4 + Seq.Data[Byte])

|

Style Header (Block12:C65~68...32[Byte])

Style Data   (Block12:C65~68...1176 x 4 + Seq.Data[Byte])

(20) BSQ(All Backing Sequence) Dump                           R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 0110 (66) | BSQ DUMP                                      |

| 0sss ssss (ss) | Backing Sequence Data Size(LSB:bit 6~0)       |

| 0sss ssss (ss) | Backing Sequence Data Size(MSB:bit13~7)       |

| 0ddd dddd (dd) | Control Data                                  |

| 0ddd dddd (dd) | Backing Sequence Data                         |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives BSQ DUMP REQUEST(32) message, and transmits this message & data.

Transmits this message & data when "Dump Backing Sequences" is executed.

NOTE 8:BSQ DUMP FORMAT

[Backing SEQ 0],...,[Backing SEQ 9]

Backing Sequence Data Size

4 step = 16 Byte = 1 size

Backing Sequence Control Data   (2642[Byte])

Backing Sequence Data

[BSEQ 1st Data(4 Byte)],...,[BSEQ nth Data]

n:BSEQ Data Step = 0~65536

Data: 2642+4x[BSEQ Data Step]

MIDI: =7xA+B -> B=1~7...8xA+(1+B),B=0...8xA

(21) SNG(All Song) Dump                                       R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0100 1000 (48) | SNG DUMP                                      |

| 0sss ssss (ss) | Song Data Size                                |

| 0ddd dddd (dd) | Control Data                                  |

| 0ddd dddd (dd) | Song Data                                     |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives SNG DUMP REQUEST(18) message, and transmits this message & data.

Transmits this message & data when "Dump Songs" is executed.

NOTE 9:SNG DUMP FORMAT

[Song 0],...,[Song 9]

Song Data Size

4 step = 16 Byte = 1 size

Song Control Data   (3922[Byte])

Song Data

[Song 1st Data(4 Byte)],...,[Song nth Data]

n:BSEQ Data Step = 0~65536

Data: 3922+4x[Song Data Step]

MIDI: =7xA+B -> B=1~7...8xA+(1+B),B=0...8xA

(22) Current Program Parameter Dump                           R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0100 0000 (40) | CURRENT PROGRAM PARAMETER DUMP                |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives PROGRAM PARAMETER DUMP REQUEST(10) message,

and transmits this message & data.

NOTE 10:Current Program Parameter DUMP FORMAT

[Parameter No.000],...,[Parameter No.171]

Data: 172[Byte]

MIDI: =7x24+4=172 -> 8x24+(1+4)=197

(23) Current Drum Program Parameter Dump                      R,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0101 0010 (52) | CURRENT DRUM PROGRAM PARAMETER DUMP           |

| 0ddd dddd (dd) | Data                                          |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Receives DRUM PROGRAM PARAMETER DUMP REQUEST(0D) message,

and transmits this message & data.

NOTE 11:Current Drum Program Parameter DUMP FORMAT

[Prog.Parameter No.000],...,[Prog.Parameter No.171]

[Drum Kit Parameter No.000],...,[Drum Kit Parameter No.879]

Data: 172+880=1052[Byte]

MIDI: =7x150+2=1052 -> 8x150+(1+2)=1203

(24) Program Parameter Change                                 R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0100 0001 (41) | PROGRAM PARAMETER CHANGE                      |

| 0000 0000 (00) | Reserve                                       |

| 0ppp pppp (pp) | Parameter Number (LSB bit 6~0)                |

| 0ppp pppp (pp) | Parameter Number (MSB bit13~7)                |

| 0vvv vvvv (vv) | Value (LSB bit 6~0)                           |

| 0vvv vvvv (vv) | Value (MSB bit13~7)                           |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

See "PROGRAM PARAMETERS" (except"FX PARAMETERS") .

NOTE 12:Parameter Number and Value Data Format

Parameter Number,Parameter Value

MSB                          LSB

|M5|M5|M5|M4|M3|M2|M1|M0|    |L7|L6|L5|L4|L3|L2|L1|L0|

MIDI Data

MSB                          LSB

| 0|M5|M4|M3|M2|M1|M0|L7|    | 0|L6|L5|L4|L3|L2|L1|L0|

(Ex) " OSC 1 Multisample=298 "

Param No.=020 (14 Hex), Value=298 (012A Hex)

MIDI Data=F0,42,3g,49,41,00,14,00,2A,02,F7

(25) Drum Program Parameter Change                            R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0101 0011 (53) | DRUM PROGRAM PARAMETER CHANGE                 |

| 0000 0000 (00) | Reserve                                       |

| 0ppp pppp (pp) | Parameter Number (LSB bit 6~0)                |

| 0ppp pppp (pp) | Parameter Number (MSB bit13~7)                |

| 0vvv vvvv (vv) | Value (LSB bit 6~0)                           |

| 0vvv vvvv (vv) | Value (MSB bit13~7)                           |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

See "PROGRAM PARAMETERS""DRUM KIT PARAMETERS" (except"FX PARAMETERS") .

NOTE 13:[Prog.Parameter No.000]=[Prog.Parameter No.000]

,...,[Prog.Parameter No.171]=[Prog.Parameter No.171]

,[Drum Kit Parameter No.000]=[Prog.Parameter No.172]

,...,[Drum Kit Parameter No.879]=[Prog.Parameter No.1051]

(Ex) " Bb0 Drumsample=126 "

Param No.=182 (B6 Hex), Value=126 (007F Hex)

MIDI Data=F0,42,3g,49,53,00,36,01,7E,00,F7

(26) Program & Drum Program  Write Request                    R,-

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0001 0001 (11) | PROGRAM & DRUM PROGRAM WRITE REQUEST          |

| 0000 0bbb (0b) | Bank Number  (03:FG, 04:DRUM)                 |

| 0ppp pppp (pp) | Program Number  (FG:00~7F, R:20~27)           |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(27) Write Completed                                          -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0010 0001 (21) | WRITE COMPLETED                               |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(28) Write Error                                              -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0010 0010 (22) | WRITE ERROR                                   |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(29) Received Message Format Error                            -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0010 0101 (26) | RECEIVED MESSAGE FORMAT ERROR                 |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(30) Data Load Completed (ACK)                                -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0010 0011 (23) | DATA LOAD COMPLETED (ACK)                     |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(31) Data Load ERROR (NAK)                                    -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,49    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0010 0011 (23) | DATA LOAD ERROR (NAK)                         |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

(32) Chord                                                    -,T

+----------------------------------------------------------------+

| Byte           | Description                                   |

|----------------------------------------------------------------|

| F0,42,3g,39    | i30 Exclusive Header g;Global Channel  [Hex]  |

| 0110 0111 (67) | CHORD                                         |

| 0000 rrrr (0r) | Root  (C:00 ~ B:0B)                           |

| 0000 bbbb (0b) | Bass  (C:00 ~ B:0B)                           |

| 0ccc cccc (0c) | Chord Type (LSB)                              |

| 000c cccc (0c) | Chord Type (MSB)                              |

| 0ttt tttt (0t) | Tension note(s) (LSB)                         |

| 000t tttt (0t) | Tension note(s) (MSB)                         |

| 1111 0111 (F7) | End of Exclusive (EOX)                        |

+----------------------------------------------------------------+

Note 14:CHORD Type

+----------------------------------------------+

| Chord Type | MSB            | LSB            |

|            |    B -A-G      |  -FE -D-C      |

|----------------------------------------------|

| No Chord   | 0000 0000 (00) | 0000 0000 (00) |

| dim        | 0000 0000 (00) | 0100 1001 (49) |

| sus2       | 0000 0001 (01) | 0000 0101 (05) |

| m          | 0000 0001 (01) | 0000 1001 (09) |

| major      | 0000 0001 (01) | 0001 0001 (11) |

| sus4       | 0000 0001 (01) | 0010 0001 (21) |

| aug        | 0000 0010 (02) | 0001 0001 (11) |

| m6         | 0000 0101 (05) | 0000 1001 (09) |

| 6          | 0000 0101 (05) | 0001 0001 (11) |

| m7b5       | 0000 1000 (08) | 0100 1001 (49) |

| 7b5        | 0000 1000 (08) | 0101 0001 (51) |

| m7         | 0000 1001 (09) | 0000 1001 (09) |

| 7sus4      | 0000 1001 (09) | 0010 0001 (21) |

| aug7       | 0000 1010 (0A) | 0001 0001 (11) |

| dimM7      | 0001 0000 (10) | 0100 1001 (49) |

| M7b5       | 0001 0000 (10) | 0101 0001 (51) |

| mM7        | 0001 0001 (11) | 0000 1001 (09) |

| M7         | 0001 0001 (11) | 0001 0001 (11) |

| M7sus4     | 0001 0001 (11) | 0010 0001 (21) |

| augM7      | 0001 0010 (12) | 0001 0001 (11) |

+----------------------------------------------+

Note 15:Tension Note(s)

+----------------------------------------------+

| Tension    | MSB                LSB          |

|----------------------------------------------|

| b9         | 0000 0000 (00) | 0000 0010 (00) |

| 9          | 0000 0000 (00) | 0000 0100 (04) |

| #9         | 0000 0000 (00) | 0000 1000 (08) |

| 11         | 0000 0000 (00) | 0010 0000 (20) |

| #11        | 0000 0000 (00) | 0100 0000 (40) |

| b13        | 0000 0010 (02) | 0000 0000 (00) |

| 13         | 0000 0100 (04) | 0000 0000 (00) |

+----------------------------------------------+

4.Parameter Tables

ARRANGEMENT PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|Arrangement C11 Parameters                             |

|-------------------------------------------------------|

|000|   Arragement Name(1/16)   | 20~7E |" " ~ "~"      |

|001|   Arragement Name(2/16)   |       |(ASCII Code)   |

|002|   Arragement Name(3/16)   |       |               |

|003|   Arragement Name(4/16)   |       |               |

|004|   Arragement Name(5/16)   |       |               |

|005|   Arragement Name(6/16)   |       |               |

|006|   Arragement Name(7/16)   |       |               |

|007|   Arragement Name(8/16)   |       |               |

|008|   Arragement Name(9/16)   |       |               |

|009|   Arragement Name(10/16)  |       |               |

|010|   Arragement Name(11/16)  |       |               |

|011|   Arragement Name(12/16)  |       |               |

|012|   Arragement Name(13/16)  |       |               |

|013|   Arragement Name(14/16)  |       |               |

|014|   Arragement Name(15/16)  |       |               |

|015|   Arragement Name(16/16)  |       |               |

|016|   Arragement Category     | 00~0F |8 Beat~User 4  |

|017|   Style No.               | 00~7F |               |[Style No.][Style Bank]

|018|   Style Bank              | 00,01 |               | Bank=00,No.=00~7F : A11~A88~B88

|019|   Initial Variation(1/2)  | 00~03 |Var.1~4        | Bank=01,No.=00~2F : C11~C68

|020|   Initial Variation(2/2)  |       |00:->Var.1~4   |

|   |    (b3)Fill               | 0,1   |04:Intro 1     |

|   |    (b2)Intro              | 0,1   |05:Intro 2     |

|   |    (b1)...Reserve         | ----- |08:Fill 1      |

|   |    (b0)1 or 2             | 0,1   |09:Fill 2      |

|021|   Initial Tempo           | 0A~D2 |40~240         |

|022|   FULL KBD PLAY           | 00,01 |Off,On         |

|023|   Split Point             | 24~60 |C2~C7          |

|024|   Ensemble Type           | 00~09 |Duet~Brass Ens.|

|025|   Transpose               | F5~0B |-11~+11        |

|026|   ...Reserve              | ----- | -----         |

|027|   ...Reserve              | ----- | -----         |

|028|   Chord Scan Parameters   |       |               |

|   |    (b7)...Reserve         | ----- | -----         |

|   |    (b6)...Reserve         | ----- | -----         |"Lower type" is effective

|   |    (b5)...(Lower Type)    | (0,1) |(Normal,Easy)  | at first released system.

|   |    (b4)...Reserve         | ----- | -----         |

|   |    (b3)...Reserve         | ----- | -----         |

|   |    (b2)...Reserve         | ----- | -----         |[Chord Scanning]

|   |    (b1)Chord Scan Upper   | 0,1   |Off,On         |00:Off    02:Upper

|   |    (b0)Chord Scan Lower   | 0,1   |Off,On         |01:Lower  03:FULL

|029|   Var.1 Default Drum Map  | 00~07 |Map 1~8        |

|030|   Var.2 Default Drum Map  | 00~07 |Map 1~8        |

|031|   Var.3 Default Drum Map  | 00~07 |Map 1~8        |

|032|   Var.4 Default Drum Map  | 00~07 |Map 1~8        |

|033|   Kick Designation        | 00~03 |A~D            |

|034|   Snare Designation       | 00~03 |A~D            |[Fill 1][Fill 2]

|035|   ...Reserve              | ----- |-----          |00:Off     07:1<->4

|036|   ...Reserve              | ----- |-----          |01:->1     08:2<->3

|037|   Fill 1(Var.Change)      | 00~0C |Off~Down       |02:->2     09:2<->4

|038|   Fill 2(Var.Change)      | 00~0C |Off~Down       |03:->3     0A:3<->4

|-------------------------------------------------------|04:->4     0B:Up

|       DRUMS PARAMETERS                                |05:1<->2   0C:Down

|-------------------------------------------------------|06:1<->3

|039|   Program No.             | 00~7F |               |

|040|   Program Bank            | 00~04 |               |[Program No.][Program Bank]

|041|   Volume                  | 00~7F |000~127        | Bank=00,No.=00~7F : A11~A88~B88

|042|   Pan                     | FF~1F |Off~Prog       | Bank=01,No.=00~7F : C11~C88~D88

|043|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       | Bank=02,No.=00~3F : E11~E88

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       | Bank=03,No.=00~7F : F11~F88~G88

|044|   ...Reserve              | ----- |-----          | Bank=04,No.=00~27 : R11~R58

|045|   Out Status              |       |               |[Pan]

|   |    (b7)...Reserve         | ----- |-----          |FF : OFF

|   |    (b6)...Reserve         | ----- |-----          |00 : L15

|   |    (b5)...Reserve         | ----- |-----          |    |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |0F : CNT

|   |    (b3)...Reserve         | ----- |-----          |    |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |1E : R15

|   |    (b1)External Out       | 0,1   |Off,On         |1F : PROG

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|046|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       PERC PARAMETERS                                 |

|-------------------------------------------------------|

|047|   Program No.             | 00~7F |               |

|048|   Program Bank            | 00~04 |               |

|049|   Volume                  | 00~7F |000~127        |

|050|   Pan                     | FF~1F |Off~Prog       |

|051|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|052|   ...Reserve              | ----- |-----          |

|053|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|054|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       BASS PARAMETERS                                 |

|-------------------------------------------------------|

|055|   Program No.             | 00~7F |               |

|056|   Program Bank            | 00~04 |               |

|057|   Volume                  | 00~7F |000~127        |

|058|   Pan                     | FF~1F |Off~Prog       |

|059|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|060|   Octave                  | FE~02 |-2~+2          |

|061|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|062|   Wrap-Around Point       | FF~0B |Style~12       |

|-------------------------------------------------------|

|       ACC1 PARAMETERS                                 |

|-------------------------------------------------------|

|063|   Program No.             | 00~7F |               |

|064|   Program Bank            | 00~04 |               |

|065|   Volume                  | 00~7F |000~127        |

|066|   Pan                     | FF~1F |Off~Prog       |

|067|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|068|   Octave                  | FE~02 |-2~+2          |

|069|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|070|   Wrap-Around Point       | FF~0B |Style~12       |

|-------------------------------------------------------|

|       ACC2 PARAMETERS                                 |

|-------------------------------------------------------|

|071|   Program No.             | 00~7F |               |

|072|   Program Bank            | 00~04 |               |

|073|   Volume                  | 00~7F |000~127        |

|074|   Pan                     | FF~1F |Off~Prog       |

|075|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|076|   Octave                  | FE~02 |-2~+2          |

|077|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|078|   Wrap-Around Point       | FF~0B |Style~12       |

|-------------------------------------------------------|

|       ACC3 PARAMETERS                                 |

|-------------------------------------------------------|

|079|   Program No.             | 00~7F |               |

|080|   Program Bank            | 00~04 |               |

|081|   Volume                  | 00~7F |000~127        |

|082|   Pan                     | FF~1F |Off~Prog       |

|083|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|084|   Octave                  | FE~02 |-2~+2          |

|085|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)Synchro Backing    | 0,1   |Off,On         |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|086|   Wrap-Around Point       | FF~0B |Style~12       |

|-------------------------------------------------------|

|       MainKBD PARAMETERS                              |

|-------------------------------------------------------|

|087|   Program No.             | 00~7F |               |

|088|   Program Bank            | 00~04 |               |

|089|   Volume                  | 00~7F |000~127        |

|090|   Pan                     | FF~1F |Off~Prog       |

|091|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|092|   Octave                  | FE~02 |-2~+2          |

|093|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|094|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       SubKBD PARAMETERS                               |

|-------------------------------------------------------|

|095|   Program No.             | 00~7F |               |

|096|   Program Bank            | 00~04 |               |

|097|   Volume                  | 00~7F |000~127        |

|098|   Pan                     | FF~1F |Off~Prog       |

|099|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|100|   Octave                  | FE~02 |-2~+2          |

|101|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|102|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       LowerKBD PARAMETERS                             |

|-------------------------------------------------------|

|103|   Program No.             | 00~7F |               |

|104|   Program Bank            | 00~04 |               |

|105|   Volume                  | 00~7F |000~127        |

|106|   Pan                     | FF~1F |Off~Prog       |

|107|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|108|   Octave                  | FE~02 |-2~+2          |

|109|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|110|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       Harmony PARAMETERS                              |

|-------------------------------------------------------|

|111|   Program No.             | 00~7F |               |

|112|   Program Bank            | 00~04 |               |

|113|   Volume                  | 00~7F |000~127        |

|114|   Pan                     | FF~1F |Off~Prog       |

|115|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|116|   ...Reserve              | ----- |-----          |

|117|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)...Reserve         | ----- |-----          |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|118|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       K.Bass PARAMETERS                               |

|-------------------------------------------------------|

|119|   Program No.             | 00~7F |               |

|120|   Program Bank            | 00~04 |               |

|121|   Volume                  | 00~7F |000~127        |

|122|   Pan                     | FF~1F |Off~Prog       |

|123|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|124|   ...Reserve              | ----- |-----          |

|125|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)...Reserve         | ----- |-----          |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|126|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|127|   Main Vel. Win. Top      | 01~7F |001~127        |

|128|   Main Vel. Win. Bottom   | 01~7F |001~127        |

|129|   Sub Vel. Win. Top       | 01~7F |001~127        |

|130|   Sub Vel. Win. Bottom    | 01~7F |001~127        |

|131|   Lower Vel. Win. Top     | 01~7F |001~127        |

|132|   Lower Vel. Win. Bottom  | 01~7F |001~127        |

|133|   KBD Scale Type          | 00~29 |               |[Scale Type]

|134|   KBD Scale Key           | 00~0B |C,C#~Bb,B      |00:Equal Temperament

|-------------------------------------------------------|~29:User Scale 32

|       ACC FX PARAMETERS                               |

|-------------------------------------------------------|

|135|   FX 1 Type               | 00~2F |               |[FX 1/2 Type]

|136|   FX 2 Type               | 00~2F |               |0:NoEffect

|137|   FX 1 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |~2F:Delay/RotarySpeaker

|138|   FX 1 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |

|139|   FX 2 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |

|140|   FX 2 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |[Effect Output C/D Pan]

|141|   Effect Output C Pan     | 00~65 |Off,R,01:99~L  |When Placement is Parallel3

|142|   Effect Output D Pan     | 00~65 |Off,R,01:99~L  |No.141(bit7~4) FX 1 Return L : 0~9

|143|   Effect I/O              |       |               |No.141(bit3~0) FX 1 Return R : 0~9

|   |    (b7)...Reserve         | ----- |-----          |No.142(bit7~4) FX 2 Return L : 0~9

|   |    (b6)Placement(1/3)     | 0~5   |               |No.142(bit3~0) FX 2 Return R : 0~9

|   |    (b5)Placement(2/3)     |       |               |

|   |    (b4)Placement(3/3)     |       |               |[Placement]

|   |    (b3)Effect 2(R) On/Off | 0,1   |Off,On         |0:Serial

|   |    (b2)Effect 2(L) On/Off | 0,1   |Off,On         |1:Parallel 1

|   |    (b1)Effect 1(R) On/Off | 0,1   |Off,On         |2:Parallel 2

|   |    (b0)Effect 1(L) On/Off | 0,1   |Off,On         |3:Parallel 3

|144|   FX 1 Parameter(1/8)     |       |               |4:Serial Sub

|145|   FX 1 Parameter(2/8)     |       |               |5:Parallel Sub

|146|   FX 1 Parameter(3/8)     |       |               |

|147|   FX 1 Parameter(4/8)     |       |               |

|148|   FX 1 Parameter(5/8)     |       |               |

|149|   FX 1 Parameter(6/8)     |       |               |

|150|   FX 1 Parameter(7/8)     |       |               |

|151|   FX 1 Parameter(8/8)     |       |               |

|152|   FX 1 D-Mod.Source       | 00~06 |None~VDA EG    |

|153|   FX 1 D-Mod.Amount       | F1~OF |-15~+15        |

|154|   FX 2 Parameter(1/8)     |       |               |

|155|   FX 2 Parameter(2/8)     |       |               |

|156|   FX 2 Parameter(3/8)     |       |               |

|157|   FX 2 Parameter(4/8)     |       |               |

|158|   FX 2 Parameter(5/8)     |       |               |

|159|   FX 2 Parameter(6/8)     |       |               |

|160|   FX 2 Parameter(7/8)     |       |               |

|161|   FX 2 Parameter(8/8)     |       |               |

|162|   FX 2 D-Mod.Source       | 00~06 |None~VDA EG    |

|163|   FX 2 D-Mod.Amount       | F1~OF |-15~+15        |

|-------------------------------------------------------|

|164~20991  Arrangement C12~C88,D11~D88 (Same as C11)   |

+-------------------------------------------------------+

ARRANGEMENT GLOBAL PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

| 00|   " Main " MIDI Channel   | 00~0F |1~16           |

| 01|   " Sub " MIDI Channel    | 00~0F |1~16           |

| 02|   " Lower " MIDI Channel  | 00~0F |1~16           |

| 03|   " Drum " MIDI Channel   | 00~0F |1~16           |

| 04|   " Perc " MIDI Channel   | 00~0F |1~16           |

| 05|   " Bass " MIDI Channel   | 00~0F |1~16           |

| 06|   " ACC1 " MIDI Channel   | 00~0F |1~16           |

| 07|   " ACC2 " MIDI Channel   | 00~0F |1~16           |

| 08|   " ACC3 " MIDI Channel   | 00~0F |1~16           |

| 09|   " Harmo " MIDI Channel  | 00~0F |1~16           |

| 10|   " K.Bas " MIDI Channel  | 00~0F |1~16           |

| 11|   " Chord " MIDI Channel  | FF~0F |Off,1~16       |

| 12|   " Chrd2 " MIDI Channel  | FF~0F |Off,1~16       |

| 13|   " Chrd3 " MIDI Channel  | FF~0F |Off,1~16       |

| 14|   " ArrChg " MIDI Channel | FF~0F |Off,1~16       |

| 15|   " KBD " MIDI Channel    | 00~0F |1~16           |

| 16|   Chord Memory On/Off     | 00,01 |Off,On         |

| 17|   Chord Sound             |       |               |

|   |    (b4)Chord Sound Type   | 0,1   |Auto,Actual    |

|   |    (b0)Chord Sound SW     | 0,1   |Off,On         |

| 18|   Bass Inversion On/Off   | 00,01 |Off,On         |

| 19|   ...Reserve              | ----- |-----          |

| 20|   Tempo Lock On/Off       | 00,01 |Off,On         |

| 21|   KBD Lock On/Off         | 00,01 |Off,On         |

| 22|   Ensemble SW On/Off      | 00,01 |Off,On         |

| 23|   ...Reserve              | ----- |-----          |

| 24|   Synchro Start/Stop      |       |               |

|   |    (b1)Stop               | 0,1   |Off,On         |

|   |    (b0)Start              | 0,1   |Off,On         |

| 25|   Dynamic Backing         | 00,01 |Off,On         |[Velocity Control]

| 26|   Velocity Control        | 00~03 |0ff~Fill2      |1:Break,2:Fill1,3:Fill2

| 27|   Split Point Lock        | 00,01 |Off,On         |

| 28|   ...Reserve              | ----- |-----          |

| 29|   ...Reserve              | ----- |-----          |

| 30|   ...Reserve              | ----- |-----          |

| 31|   ...Reserve              | ----- |-----          |

| 32|   ChordSound Sync on/off  | 0,1   |Off,On         |

| 33|   ...Reserve              | ----- |-----          |

| 34|   ...Reserve              | ----- |-----          |

| 35|   ...Reserve              | ----- |-----          |

| 36|   Lower Chord Recog. Mode | 00~02 |Easy~One Finger|[Lower Chord Scan Mode]

| 37|   ...Reserve              | ----- |-----          |00:Easy,

| 38|   ...Reserve              | ----- |-----          |01:Normal,

| 39|   ...Reserve              | ----- |-----          |02:One Finger

| 40|   ...Reserve              | ----- |-----          |

| 41|   ...Reserve              | ----- |-----          |"Lower Chord Scan Mode"

| 42|   ...Reserve              | ----- |-----          |is not effective

| 43|   ...Reserve              | ----- |-----          |at first released system.

| 44|   ...Reserve              | ----- |-----          |

| 45|   ...Reserve              | ----- |-----          |

| 46|   ...Reserve              | ----- |-----          |

| 47|   ...Reserve              | ----- |-----          |

+-------------------------------------------------------+

KBD SET PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|KBD SET 11 Parameters                                  |

|-------------------------------------------------------|

| 00|   FULL KBD PLAY           | 00,01 |Off,On         |

| 01|   Split Point             | 24~60 |C2~C7          |

| 02|   Chord Scan Parameters   |       |               |

|   |    (b7)...Reserve         | ----- | -----         |

|   |    (b6)...Reserve         | ----- | -----         |"Lower type" is effective

|   |    (b5)...(Lower Type)    | (0,1) |(Normal,Easy)  | at first released system.

|   |    (b4)...Reserve         | ----- | -----         |

|   |    (b3)...Reserve         | ----- | -----         |

|   |    (b2)...Reserve         | ----- | -----         |[Chord Scanning]

|   |    (b1)Chord Scan Upper   | 0,1   |Off,On         |00:Off    02:Upper

|   |    (b0)Chord Scan Lower   | 0,1   |Off,On         |01:Lower  03:FULL

| 03|   ...Reserve              | ----- |-----          |

| 04|   ...Reserve              | ----- |-----          |

| 05|   ...Reserve              | ----- |-----          |

| 06|   ...Reserve              | ----- |-----          |

| 07|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

| 08|   Main Vel. Win. Top      | 01~7F |001~127        |

| 09|   Main Vel. Win. Bottom   | 01~7F |001~127        |

| 10|   Sub Vel. Win. Top       | 01~7F |001~127        |

| 11|   Sub Vel. Win. Bottom    | 01~7F |001~127        |

| 12|   Lower Vel. Win. Top     | 01~7F |001~127        |

| 13|   Lower Vel. Win. Bottom  | 01~7F |001~127        |

| 14|   KBD Scale Type          | 00~29 |               |[Scale Type]

| 15|   KBD Scale Key           | 00~0B |C,C#~Bb,B      |00:Equal Temperament

|-------------------------------------------------------|~29:User Scale 32

|       MainKBD PARAMETERS                              |

|-------------------------------------------------------|

| 16|   Program No.             | 00~7F |               |

| 17|   Program Bank            | 00~04 |               |[Program No.][Program Bank]

| 18|   Volume                  | 00~7F |000~127        | Bank=00,No.=00~7F : A11~A88~B88

| 19|   Pan                     | FF~1F |Off~Prog       | Bank=01,No.=00~7F : C11~C88~D88

| 20|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       | Bank=02,No.=00~3F : E11~E88

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       | Bank=03,No.=00~7F : F11~F88~G88

| 21|   Octave                  | FE~02 |-2~+2          | Bank=04,No.=00~27 : R11~R58

| 22|   Out Status              |       |               |[Pan]

|   |    (b7)...Reserve         | ----- |-----          |FF : OFF

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |00 : L15

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |    |

|   |    (b4)...Reserve         | ----- |-----          |0F : CNT

|   |    (b3)...Reserve         | ----- |-----          |    |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |1E : R15

|   |    (b1)External Out       | 0,1   |Off,On         |1F : PROG

|   |    (b0)Internal Out       | 0,1   |Off,On         |

| 23|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       SubKBD PARAMETERS                               |

|-------------------------------------------------------|

| 24|   Program No.             | 00~7F |               |

| 25|   Program Bank            | 00~04 |               |

| 26|   Volume                  | 00~7F |000~127        |

| 27|   Pan                     | FF~1F |Off~Prog       |

| 28|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

| 29|   Octave                  | FE~02 |-2~+2          |

| 30|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

| 31|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       LowerKBD PARAMETERS                             |

|-------------------------------------------------------|

| 32|   Program No.             | 00~7F |               |

| 33|   Program Bank            | 00~04 |               |

| 34|   Volume                  | 00~7F |000~127        |

| 35|   Pan                     | FF~1F |Off~Prog       |

| 36|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

| 37|   Octave                  | FE~02 |-2~+2          |

| 38|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)Enable Damper Pedal| 0,1   |Off,On         |

|   |    (b5)Enable Joystick X  | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

| 39|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       Harmony PARAMETERS                              |

|-------------------------------------------------------|

| 40|   Program No.             | 00~7F |               |

| 41|   Program Bank            | 00~04 |               |

| 42|   Volume                  | 00~7F |000~127        |

| 43|   Pan                     | FF~1F |Off~Prog       |

| 44|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

| 45|   ...Reserve              | ----- |-----          |

| 46|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)...Reserve         | ----- |-----          |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

| 47|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       K.Bass PARAMETERS                               |

|-------------------------------------------------------|

| 48|   Program No.             | 00~7F |               |

| 49|   Program Bank            | 00~04 |               |

| 50|   Volume                  | 00~7F |000~127        |

| 51|   Pan                     | FF~1F |Off~Prog       |

| 52|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

| 53|   ...Reserve              | ----- |-----          |

| 54|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)...Reserve         | ----- |-----          |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

| 55|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|56~3583      KBD SET 12~88 (Same as 11)                |

+-------------------------------------------------------+

PROGRAM PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|Program F11 Parameters                                 |

|-------------------------------------------------------|

|000|   Program Name(1/16)      | 20~7E |" " ~ "~"      |

|001|   Program Name(2/16)      |       |(ASCII Code)   |

|002|   Program Name(3/16)      |       |               |

|003|   Program Name(4/16)      |       |               |

|004|   Program Name(5/16)      |       |               |

|005|   Program Name(6/16)      |       |               |

|006|   Program Name(7/16)      |       |               |

|007|   Program Name(8/16)      |       |               |

|008|   Program Name(9/16)      |       |               |

|009|   Program Name(10/16)     |       |               |

|010|   Program Name(11/16)     |       |               |

|011|   Program Name(12/16)     |       |               |

|012|   Program Name(13/16)     |       |               |

|013|   Program Name(14/16)     |       |               |

|014|   Program Name(15/16)     |       |               |

|015|   Program Name(16/16)     |       |               |

|016|   Program Category        | 00~0F |Keyboard~User  |

|017|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       Oscillator                                      |

|-------------------------------------------------------|

|018|   Oscillator Mode         | 00~02 |               |[Oscillator Mode]

|019|   Mono/Hold               |       |               |When F11~F88~G88 Programs

|   |    (b1)Hold               | 0,1   |Poly/Mono      |  00:Single,01:Double

|   |    (b0)Mono               | 0,1   |Off/On         |When R51~58 Programs

|020|   OSC1 Multisample(LSB)   | 0000  |0~522          |  02:Drum

|021|   OSC1 Multisample(MSB)   | ~020A | 16bit data    |

|022|   OSC2 Multisample(LSB)   | 0000  |0~522          |[Multi Sample]

|023|   OSC2 Multisample(MSB)   | ~020A | 16bit data    |When F11~F88~G88 Programs

|024|   OSC1 Octave             | FE~01 |32'/16'/8'/4'  |  000:A.Piano 1

|025|   OSC2 Octave             | FE~01 |32'/16'/8'/4'  |  ~522:Tb & Tp 3

|026|   OSC2 Transpose(Interval)| F4~0C |-12 ~+12       |When R51~58 Programs

|027|   OSC2 Detune             | CE~32 |-50~+50        |  Reserve

|028|   OSC2 Delay              | 00~63 |00~99          |

|-------------------------------------------------------|[OSC Octave]

|       Pitch EG                                        |When R51~58 Programs

|-------------------------------------------------------|  8' (Fixed)

|029|   Start Level             | 9D~63 |-99~+99        |

|030|   Attack Time             | 00~63 |00~99          |

|031|   Attack Level            | 9D~63 |-99~+99        |

|032|   Decay Time              | 00~63 |00~99          |

|033|   Release Time            | 00~63 |00~99          |

|034|   Release Level           | 9D~63 |-99~+99        |

|035|   Time Mod.Velocity       | 9D~63 |-99~+99        |

|036|   Level Velocity Sense    | 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       VDF MG                                          |

|-------------------------------------------------------|

|037|   Waveform/Enable/Sync    |       |               |

|   |    (b7)Key Sync           | 0,1   |Off,On         |

|   |    (b6)OSC2 MG Enable     | 0,1   |Off,On         |

|   |    (b5)OSC1 MG Enable     | 0,1   |Off,On         |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |[LFO Waveform]

|   |    (b2)LFO Waveform(1/3)  | 0~5   |               |xxxxx000:Triangle

|   |    (b1)LFO Waveform(2/3)  |       |               |xxxxx001:Up Saw

|   |    (b0)LFO Waveform(3/3)  |       |               |xxxxx010:Down Saw

|038|   Frequency               | 00~63 |00~99          |xxxxx011:Square 1

|039|   Delay                   | 00~63 |00~99          |xxxxx100:Random

|040|   Intensity               | 00~63 |00~99          |xxxxx101:Square 2

|-------------------------------------------------------|

|       Aftertouch                                      |

|-------------------------------------------------------|

|041|   Pitch Bend Range        | F4~0C |-12~+12        |

|042|   Cutoff Frequency        | 9D~63 |-99~+99        |

|043|   VDF MG Intensity        | 00~63 |00~99          |

|044|   OSC Level               | 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       Joystick                                        |

|-------------------------------------------------------|

|045|   Pitch Bend Range(JS X)  | F4~0C |-12~+12        |

|046|   Cutoff Frequency(JS X)  | 9D~63 |-99~+99        |

|047|   VDF MG Intensity(JS-Y)  | 00~63 |00~99          |

|-------------------------------------------------------|

|    OSC 1 Parameters                                   |

|-------------------------------------------------------|

|       OSC 1 Pitch EG                                  |

|-------------------------------------------------------|

|048|   Pitch EG Intensity      | 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       OSC 1 Pitch MG                                  |

|-------------------------------------------------------|

|049|   Waveform/Enable/Sync    |       |               |

|   |    (b7)Key Sync           | 0,1   |Off,On         |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |[LFO Waveform]

|   |    (b2)LFO Waveform(1/3)  | 0~5   |               |xxxxx000:Triangle

|   |    (b1)LFO Waveform(2/3)  |       |               |xxxxx001:Up Saw

|   |    (b0)LFO Waveform(3/3)  |       |               |xxxxx010:Down Saw

|050|   Frequency               | 00~63 |00~99          |xxxxx011:Square 1

|051|   Delay                   | 00~63 |00~99          |xxxxx100:Random

|052|   Fade In                 | 00~63 |00~99          |xxxxx101:Square 2

|053|   Intensity               | 00~63 |00~99          |

|054|   KBD Tracking(Freq.Mod.) | 9D~63 |-99~+99        |

|055|   Int.Mod. by Aftertouch  | 00~63 |00~99          |

|056|   Inr.Mod. by Joystick +Y | 00~63 |00~99          |

|057|   Freq.Control by JS & AT | 00~09 |0~9            |

|-------------------------------------------------------|

|       OSC 1 VDF                                       |

|-------------------------------------------------------|

|058|   Cutoff Frequency        | 00~63 |00~99          |

|059|   KBD Track Pivot Key     | 00~7F |C-1~G9         |

|060|   KBD Track Int.(Cutoff)  | 9D~63 |-99~+99        |

|061|   EG Intensity            | 00~63 |00~99          |

|062|   EGTime Mod. by KBDTrack | 00~63 |00~99          |

|063|   EGTime Mod. by Velocity | 00~63 |00~99          |

|064|   Velocity Sens.to EG Int.| 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       VDF EG 1                                        |

|-------------------------------------------------------|

|065|   Attack Time             | 00~63 |00~99          |

|066|   Attack Level            | 9D~63 |-99~+99        |

|067|   Decay Time              | 00~63 |00~99          |

|068|   Break Point             | 9D~63 |-99~+99        |

|069|   Slope Time              | 00~63 |00~99          |

|070|   Sustain Level           | 9D~63 |-99~+99        |

|071|   Release Time            | 00~63 |00~99          |

|072|   Release Level           | 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       OSC 1 VDA                                       |

|-------------------------------------------------------|

|073|   OSC Level               | 00~63 |00~99          |

|074|   KBD Track Pivot Key     | 00~7F |C-1~G9         |

|075|   KBD Track Int.(Level)   | 9D~63 |-99~+99        |

|076|   Velocity Sens.to Level  | 9D~63 |-99~+99        |

|077|   EGTime Mod. by KBDTrack | 00~63 |00~99          |

|078|   EGTime Mod. by Velocity | 00~63 |00~99          |

|-------------------------------------------------------|

|       VDA EG 1                                        |

|-------------------------------------------------------|

|079|   Attack Time             | 00~63 |00~99          |

|080|   Attack Level            | 00~63 |00~99          |

|081|   Decay Time              | 00~63 |00~99          |

|082|   Break Point             | 00~63 |00~99          |

|083|   Slope Time              | 00~63 |00~99          |

|084|   Sustain Level           | 00~63 |00~99          |

|085|   Release Time            | 00~63 |00~99          |

|-------------------------------------------------------|

|       OSC 1 EG Time Mod. by KBD Track & Velocity      |

|                       and Mod. SW & Polarity          |

|-------------------------------------------------------|

|086|   VDF EG 1 KBD Track      |       |               |

|   |    (b7)Release Time Pol.  | 0,1   |+,-            |

|   |    (b6)Slope Time Pol.    | 0,1   |+,-            |

|   |    (b5)Decay Time Pol.    | 0,1   |+,-            |

|   |    (b4)Attack Time Pol.   | 0,1   |+,-            |

|   |    (b3)Release Time SW    | 0,1   |"0" , "+"or"-" |

|   |    (b2)Slope Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b1)Decay Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b0)Attack Time SW     | 0,1   |"0" , "+"or"-" |

|087|   VDF EG 1 Velocity       |       |               |

|   |    (b7)Release Time Pol.  | 0,1   |+,-            |

|   |    (b6)Slope Time Pol.    | 0,1   |+,-            |

|   |    (b5)Decay Time Pol.    | 0,1   |+,-            |

|   |    (b4)Attack Time Pol.   | 0,1   |+,/-           |

|   |    (b3)Release Time SW    | 0,1   |"0" , "+"or"-" |

|   |    (b2)Slope Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b1)Decay Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b0)Attack Time SW     | 0,1   |"0" , "+"or"-" |

|088|   VDA EG 1 KBD Track      |       |               |

|   |    (b7)Release Time Pol.  | 0,1   |+,-            |

|   |    (b6)Slope Time Pol.    | 0,1   |+,-            |

|   |    (b5)Decay Time Pol.    | 0,1   |+,-            |

|   |    (b4)Attack Time Pol.   | 0,1   |+,-            |

|   |    (b3)Release Time SW    | 0,1   |"0" , "+"or"-" |

|   |    (b2)Slope Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b1)Decay Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b0)Attack Time SW     | 0,1   |"0" , "+"or"-" |

|089|   VDA EG 1 Velocity       |       |               |

|   |    (b7)Release Time Pol.  | 0,1   |+,-            |

|   |    (b6)Slope Time Pol.    | 0,1   |+,-            |

|   |    (b5)Decay Time Pol.    | 0,1   |+,-            |

|   |    (b4)Attack Time Pol.   | 0,1   |+,-            |

|   |    (b3)Release Time SW    | 0,1   |"0" , "+"or"-" |

|   |    (b2)Slope Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b1)Decay Time SW      | 0,1   |"0" , "+"or"-" |

|   |    (b0)Attack Time SW     | 0,1   |"0" , "+"or"-" |

|-------------------------------------------------------|

|       OSC 1 Send Level                                |

|-------------------------------------------------------|

|090|   (b7~b4)C Send Level     | 0~9   |0~9            |

|   |   (b3~b0)D Send Level     | 0~9   |0~9            |

|-------------------------------------------------------|

|       OSC 1 VDF Color                                 |

|-------------------------------------------------------|

|091|   Color (Intensity)       | 00~63 |00~99          |

|092|   Velocity Sens.to Color  | 9D~63 |-99~+99        |

|-------------------------------------------------------|

|       VDF 1/VDA 1 KBD Track Mode                      |

|-------------------------------------------------------|

|093|   KBD Track Mode          |       |               |

|   |    (b7~b6)...Reserve      | ----- |-----          |

|   |    (b5~b4)VDA 1           | 0~3   |Off,Low,Hi,All |

|   |    (b3~b2)...Reserve      | ----- |-----          |

|   |    (b1~b0)VDF 1           | 0~3   |Off,Low,Hi,All |

|-------------------------------------------------------|

|       OSC 1 Pan                                       |[Pan]

|-------------------------------------------------------|FF : OFF

|094|   Pan                     | FF~1E |               |00 : L15

|-------------------------------------------------------|    |

|    OSC 2 Parameters (same as OSC 1)                   |0F : CNT

|-------------------------------------------------------|    |

|095|   OSC 2 Pitch EG  same as "048 OSC 1 Pitch EG"    |1E : R15

| * |                   *                               |

| * |                   *                               |

| * |                   *                               |

|141|   OSC 2 Pan       same as "094 OSC 1 Pan"         |

|-------------------------------------------------------|

|142|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       FX PARAMETERS                                   |

|-------------------------------------------------------|

|143|   FX 1 Type               | 00~2F |               |[FX 1/2 Type]

|144|   FX 2 Type               | 00~2F |               |0:NoEffect

|145|   FX 1 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |~2F:Delay/RotarySpeaker

|146|   FX 1 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |

|147|   FX 2 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |

|148|   FX 2 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |[Effect Output C/D Pan]

|149|   Effect Output C Pan     | 00~65 |Off,R,01:99~L  |When Placement is Parallel3

|150|   Effect Output D Pan     | 00~65 |Off,R,01:99~L  |No.141(bit7~4) FX 1 Return L : 0~9

|151|   Effect I/O              |       |               |No.141(bit3~0) FX 1 Return R : 0~9

|   |    (b7)...Reserve         | ----- |-----          |No.142(bit7~4) FX 2 Return L : 0~9

|   |    (b6)Placement(1/3)     | 0~5   |               |No.142(bit3~0) FX 2 Return R : 0~9

|   |    (b5)Placement(2/3)     |       |               |

|   |    (b4)Placement(3/3)     |       |               |[Placement]

|   |    (b3)Effect 2(R) On/Off | 0,1   |Off,On         |0:Serial

|   |    (b2)Effect 2(L) On/Off | 0,1   |Off,On         |1:Parallel 1

|   |    (b1)Effect 1(R) On/Off | 0,1   |Off,On         |2:Parallel 2

|   |    (b0)Effect 1(L) On/Off | 0,1   |Off,On         |3:Parallel 3

|152|   FX 1 Parameter(1/8)     |       |               |4:Serial Sub

|153|   FX 1 Parameter(2/8)     |       |               |

|154|   FX 1 Parameter(3/8)     |       |               |

|155|   FX 1 Parameter(4/8)     |       |               |

|156|   FX 1 Parameter(5/8)     |       |               |

|157|   FX 1 Parameter(6/8)     |       |               |

|158|   FX 1 Parameter(7/8)     |       |               |

|159|   FX 1 Parameter(8/8)     |       |               |

|160|   FX 1 D-Mod.Source       | 00~06 |None~VDA EG    |

|161|   FX 1 D-Mod.Amount       | F1~OF |-15~+15        |

|162|   FX 2 Parameter(1/8)     |       |               |

|163|   FX 2 Parameter(2/8)     |       |               |

|164|   FX 2 Parameter(3/8)     |       |               |

|165|   FX 2 Parameter(4/8)     |       |               |

|166|   FX 2 Parameter(5/8)     |       |               |

|167|   FX 2 Parameter(6/8)     |       |               |

|168|   FX 2 Parameter(7/8)     |       |               |

|169|   FX 2 Parameter(8/8)     |       |               |

|170|   FX 2 D-Mod.Source       | 00~06 |None~VDA EG    |

|171|   FX 2 D-Mod.Amount       | F1~OF |-15~+15        |

|-------------------------------------------------------|

|172~23391  Program F12~F88,G11~G88,R51~58 (Same as F11)|

+-------------------------------------------------------+

DRUM KIT PARAMETERS (R BANK)

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|User Kit R51 A0 Key's Parameters                       |

|-------------------------------------------------------|

|  0|   Drumsample(LSB)         | 0000  |0~405          |

|  1|   Drumsample(MSB)         | ~0195 | 16bit data    |[Exclusive Group]

|  2|   Exclusive Group         | 0~11  |               |00   :None

|  3|   Pan                     | FF~1E |Off,L15~R15    |01~16:1~16

|  4|   Detune                  | CE~32 |-50~+50        |17   :Self

|  5|   Transpose               | E8~18 |-24~+24        |

|  6|   Adjust Level            | 9D~63 |-99~+99        |

|  7|   ...Reserve (Fc)         | ----- |-----          |

|  8|   Adjust Decay Time       | 9D~63 |99~+99         |

|  9|   (b7~b4)C Send Level     | 0~9   |0~9            |

|   |   (b3~b0)D Send Level     | 0~9   |0~9            |

|-------------------------------------------------------|

|010~879    A#0~C8 Key's Parameters                     |

|-------------------------------------------------------|

|880~7039   R52~R58 User Kit Parameters                 |

|             (same as User Kit R51)                    |

+-------------------------------------------------------+

GLOBAL PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

| 00|   Master Tune             | CE~32 |-50~+50        |

| 01|   ...Reserve              | ----- |-----          |

| 02|   Damper Porality         | 00,01 |Reverse,Korg   |

| 03|   Assignable Pedal 1      | 00~40 |Off~Data Entry |

| 04|   Assignable Pedal 2      | 00~40 |Off~Data Entry |

| 05|   ...Reserve              | ----- |-----          |

| 06|   Global MIDI Channel     | 00~0F |1~16           |

| 07|   ...Reserve              | ----- |-----          |

| 08|   Status                  | ----- |-----          |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)MIDI EXT Clock     | 0,1   |MIDI IN,To Host|

|   |    (b5)Host Baud Rate     | 0,1   |38.4k,31.25k   |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)XposePosition(1/2) | 00~02 |               |[Transpose Position]

|   |    (b2)XposePosition(2/2) |       |2:PreOSC & MIDI|0:Post-KBD

|   |    (b1)MIDI Clock Int/Ext | 0,1   |Internal,EXT   |1:Pre-Oscillator

|   |    (b0)Local Control      | 0,1   |Off,On         |2:Pre-Oscillator & MIDI OUT

| 09|   MIDI Filter             |       |               |

|   |    (b7)Discard KORG GM    | 0,1   |No,Yes         |[Assign. Pedal 1,2][EC5 A~E][SW 1,2]

|   |    (b6)Note No.Filter(1/2)| 1     |               |00:Off                  20:SYNCHRO STOP

|   |    (b5)Note No.Filter(2/2)| 0     |               |01:DRUMS PLAY/MUTE      21:RIT.

|   |    (b4)Exclusive          | 0,1   |Dis,Ena        |02:PERC PLAY/MUTE       22:ACCEL.

|   |    (b3)Control Change     | 0,1   |Dis,Ena        |03:BASS PLAY/MUTE       23:TRANSPOSE(b)

|   |    (b2)Aftertouch         | 0,1   |Dis,Ena        |04:ACC1 PLAY/MUTE       24:TRANSPOSE(#)

|   |    (b1)ProgBank Change    | 0,1   |Dis,Ena        |05:ACC2 PLAY/MUTE       25:OCTAVE DOWN

|   |    (b0)Program Change     | 0,1   |Dis,Ena        |06:ACC3 PLAY/MUTE       26:OCTAVE UP

| 10|   EC5 Pedal "A"           | 00~28 |Off~C.L.+Damper|07:LOWER KBD PLAY/MUTE  27:RESET

| 11|   EC5 Pedal "B"           | 00~28 |Off~C.L.+Damper|08:SUB KBD PLAY/MUTE    28:TAP TEMPO

| 12|   EC5 Pedal "C"           | 00~28 |Off~C.L.+Damper|09:MAIN KBD PLAY/MUTE   29:Arrange/Style Up

| 13|   EC5 Pedal "D"           | 00~28 |Off~C.L.+Damper|0A:CHORD MEMORY         2A:Arrange/Style Down

| 14|   EC5 Pedal "E"           | 00~28 |Off~C.L.+Damper|0B:CHORD SOUND          2B:Program Up

| 15|   SW 1                    | 00~28 |Off~C.L.+Damper|0C:BASS INVERSION       2C:Program Down

| 16|   SW 2                    | 00~28 |Off~C.L.+Damper|0D:ENSEMBLE             2D:KBD Set Up

| 17|   Velocity Curve          | 00~08 |1~9            |OE:TEMPO LOCK           2E:KBD Set Down

| 18|   Aftertouch Curve        | 00~07 |1~8            |0F:KBD LOCK             2F:Variation Up

| 19|   rit.type                | 00~06 |1~7            |10:FULL KBD PLAY        30:Variation Down

| 20|   rit.beat                | 00~1F |1~32           |11:VARIATION 1          31:Scale Change

| 21|   accel.type              | 00~06 |1~7            |12:VARIATION 2          32:FX 1 On/Off

| 22|   accel.beat              | 00~1F |1~32           |13:VARIATION 3          33:FX 2 On/Off

| 23|   ...Reserve              | ----- |-----          |14:VARIATION 4          34:Damper

| 24|   ...Reserve              | ----- |-----          |15:FILL 1               35:Chord Latch

| 25|   ...Reserve              | ----- |-----          |16:FILL 2               36:Chord Latch+Damper

| 26|   ...Reserve              | ----- |-----          |17:INTRO/ENDING 1       37:Master Volume

| 27|   ...Reserve              | ----- |-----          |18:INTRO/ENDING 2       38:KBD Expression

| 28|   Special Scale Type      | 00~29 |               |19:Intro 1              39:VDF Cutoff

| 29|   Special Scale Key       | 00~0B |C,C#~Bb,B      |1A:Intro 2              3A:Joystick +X(BSEND)

| 30|   Normal Scale Type       | 00~29 |               |1B:Break                3B:Joystick -X(BSEND)

| 31|   Normal Scale Key        | 00~0B |C,C#~Bb,B      |1C:START/STOP           3C:Joystick +Y(CC#1)

| 32|   Userscale 1  [C ]       | CE~32 |-50~+50        |1D:REC/WRITE            3D:Joystick -Y(CC#2)

| 33|                [C#]       | CE~32 |-50~+50        |1E:FADE IN/OUT          3E:Effect Control 1(CC#12)

| 34|                [D ]       | CE~32 |-50~+50        |1F:SYNCHRO START        3F:Effect Control 2(CC#13)

| 35|                [Eb]       | CE~32 |-50~+50        |                        4F:Data Entry

| 36|                [E ]       | CE~32 |-50~+50        |[Scale Type]

| 37|                [F ]       | CE~32 |-50~+50        |00:Equal Temperament

| 38|                [F#]       | CE~32 |-50~+50        |~29:User Scale 32

| 39|                [G ]       | CE~32 |-50~+50        |

| 40|                [Ab]       | CE~32 |-50~+50        |

| 41|                [A ]       | CE~32 |-50~+50        |

| 42|                [Bb]       | CE~32 |-50~+50        |

| 43|                [B ]       | CE~32 |-50~+50        |

|-------------------------------------------------------|

|44~55          Userscale 2  [C ]~[B ]                  |

|-------------------------------------------------------|

|56~67          Userscale 3  [C ]~[B ]                  |

|-------------------------------------------------------|

|68~79          Userscale 4  [C ]~[B ]                  |

|-------------------------------------------------------|

|80~91          Userscale 5  [C ]~[B ]                  |

|-------------------------------------------------------|

|92~103         Userscale 6  [C ]~[B ]                  |

|-------------------------------------------------------|

|104~115        Userscale 7  [C ]~[B ]                  |

|-------------------------------------------------------|

|116~127        Userscale 8  [C ]~[B ]                  |

|-------------------------------------------------------|

|128~139        Userscale 9  [C ]~[B ]                  |

|-------------------------------------------------------|

|140~151        Userscale 10 [C ]~[B ]                  |

|-------------------------------------------------------|

|152~163        Userscale 11 [C ]~[B ]                  |

|-------------------------------------------------------|

|164~175        Userscale 12 [C ]~[B ]                  |

|-------------------------------------------------------|

|176~187        Userscale 13 [C ]~[B ]                  |

|-------------------------------------------------------|

|188~199        Userscale 14 [C ]~[B ]                  |

|-------------------------------------------------------|

|200~211        Userscale 15 [C ]~[B ]                  |

|-------------------------------------------------------|

|212~223        Userscale 16 [C ]~[B ]                  |

|-------------------------------------------------------|

|224~235        Userscale 17 [C ]~[B ]                  |

|-------------------------------------------------------|

|236~247        Userscale 18 [C ]~[B ]                  |

|-------------------------------------------------------|

|248~259        Userscale 19 [C ]~[B ]                  |

|-------------------------------------------------------|

|260~271        Userscale 20 [C ]~[B ]                  |

|-------------------------------------------------------|

|272~283        Userscale 21 [C ]~[B ]                  |

|-------------------------------------------------------|

|284~295        Userscale 22 [C ]~[B ]                  |

|-------------------------------------------------------|

|296~307        Userscale 23 [C ]~[B ]                  |

|-------------------------------------------------------|

|308~319        Userscale 24 [C ]~[B ]                  |

|-------------------------------------------------------|

|320~331        Userscale 25 [C ]~[B ]                  |

|-------------------------------------------------------|

|332~343        Userscale 26 [C ]~[B ]                  |

|-------------------------------------------------------|

|344~355        Userscale 27 [C ]~[B ]                  |

|-------------------------------------------------------|

|356~367        Userscale 28 [C ]~[B ]                  |

|-------------------------------------------------------|

|368~379        Userscale 29 [C ]~[B ]                  |

|-------------------------------------------------------|

|380~391        Userscale 30 [C ]~[B ]                  |

|-------------------------------------------------------|

|392~403        Userscale 31 [C ]~[B ]                  |

|-------------------------------------------------------|

|404~415        Userscale 32 [C ]~[B ]                  |

+-------------------------------------------------------+

STYLE PARAMETERS

STYLE HEADER

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       Style C*1 or C*5                                |

|-------------------------------------------------------|

| 00|   Data Address LSB        |       |               |

| 01|   Data Address MSB        |       |               |

| 02|   ...Reserve(Segment LSB) | 00    |               |

| 03|   ...Reserve(Segment MSB) | 00    |               |

| 04|   Size LSB                |       |               |

| 05|   Size MSB                |       |               |

|-------------------------------------------------------|

|       Style C*2 or C*6                                |

|-------------------------------------------------------|

| 06|   Data Address LSB        |       |               |

| 07|   Data Address MSB        |       |               |

| 08|   ...Reserve(Segment LSB) | 00    |               |

| 09|   ...Reserve(Segment MSB) | 00    |               |

| 10|   Size LSB                |       |               |

| 11|   Size MSB                |       |               |

|-------------------------------------------------------|

|       Style C*3 or C*7                                |

|-------------------------------------------------------|

| 12|   Data Address LSB        |       |               |

| 13|   Data Address MSB        |       |               |

| 14|   ...Reserve(Segment LSB) | 00    |               |

| 15|   ...Reserve(Segment MSB) | 00    |               |

| 16|   Size LSB                |       |               |

| 17|   Size MSB                |       |               |

|-------------------------------------------------------|

|       Style C*4 or C*8                                |

|-------------------------------------------------------|

| 18|   Data Address LSB        |       |               |

| 19|   Data Address MSB        |       |               |

| 20|   ...Reserve(Segment LSB) | 00    |               |

| 21|   ...Reserve(Segment MSB) | 00    |               |

| 22|   Size LSB                |       |               |

| 23|   Size MSB                |       |               |

|-------------------------------------------------------|

|24~31          Reserve                                 |

+-------------------------------------------------------+

STYLE CONTROL DATA

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|000|   Style Name(1/16)        | 20~7E |" " ~ "~"      |

|001|   Style Name(2/16)        |       |(ASCII Code)   |

|002|   Style Name(3/16)        |       |               |

|003|   Style Name(4/16)        |       |               |

|004|   Style Name(5/16)        |       |               |

|005|   Style Name(6/16)        |       |               |

|006|   Style Name(7/16)        |       |               |

|007|   Style Name(8/16)        |       |               |

|008|   Style Name(9/16)        |       |               |

|009|   Style Name(10/16)       |       |               |

|010|   Style Name(11/16)       |       |               |

|011|   Style Name(12/16)       |       |               |

|012|   Style Name(13/16)       |       |               |

|013|   Style Name(14/16)       |       |               |

|014|   Style Name(15/16)       |       |               |

|015|   Style Name(16/16)       |       |               |

|016|   Style Category          | 00~0F |8 Beat~User 4  |

|017|   ...Reserve              | ----- |-----          |

|018|   ...Reserve(Type)        | 0     |User Created   |

|019|   Tempo                   | 0A~D2 |40~240         |[Meter]

|020|   Meter                   | 10~3F |               |10~14:1/4~5/4

|-------------------------------------------------------|20~29:1/8~10/8

|021|   Note Retrigger Switch   |       |               |30~3F:1/16~16/16

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)ACC3               | 0,1   |Off,On         |

|   |    (b4)ACC2               | 0,1   |Off,On         |

|   |    (b3)ACC1               | 0,1   |Off,On         |

|   |    (b2)Bass               | 0,1   |Off,On         |

|   |    (b1)...Reserve         | ----- |-----          |

|   |    (b0)...Reserve         | ----- |-----          |

|-------------------------------------------------------|

|       Wrap-Around Point(Note Shift Up Range)          |

|-------------------------------------------------------|

|022|   Bass                    | 00~0B |1~12           |

|023|   ACC1                    | 00~0B |1~12           |

|024|   ACC2                    | 00~0B |1~12           |

|025|   ACC3                    | 00~0B |1~12           |

|-------------------------------------------------------|

|026|   Tension Available       |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)ACC3               | 0,1   |Off,On         |

|   |    (b4)ACC2               | 0,1   |Off,On         |

|   |    (b3)ACC1               | 0,1   |Off,On         |

|   |    (b2)...Reserve         | ----- |-----          |

|   |    (b1)...Reserve         | ----- |-----          |

|   |    (b0)...Reserve         | ----- |-----          |

|-------------------------------------------------------|

|027|   ...Reserve              | ----- |-----          |

|028|   ...Reserve              | ----- |-----          |

|029|   ...Reserve              | ----- |-----          |

|030|   ...Reserve              | ----- |-----          |

|031|   ...Reserve              | ----- |-----          |

|032|   ...Reserve              | ----- |-----          |

|033|   ...Reserve              | ----- |-----          |

|034|   ...Reserve              | ----- |-----          |

|035|   ...Reserve              | ----- |-----          |

|036|   ...Reserve              | ----- |-----          |

|037|   ...Reserve              | ----- |-----          |

|038|   ...Reserve              | ----- |-----          |

|039|   ...Reserve              | ----- |-----          |

|040|   ...Reserve              | ----- |-----          |

|041|   ...Reserve              | ----- |-----          |

|042|   ...Reserve              | ----- |-----          |

|043|   ...Reserve              | ----- |-----          |

|044|   ...Reserve              | ----- |-----          |

|045|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       DRUMS PARAMETERS                                |

|-------------------------------------------------------|

|046|   Program No.             | 00~7F |               |[Program No.][Program Bank]

|047|   Program Bank            | 00~04 |               | Bank=00,No.=00~7F : A11~A88~B88

|048|   Volume                  | 00~7F |000~127        | Bank=01,No.=00~7F : C11~C88~D88

|049|   Pan                     | FF~1F |Off~Prog       | Bank=02,No.=00~3F : E11~E88

|-------------------------------------------------------| Bank=03,No.=00~7F : F11~F88~G88

|       PERC PARAMETERS                                 | Bank=04,No.=00~27 : R11~R58

|-------------------------------------------------------|[Pan]

|050|   Program No.             | 00~7F |               |FF : OFF

|051|   Program Bank            | 00~04 |               |00 : L15

|052|   Volume                  | 00~7F |000~127        |    |

|053|   Pan                     | FF~1F |Off~Prog       |0F : CNT

|-------------------------------------------------------|    |

|       BASS PARAMETERS                                 |1E : R15

|-------------------------------------------------------|1F : PROG

|054|   Program No.             | 00~7F |               |

|055|   Program Bank            | 00~04 |               |

|056|   Volume                  | 00~7F |000~127        |

|057|   Pan                     | FF~1F |Off~Prog       |

|-------------------------------------------------------|

|       ACC1 PARAMETERS                                 |

|-------------------------------------------------------|

|058|   Program No.             | 00~7F |               |

|059|   Program Bank            | 00~04 |               |

|060|   Volume                  | 00~7F |000~127        |

|061|   Pan                     | FF~1F |Off~Prog       |

|-------------------------------------------------------|

|       ACC2 PARAMETERS                                 |

|-------------------------------------------------------|

|062|   Program No.             | 00~7F |               |

|063|   Program Bank            | 00~04 |               |

|064|   Volume                  | 00~7F |000~127        |

|065|   Pan                     | FF~1F |Off~Prog       |

|-------------------------------------------------------|

|       ACC3 PARAMETERS                                 |

|-------------------------------------------------------|

|066|   Program No.             | 00~7F |               |

|067|   Program Bank            | 00~04 |               |

|068|   Volume                  | 00~7F |000~127        |

|069|   Pan                     | FF~1F |Off~Prog       |

|-------------------------------------------------------|

|       Variation 1,CV 1 Parameters                     |

|-------------------------------------------------------|

|070|   Key                     | 00~17 |Cmaj~Bmin      |[Key]

|071|   Length                  | 01~10 |1~16           |0:Cmaj,1:Cmin

|-------------------------------------------------------|~22:Bmaj,23:Bmin

|       Variation 1,CV 2 Parameters                     |

|-------------------------------------------------------|

|072|   Key                     | 00~17 |Cmaj~Bmin      |

|073|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 1,CV 3 Parameters                     |

|-------------------------------------------------------|

|074|   Key                     | 00~17 |Cmaj~Bmin      |

|075|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 1,CV 4 Parameters                     |

|-------------------------------------------------------|

|076|   Key                     | 00~17 |Cmaj~Bmin      |

|077|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 1,CV 5 Parameters                     |

|-------------------------------------------------------|

|078|   Key                     | 00~17 |Cmaj~Bmin      |

|079|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 1,CV 6 Parameters                     |

|-------------------------------------------------------|

|080|   Key                     | 00~17 |Cmaj~Bmin      |

|081|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 1 Parameters                     |

|-------------------------------------------------------|

|082|   Key                     | 00~17 |Cmaj~Bmin      |

|083|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 2 Parameters                     |

|-------------------------------------------------------|

|084|   Key                     | 00~17 |Cmaj~Bmin      |

|085|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 3 Parameters                     |

|-------------------------------------------------------|

|086|   Key                     | 00~17 |Cmaj~Bmin      |

|087|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 4 Parameters                     |

|-------------------------------------------------------|

|088|   Key                     | 00~17 |Cmaj~Bmin      |

|089|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 5 Parameters                     |

|-------------------------------------------------------|

|090|   Key                     | 00~17 |Cmaj~Bmin      |

|091|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 2,CV 6 Parameters                     |

|-------------------------------------------------------|

|092|   Key                     | 00~17 |Cmaj~Bmin      |

|093|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 1 Parameters                     |

|-------------------------------------------------------|

|094|   Key                     | 00~17 |Cmaj~Bmin      |

|095|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 2 Parameters                     |

|-------------------------------------------------------|

|096|   Key                     | 00~17 |Cmaj~Bmin      |

|097|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 3 Parameters                     |

|-------------------------------------------------------|

|098|   Key                     | 00~17 |Cmaj~Bmin      |

|099|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 4 Parameters                     |

|-------------------------------------------------------|

|100|   Key                     | 00~17 |Cmaj~Bmin      |

|101|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 5 Parameters                     |

|-------------------------------------------------------|

|102|   Key                     | 00~17 |Cmaj~Bmin      |

|103|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 3,CV 6 Parameters                     |

|-------------------------------------------------------|

|104|   Key                     | 00~17 |Cmaj~Bmin      |

|105|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 1 Parameters                     |

|-------------------------------------------------------|

|106|   Key                     | 00~17 |Cmaj~Bmin      |

|107|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 2 Parameters                     |

|-------------------------------------------------------|

|108|   Key                     | 00~17 |Cmaj~Bmin      |

|109|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 3 Parameters                     |

|-------------------------------------------------------|

|110|   Key                     | 00~17 |Cmaj~Bmin      |

|111|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 4 Parameters                     |

|-------------------------------------------------------|

|112|   Key                     | 00~17 |Cmaj~Bmin      |

|113|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 5 Parameters                     |

|-------------------------------------------------------|

|114|   Key                     | 00~17 |Cmaj~Bmin      |

|115|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 4,CV 6 Parameters                     |

|-------------------------------------------------------|

|116|   Key                     | 00~17 |Cmaj~Bmin      |

|117|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Intro 1,CV 1 Parameters                         |

|-------------------------------------------------------|

|118|   Key                     | 00~17 |Cmaj~Bmin      |

|119|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Intro 1,CV 2 Parameters                         |

|-------------------------------------------------------|

|120|   Key                     | 00~17 |Cmaj~Bmin      |

|121|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Intro 2,CV 1 Parameters                         |

|-------------------------------------------------------|

|122|   Key                     | 00~17 |Cmaj~Bmin      |

|123|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Intro 2,CV 2 Parameters                         |

|-------------------------------------------------------|

|124|   Key                     | 00~17 |Cmaj~Bmin      |

|125|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Ending 1,CV 1 Parameters                        |

|-------------------------------------------------------|

|126|   Key                     | 00~17 |Cmaj~Bmin      |

|127|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Ending 1,CV 2 Parameters                        |

|-------------------------------------------------------|

|128|   Key                     | 00~17 |Cmaj~Bmin      |

|129|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Ending 2,CV 1 Parameters                        |

|-------------------------------------------------------|

|130|   Key                     | 00~17 |Cmaj~Bmin      |

|131|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Ending 2,CV 2 Parameters                        |

|-------------------------------------------------------|

|132|   Key                     | 00~17 |Cmaj~Bmin      |

|133|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Fill 1,CV 1 Parameters                          |

|-------------------------------------------------------|

|134|   Key                     | 00~17 |Cmaj~Bmin      |

|135|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Fill 1,CV 2 Parameters                          |

|-------------------------------------------------------|

|136|   Key                     | 00~17 |Cmaj~Bmin      |

|137|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Fill 2,CV 1 Parameters                          |

|-------------------------------------------------------|

|138|   Key                     | 00~17 |Cmaj~Bmin      |

|139|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Fill 2,CV 2 Parameters                          |

|-------------------------------------------------------|

|140|   Key                     | 00~17 |Cmaj~Bmin      |

|141|   Length                  | 01~10 |1~16           |

|-------------------------------------------------------|

|       Variation 1 Chord Variation Table               |

|-------------------------------------------------------|

|142|   Major                   |0~5    |CV1~CV6        |

|143|   6                       |0~5    |CV1~CV6        |

|144|   M7                      |0~5    |CV1~CV6        |

|145|   M7b5                    |0~5    |CV1~CV6        |

|146|   sus4                    |0~5    |CV1~CV6        |

|147|   sus2                    |0~5    |CV1~CV6        |

|148|   M7sus4                  |0~5    |CV1~CV6        |

|149|   m                       |0~5    |CV1~CV6        |

|150|   m6                      |0~5    |CV1~CV6        |

|151|   m7                      |0~5    |CV1~CV6        |

|152|   m7b5                    |0~5    |CV1~CV6        |

|153|   mM7                     |0~5    |CV1~CV6        |

|154|   7                       |0~5    |CV1~CV6        |

|155|   7b5                     |0~5    |CV1~CV6        |

|156|   7sus4                   |0~5    |CV1~CV6        |

|157|   dim                     |0~5    |CV1~CV6        |

|158|   dimM7                   |0~5    |CV1~CV6        |

|159|   aug                     |0~5    |CV1~CV6        |

|160|   aug7                    |0~5    |CV1~CV6        |

|161|   augM7                   |0~5    |CV1~CV6        |

|-------------------------------------------------------|

|       Variation 2 Chord Variation Table               |

|-------------------------------------------------------|

|162|   Major                   |0~5    |CV1~CV6        |

|163|   6                       |0~5    |CV1~CV6        |

|164|   M7                      |0~5    |CV1~CV6        |

|165|   M7b5                    |0~5    |CV1~CV6        |

|166|   sus4                    |0~5    |CV1~CV6        |

|167|   sus2                    |0~5    |CV1~CV6        |

|168|   M7sus4                  |0~5    |CV1~CV6        |

|169|   m                       |0~5    |CV1~CV6        |

|170|   m6                      |0~5    |CV1~CV6        |

|171|   m7                      |0~5    |CV1~CV6        |

|172|   m7b5                    |0~5    |CV1~CV6        |

|173|   mM7                     |0~5    |CV1~CV6        |

|174|   7                       |0~5    |CV1~CV6        |

|175|   7b5                     |0~5    |CV1~CV6        |

|176|   7sus4                   |0~5    |CV1~CV6        |

|177|   dim                     |0~5    |CV1~CV6        |

|178|   dimM7                   |0~5    |CV1~CV6        |

|179|   aug                     |0~5    |CV1~CV6        |

|180|   aug7                    |0~5    |CV1~CV6        |

|181|   augM7                   |0~5    |CV1~CV6        |

|-------------------------------------------------------|

|       Variation 3 Chord Variation Table               |

|-------------------------------------------------------|

|182|   Major                   |0~5    |CV1~CV6        |

|183|   6                       |0~5    |CV1~CV6        |

|184|   M7                      |0~5    |CV1~CV6        |

|185|   M7b5                    |0~5    |CV1~CV6        |

|186|   sus4                    |0~5    |CV1~CV6        |

|187|   sus2                    |0~5    |CV1~CV6        |

|188|   M7sus4                  |0~5    |CV1~CV6        |

|189|   m                       |0~5    |CV1~CV6        |

|190|   m6                      |0~5    |CV1~CV6        |

|191|   m7                      |0~5    |CV1~CV6        |

|192|   m7b5                    |0~5    |CV1~CV6        |

|193|   mM7                     |0~5    |CV1~CV6        |

|194|   7                       |0~5    |CV1~CV6        |

|195|   7b5                     |0~5    |CV1~CV6        |

|196|   7sus4                   |0~5    |CV1~CV6        |

|197|   dim                     |0~5    |CV1~CV6        |

|198|   dimM7                   |0~5    |CV1~CV6        |

|199|   aug                     |0~5    |CV1~CV6        |

|200|   aug7                    |0~5    |CV1~CV6        |

|201|   augM7                   |0~5    |CV1~CV6        |

|-------------------------------------------------------|

|       Variation 4 Chord Variation Table               |

|-------------------------------------------------------|

|202|   Major                   |0~5    |CV1~CV6        |

|203|   6                       |0~5    |CV1~CV6        |

|204|   M7                      |0~5    |CV1~CV6        |

|205|   M7b5                    |0~5    |CV1~CV6        |

|206|   sus4                    |0~5    |CV1~CV6        |

|207|   sus2                    |0~5    |CV1~CV6        |

|208|   M7sus4                  |0~5    |CV1~CV6        |

|209|   m                       |0~5    |CV1~CV6        |

|210|   m6                      |0~5    |CV1~CV6        |

|211|   m7                      |0~5    |CV1~CV6        |

|212|   m7b5                    |0~5    |CV1~CV6        |

|213|   mM7                     |0~5    |CV1~CV6        |

|214|   7                       |0~5    |CV1~CV6        |

|215|   7b5                     |0~5    |CV1~CV6        |

|216|   7sus4                   |0~5    |CV1~CV6        |

|217|   dim                     |0~5    |CV1~CV6        |

|218|   dimM7                   |0~5    |CV1~CV6        |

|219|   aug                     |0~5    |CV1~CV6        |

|220|   aug7                    |0~5    |CV1~CV6        |

|221|   augM7                   |0~5    |CV1~CV6        |

|-------------------------------------------------------|

|       Intro 1 Chord Variation Table                   |

|-------------------------------------------------------|

|222|   Major                   |0~1    |CV1~CV2        |

|223|   6                       |0~1    |CV1~CV2        |

|224|   M7                      |0~1    |CV1~CV2        |

|225|   M7b5                    |0~1    |CV1~CV2        |

|226|   sus4                    |0~1    |CV1~CV2        |

|227|   sus2                    |0~1    |CV1~CV2        |

|228|   M7sus4                  |0~1    |CV1~CV2        |

|229|   m                       |0~1    |CV1~CV2        |

|230|   m6                      |0~1    |CV1~CV2        |

|231|   m7                      |0~1    |CV1~CV2        |

|232|   m7b5                    |0~1    |CV1~CV2        |

|233|   mM7                     |0~1    |CV1~CV2        |

|234|   7                       |0~1    |CV1~CV2        |

|235|   7b5                     |0~1    |CV1~CV2        |

|236|   7sus4                   |0~1    |CV1~CV2        |

|237|   dim                     |0~1    |CV1~CV2        |

|238|   dimM7                   |0~1    |CV1~CV2        |

|239|   aug                     |0~1    |CV1~CV2        |

|240|   aug7                    |0~1    |CV1~CV2        |

|241|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Intro 2 Chord Variation Table                   |

|-------------------------------------------------------|

|242|   Major                   |0~1    |CV1~CV2        |

|243|   6                       |0~1    |CV1~CV2        |

|244|   M7                      |0~1    |CV1~CV2        |

|245|   M7b5                    |0~1    |CV1~CV2        |

|246|   sus4                    |0~1    |CV1~CV2        |

|247|   sus2                    |0~1    |CV1~CV2        |

|248|   M7sus4                  |0~1    |CV1~CV2        |

|249|   m                       |0~1    |CV1~CV2        |

|250|   m6                      |0~1    |CV1~CV2        |

|251|   m7                      |0~1    |CV1~CV2        |

|252|   m7b5                    |0~1    |CV1~CV2        |

|253|   mM7                     |0~1    |CV1~CV2        |

|254|   7                       |0~1    |CV1~CV2        |

|255|   7b5                     |0~1    |CV1~CV2        |

|256|   7sus4                   |0~1    |CV1~CV2        |

|257|   dim                     |0~1    |CV1~CV2        |

|258|   dimM7                   |0~1    |CV1~CV2        |

|259|   aug                     |0~1    |CV1~CV2        |

|260|   aug7                    |0~1    |CV1~CV2        |

|261|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Ending 1 Chord Variation Table                  |

|-------------------------------------------------------|

|262|   Major                   |0~1    |CV1~CV2        |

|263|   6                       |0~1    |CV1~CV2        |

|264|   M7                      |0~1    |CV1~CV2        |

|265|   M7b5                    |0~1    |CV1~CV2        |

|266|   sus4                    |0~1    |CV1~CV2        |

|267|   sus2                    |0~1    |CV1~CV2        |

|268|   M7sus4                  |0~1    |CV1~CV2        |

|269|   m                       |0~1    |CV1~CV2        |

|270|   m6                      |0~1    |CV1~CV2        |

|271|   m7                      |0~1    |CV1~CV2        |

|272|   m7b5                    |0~1    |CV1~CV2        |

|273|   mM7                     |0~1    |CV1~CV2        |

|274|   7                       |0~1    |CV1~CV2        |

|275|   7b5                     |0~1    |CV1~CV2        |

|276|   7sus4                   |0~1    |CV1~CV2        |

|277|   dim                     |0~1    |CV1~CV2        |

|278|   dimM7                   |0~1    |CV1~CV2        |

|279|   aug                     |0~1    |CV1~CV2        |

|280|   aug7                    |0~1    |CV1~CV2        |

|281|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Ending 2 Chord Variation Table                  |

|-------------------------------------------------------|

|282|   Major                   |0~1    |CV1~CV2        |

|283|   6                       |0~1    |CV1~CV2        |

|284|   M7                      |0~1    |CV1~CV2        |

|285|   M7b5                    |0~1    |CV1~CV2        |

|286|   sus4                    |0~1    |CV1~CV2        |

|287|   sus2                    |0~1    |CV1~CV2        |

|288|   M7sus4                  |0~1    |CV1~CV2        |

|289|   m                       |0~1    |CV1~CV2        |

|290|   m6                      |0~1    |CV1~CV2        |

|291|   m7                      |0~1    |CV1~CV2        |

|292|   m7b5                    |0~1    |CV1~CV2        |

|293|   mM7                     |0~1    |CV1~CV2        |

|294|   7                       |0~1    |CV1~CV2        |

|295|   7b5                     |0~1    |CV1~CV2        |

|296|   7sus4                   |0~1    |CV1~CV2        |

|297|   dim                     |0~1    |CV1~CV2        |

|298|   dimM7                   |0~1    |CV1~CV2        |

|299|   aug                     |0~1    |CV1~CV2        |

|300|   aug7                    |0~1    |CV1~CV2        |

|301|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Fill 1 Chord Variation Table                    |

|-------------------------------------------------------|

|302|   Major                   |0~1    |CV1~CV2        |

|303|   6                       |0~1    |CV1~CV2        |

|304|   M7                      |0~1    |CV1~CV2        |

|305|   M7b5                    |0~1    |CV1~CV2        |

|306|   sus4                    |0~1    |CV1~CV2        |

|307|   sus2                    |0~1    |CV1~CV2        |

|308|   M7sus4                  |0~1    |CV1~CV2        |

|309|   m                       |0~1    |CV1~CV2        |

|310|   m6                      |0~1    |CV1~CV2        |

|311|   m7                      |0~1    |CV1~CV2        |

|312|   m7b5                    |0~1    |CV1~CV2        |

|313|   mM7                     |0~1    |CV1~CV2        |

|314|   7                       |0~1    |CV1~CV2        |

|315|   7b5                     |0~1    |CV1~CV2        |

|316|   7sus4                   |0~1    |CV1~CV2        |

|317|   dim                     |0~1    |CV1~CV2        |

|318|   dimM7                   |0~1    |CV1~CV2        |

|319|   aug                     |0~1    |CV1~CV2        |

|320|   aug7                    |0~1    |CV1~CV2        |

|321|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Fill 2 Chord Variation Table                    |

|-------------------------------------------------------|

|322|   Major                   |0~1    |CV1~CV2        |

|323|   6                       |0~1    |CV1~CV2        |

|324|   M7                      |0~1    |CV1~CV2        |

|325|   M7b5                    |0~1    |CV1~CV2        |

|326|   sus4                    |0~1    |CV1~CV2        |

|327|   sus2                    |0~1    |CV1~CV2        |

|328|   M7sus4                  |0~1    |CV1~CV2        |

|329|   m                       |0~1    |CV1~CV2        |

|330|   m6                      |0~1    |CV1~CV2        |

|331|   m7                      |0~1    |CV1~CV2        |

|332|   m7b5                    |0~1    |CV1~CV2        |

|333|   mM7                     |0~1    |CV1~CV2        |

|334|   7                       |0~1    |CV1~CV2        |

|335|   7b5                     |0~1    |CV1~CV2        |

|336|   7sus4                   |0~1    |CV1~CV2        |

|337|   dim                     |0~1    |CV1~CV2        |

|338|   dimM7                   |0~1    |CV1~CV2        |

|339|   aug                     |0~1    |CV1~CV2        |

|340|   aug7                    |0~1    |CV1~CV2        |

|341|   augM7                   |0~1    |CV1~CV2        |

|-------------------------------------------------------|

|       Pattern 00, Control Data                        |

|-------------------------------------------------------|[Meter]

|342|   Meter                   | 10~3F |               |10~14:1/4~5/4

|343|   Length                  | 01~10 |1~16           |20~29:1/8~10/8

|-------------------------------------------------------|30~3F:1/16~16/16

|344~541        Pattern 01~99, Control Data (Same as 00)|

|-------------------------------------------------------|

|       Variation 1,CV1, DRUMS Data Address             |

|-------------------------------------------------------|

|542|   LSB                     |       |               |

|543|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV1, PERC Data Address              |

|-------------------------------------------------------|

|544|   LSB                     |       |               |

|545|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV1, BASS Data Address              |

|-------------------------------------------------------|

|546|   LSB                     |       |               |

|547|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV1, ACC1 Data Address              |

|-------------------------------------------------------|

|548|   LSB                     |       |               |

|549|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV1, ACC2 Data Address              |

|-------------------------------------------------------|

|550|   LSB                     |       |               |

|551|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV1, ACC3 Data Address              |

|-------------------------------------------------------|

|552|   LSB                     |       |               |

|553|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, DRUMS Data Address             |

|-------------------------------------------------------|

|554|   LSB                     |       |               |

|555|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, PERC Data Address              |

|-------------------------------------------------------|

|556|   LSB                     |       |               |

|557|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, BASS Data Address              |

|-------------------------------------------------------|

|558|   LSB                     |       |               |

|559|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, ACC1 Data Address              |

|-------------------------------------------------------|

|560|   LSB                     |       |               |

|561|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, ACC2 Data Address              |

|-------------------------------------------------------|

|562|   LSB                     |       |               |

|563|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV2, ACC3 Data Address              |

|-------------------------------------------------------|

|564|   LSB                     |       |               |

|565|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, DRUMS Data Address             |

|-------------------------------------------------------|

|566|   LSB                     |       |               |

|567|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, PERC Data Address              |

|-------------------------------------------------------|

|568|   LSB                     |       |               |

|569|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, BASS Data Address              |

|-------------------------------------------------------|

|570|   LSB                     |       |               |

|571|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, ACC1 Data Address              |

|-------------------------------------------------------|

|572|   LSB                     |       |               |

|573|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, ACC2 Data Address              |

|-------------------------------------------------------|

|574|   LSB                     |       |               |

|575|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV3, ACC3 Data Address              |

|-------------------------------------------------------|

|576|   LSB                     |       |               |

|577|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, DRUMS Data Address             |

|-------------------------------------------------------|

|578|   LSB                     |       |               |

|579|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, PERC Data Address              |

|-------------------------------------------------------|

|580|   LSB                     |       |               |

|581|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, BASS Data Address              |

|-------------------------------------------------------|

|582|   LSB                     |       |               |

|583|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, ACC1 Data Address              |

|-------------------------------------------------------|

|584|   LSB                     |       |               |

|585|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, ACC2 Data Address              |

|-------------------------------------------------------|

|586|   LSB                     |       |               |

|587|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV4, ACC3 Data Address              |

|-------------------------------------------------------|

|588|   LSB                     |       |               |

|589|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, DRUMS Data Address             |

|-------------------------------------------------------|

|590|   LSB                     |       |               |

|591|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, PERC Data Address              |

|-------------------------------------------------------|

|592|   LSB                     |       |               |

|593|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, BASS Data Address              |

|-------------------------------------------------------|

|594|   LSB                     |       |               |

|595|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, ACC1 Data Address              |

|-------------------------------------------------------|

|596|   LSB                     |       |               |

|597|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, ACC2 Data Address              |

|-------------------------------------------------------|

|598|   LSB                     |       |               |

|599|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV5, ACC3 Data Address              |

|-------------------------------------------------------|

|600|   LSB                     |       |               |

|601|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, DRUMS Data Address             |

|-------------------------------------------------------|

|602|   LSB                     |       |               |

|603|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, PERC Data Address              |

|-------------------------------------------------------|

|604|   LSB                     |       |               |

|605|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, BASS Data Address              |

|-------------------------------------------------------|

|606|   LSB                     |       |               |

|607|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, ACC1 Data Address              |

|-------------------------------------------------------|

|608|   LSB                     |       |               |

|609|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, ACC2 Data Address              |

|-------------------------------------------------------|

|610|   LSB                     |       |               |

|611|   MSB                     |       |               |

|-------------------------------------------------------|

|       Variation 1,CV6, ACC3 Data Address              |

|-------------------------------------------------------|

|612|   LSB                     |       |               |

|613|   MSB                     |       |               |

|-------------------------------------------------------|

|614~829        Variation 2~4 Data Addresses            |

|                       (Same as Var.1)                 |

|-------------------------------------------------------|

|       Intro 1,CV1, DRUMS Data Address                 |

|-------------------------------------------------------|

|830|   LSB                     |       |               |

|831|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV1, PERC Data Address                  |

|-------------------------------------------------------|

|832|   LSB                     |       |               |

|833|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV1, BASS Data Address                  |

|-------------------------------------------------------|

|834|   LSB                     |       |               |

|835|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV1, ACC1 Data Address                  |

|-------------------------------------------------------|

|836|   LSB                     |       |               |

|837|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV1, ACC2 Data Address                  |

|-------------------------------------------------------|

|838|   LSB                     |       |               |

|839|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV1, ACC3 Data Address                  |

|-------------------------------------------------------|

|840|   LSB                     |       |               |

|841|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, DRUMS Data Address                 |

|-------------------------------------------------------|

|842|   LSB                     |       |               |

|843|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, PERC Data Address                  |

|-------------------------------------------------------|

|844|   LSB                     |       |               |

|845|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, BASS Data Address                  |

|-------------------------------------------------------|

|846|   LSB                     |       |               |

|847|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, ACC1 Data Address                  |

|-------------------------------------------------------|

|848|   LSB                     |       |               |

|849|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, ACC2 Data Address                  |

|-------------------------------------------------------|

|850|   LSB                     |       |               |

|851|   MSB                     |       |               |

|-------------------------------------------------------|

|       Intro 1,CV2, ACC3 Data Address                  |

|-------------------------------------------------------|

|852|   LSB                     |       |               |

|853|   MSB                     |       |               |

|-------------------------------------------------------|

|854~877        Intro 2,CV1~2, Data Addresses           |

|                       (Same as Intro 1)               |

|-------------------------------------------------------|

|878~901        Ending 1,CV1~2, Data Addresses          |

|                       (Same as Intro 1)               |

|-------------------------------------------------------|

|902~925        Ending 2,CV1~2, Data Addresses          |

|                       (Same as Intro 1)               |

|-------------------------------------------------------|

|926~949        Fill 1,CV1~2, Data Addresses            |

|                       (Same as Intro 1)               |

|-------------------------------------------------------|

|950~973        Fill 2,CV1~2, Data Addresses            |

|                       (Same as Intro 1)               |

|-------------------------------------------------------|

|       Pattern 00, Data Address                        |

|-------------------------------------------------------|

|974|   LSB                     |       |               |

|975|   MSB                     |       |               |

|-------------------------------------------------------|

|976~1173       Pattern 01~99, Data Addresseses         |

|                       (Same as 00)                    |

|-------------------------------------------------------|

|       End Pattern Address                             |

|-------------------------------------------------------|

|1174|  LSB                     |       |               |

|1175|  MSB                     |       |               |

+-------------------------------------------------------+

STYLE SEQUENCE DATA

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       Event 1                                         |

|-------------------------------------------------------|

|  0|   1st Data                |       |               |

|  1|   2nd Data                |       |               |

|  2|   3rd Data                |       |               |

|  3|   4th Data                |       |               |

|-------------------------------------------------------|

|4~             Event 2~ (same as Event 1)              |

+-------------------------------------------------------+

BACKING SEQ PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       BSEQ 0 Control Data                             |

|-------------------------------------------------------|

|000|   BSEQ Name(1/16)         | 20~7E |" " ~ "~"      |

|001|   BSEQ Name(2/16)         |       |(ASCII Code)   |

|002|   BSEQ Name(3/16)         |       |               |

|003|   BSEQ Name(4/16)         |       |               |

|004|   BSEQ Name(5/16)         |       |               |

|005|   BSEQ Name(6/16)         |       |               |

|006|   BSEQ Name(7/16)         |       |               |

|007|   BSEQ Name(8/16)         |       |               |

|008|   BSEQ Name(9/16)         |       |               |

|009|   BSEQ Name(10/16)        |       |               |

|010|   BSEQ Name(11/16)        |       |               |

|011|   BSEQ Name(12/16)        |       |               |

|012|   BSEQ Name(13/16)        |       |               |

|013|   BSEQ Name(14/16)        |       |               |

|014|   BSEQ Name(15/16)        |       |               |

|015|   BSEQ Name(16/16)        |       |               |

|016|   Arrangement No.         | 0~7F  |8 Beat~User 4  |

|017|   Arrangement Bank        | 0,1   |AB,CD          |

|018|   Style No.               | 00~7F |               |[Style No.][Style Bank]

|019|   Style Bank              | 00,01 |               | Bank=00,No.=00~7F : A11~A88~B88

|020|   Initial Variation(1/2)  | 00~03 |Var.1~4        | Bank=01,No.=00~2F : C11~C68

|021|   Initial Variation(2/2)  |       |00:->Var.1~4   |

|   |    (b3)Fill               | 0,1   |04:Intro 1     |

|   |    (b2)Intro              | 0,1   |05:Intro 2     |

|   |    (b1)...Reserve         | ----- |08:Fill 1      |

|   |    (b0)1 or 2             | 0,1   |09:Fill 2      |

|022|   Tempo                   | 0A~D2 |40~240         |

|023|   FULL KBD PLAY           | 00,01 |Off,On         |

|024|   Chord Scan Parameters   |       |               |

|   |    (b7)...Reserve         | ----- | -----         |

|   |    (b6)...Reserve         | ----- | -----         |"Lower type" is effective

|   |    (b5)...(Lower Type)    | (0,1) |(Normal,Easy)  | at first released system.

|   |    (b4)...Reserve         | ----- | -----         |

|   |    (b3)...Reserve         | ----- | -----         |

|   |    (b2)...Reserve         | ----- | -----         |[Chord Scanning]

|   |    (b1)Chord Scan Upper   | 0,1   |Off,On         |00:Off    02:Upper

|   |    (b0)Chord Scan Lower   | 0,1   |Off,On         |01:Lower  03:FULL

|025|   ...Reserve              | ----- |-----          |

|026|   ...Reserve              | ----- |-----          |

|027|   ...Reserve              | ----- |-----          |[ACC FX]

|028|   ...Reserve              | ----- |-----          |0:Arrangement

|029|   ACC FX                  | 0,1   |               |1:Backing SEQ

|-------------------------------------------------------|

|       MainKBD PARAMETERS                              |

|-------------------------------------------------------|

|030|   Program No.             | 00~7F |               |[Program No.][Program Bank]

|031|   Program Bank            | 00~04 |               | Bank=00,No.=00~7F : A11~A88~B88

|032|   ...Reserve              | ----- |-----          | Bank=01,No.=00~7F : C11~C88~D88

|033|   ...Reserve              | ----- |-----          | Bank=02,No.=00~3F : E11~E88

|034|   ...Reserve              | ----- |-----          | Bank=03,No.=00~7F : F11~F88~G88

|035|   Octave                  | FE~02 |-2~+2          | Bank=04,No.=00~27 : R11~R58

|036|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)...Reserve         | ----- |-----          |

|   |    (b0)...Reserve         | ----- |-----          |

|037|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       SubKBD PARAMETERS                               |

|-------------------------------------------------------|

|038|   Program No.             | 00~7F |               |

|039|   Program Bank            | 00~04 |               |

|040|   ...Reserve              | ----- |-----          |

|041|   ...Reserve              | ----- |-----          |

|042|   ...Reserve              | ----- |-----          |

|043|   Octave                  | FE~02 |-2~+2          |

|044|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)...Reserve         | ----- |-----          |

|   |    (b0)...Reserve         | ----- |-----          |

|045|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       LowerKBD PARAMETERS                             |

|-------------------------------------------------------|

|046|   Program No.             | 00~7F |               |

|047|   Program Bank            | 00~04 |               |

|048|   ...Reserve              | ----- |-----          |

|049|   ...Reserve              | ----- |-----          |

|050|   ...Reserve              | ----- |-----          |

|051|   Octave                  | FE~02 |-2~+2          |

|052|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)...Reserve         | ----- |-----          |

|   |    (b0)...Reserve         | ----- |-----          |

|053|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       Arrangement Track Activity                      |

|-------------------------------------------------------|

|054|   KBD Track Status        | 0,1   |Mute,Play      |

|055|   Control Track Status    | 0,1   |Mute,Play      |

|056|   Chord Track Status      | 0,1   |Mute,Play      |

|-------------------------------------------------------|

|057|   Tempo Track             | 0,1   |Manual,Auto    |[Meter]

|058|   Meter                   | 10~3F |               |10~14:1/4~5/4

|059|   Split Point             | 24~60 |C2~C7          |20~29:1/8~10/8

|060|   Transpose               | F5~0B |-11~+11        |30~3F:1/16~16/16

|-------------------------------------------------------|

|       Switches                                        |

|-------------------------------------------------------|

|061|   ...Reserve              | ----- |-----          |

|062|   Chord Memory On/Off     | 00,01 |Off,On         |

|063|   Chord Sound On/Off      | 00,01 |Off,On         |

|064|   Bass Inversion On/Off   | 00,01 |Off,On         |

|065|   ...Reserve              | ----- |-----          |

|066|   KBD Lock On/Off         | 00,01 |Off,On         |

|067|   Ensemble SW On/Off      | 00,01 |Off,On         |

|068|   ...Reserve              | ----- |-----          |

|069|   Drums Play/Mute         | 0,1   |Play,Mute      |

|070|   Perc Play/Mute          | 0,1   |Play,Mute      |

|071|   Bass Play/Mute          | 0,1   |Play,Mute      |

|072|   ACC1 Play/Mute          | 0,1   |Play,Mute      |

|073|   ACC2 Play/Mute          | 0,1   |Play,Mute      |

|074|   ACC3 Play/Mute          | 0,1   |Play,Mute      |

|075|   ...Reserve              | ----- |-----          |

|076|   ...Reserve              | ----- |-----          |

|077|   ...Reserve              | ----- |-----          |

|078|   ...Reserve              | ----- |-----          |

|079|   Next BSEQ Number        | 0~9   |BSEQ 0~BSEQ 9  |

|080|   Next BSEQ Chain/A-Start |       |               |

|   |    (b1)Chain              |0,1    |Off,On         |

|   |    (b0)Auto Start         |0,1    |Off,On         |

|-------------------------------------------------------|

|       ExtraKBD Track 4 PARAMETERS                     |

|-------------------------------------------------------|

|081|   Program No.             | 00~7F |               |

|082|   Program Bank            | 00~04 |               |[Program No.][Program Bank]

|083|   Volume                  | 00~7F |000~127        | Bank=00,No.=00~7F : A11~A88~B88

|084|   Pan                     | FF~1F |Off~Prog       | Bank=01,No.=00~7F : C11~C88~D88

|085|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       | Bank=02,No.=00~3F : E11~E88

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       | Bank=03,No.=00~7F : F11~F88~G88

|086|   Out Status              |       |               | Bank=04,No.=00~27 : R11~R58

|   |    (b7)...Reserve         | ----- |-----          |[Pan]

|   |    (b6)...Reserve         | ----- |-----          |FF : OFF

|   |    (b5)...Reserve         | ----- |-----          |00 : L15

|   |    (b4)...Reserve         | ----- |-----          |    |

|   |    (b3)...Reserve         | ----- |-----          |0F : CNT

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |    |

|   |    (b1)External Out       | 0,1   |Off,On         |1E : R15

|   |    (b0)Internal Out       | 0,1   |Off,On         |1F : PROG

|087|   PitchBend Range         | F4~0D |-12~+12,Prog   |

|088|   Transpose               | E8~18 |-24~+24        |

|089|   Detune                  | CE~32 |-50~+50        |

|090|   ...Reserve(Protect)     | 0     |-----          |

|091|   MIDI Channel            | 00~0F |1~16           |

|092|   Velocity Window Top     | 01~7F |001~127        |

|093|   Vel Window Bottom       | 01~7F |001~127        |

|094|   Key Window Top          | 00~7F |C-1~G9         |

|095|   Key Window Bottom       | 00~7F |C-1~G9         |

|-------------------------------------------------------|

|       ExtraKBD Track 5 PARAMETERS                     |

|-------------------------------------------------------|

|096|   Program No.             | 00~7F |               |

|097|   Program Bank            | 00~04 |               |

|098|   Volume                  | 00~7F |000~127        |

|099|   Pan                     | FF~1F |Off~Prog       |

|100|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|101|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|102|   PitchBend Range         | F4~0D |-12~+12,Prog   |

|103|   Transpose               | E8~18 |-24~+24        |

|104|   Detune                  | CE~32 |-50~+50        |

|105|   ...Reserve(Protect)     | 0     |-----          |

|106|   MIDI Channel            | 00~0F |1~16           |

|107|   Velocity Window Top     | 01~7F |001~127        |

|108|   Vel Window Bottom       | 01~7F |001~127        |

|109|   Key Window Top          | 00~7F |C-1~G9         |

|110|   Key Window Bottom       | 00~7F |C-1~G9         |

|-------------------------------------------------------|

|       ExtraKBD Track 6 PARAMETERS                     |

|-------------------------------------------------------|

|111|   Program No.             | 00~7F |               |

|112|   Program Bank            | 00~04 |               |

|113|   Volume                  | 00~7F |000~127        |

|114|   Pan                     | FF~1F |Off~Prog       |

|115|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|116|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|117|   PitchBend Range         | F4~0D |-12~+12,Prog   |

|118|   Transpose               | E8~18 |-24~+24        |

|119|   Detune                  | CE~32 |-50~+50        |

|120|   ...Reserve(Protect)     | 0     |-----          |

|121|   MIDI Channel            | 00~0F |1~16           |

|122|   Velocity Window Top     | 01~7F |001~127        |

|123|   Vel Window Bottom       | 01~7F |001~127        |

|124|   Key Window Top          | 00~7F |C-1~G9         |

|125|   Key Window Bottom       | 00~7F |C-1~G9         |

|-------------------------------------------------------|

|       ExtraKBD Track 7 PARAMETERS                     |

|-------------------------------------------------------|

|126|   Program No.             | 00~7F |               |

|127|   Program Bank            | 00~04 |               |

|128|   Volume                  | 00~7F |000~127        |

|129|   Pan                     | FF~1F |Off~Prog       |

|130|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|131|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|132|   PitchBend Range         | F4~0D |-12~+12,Prog   |

|133|   Transpose               | E8~18 |-24~+24        |

|134|   Detune                  | CE~32 |-50~+50        |

|135|   ...Reserve(Protect)     | 0     |-----          |

|136|   MIDI Channel            | 00~0F |1~16           |

|137|   Velocity Window Top     | 01~7F |001~127        |

|138|   Vel Window Bottom       | 01~7F |001~127        |

|139|   Key Window Top          | 00~7F |C-1~G9         |

|140|   Key Window Bottom       | 00~7F |C-1~G9         |

|-------------------------------------------------------|

|       ExtraKBD Track 8 PARAMETERS                     |

|-------------------------------------------------------|

|141|   Program No.             | 00~7F |               |

|142|   Program Bank            | 00~04 |               |

|143|   Volume                  | 00~7F |000~127        |

|144|   Pan                     | FF~1F |Off~Prog       |

|145|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       |

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |

|146|   Out Status              |       |               |

|   |    (b7)...Reserve         | ----- |-----          |

|   |    (b6)...Reserve         | ----- |-----          |

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)...Reserve         | ----- |-----          |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|147|   PitchBend Range         | F4~0D |-12~+12,Prog   |

|148|   Transpose               | E8~18 |-24~+24        |

|149|   Detune                  | CE~32 |-50~+50        |

|150|   ...Reserve(Protect)     | 0     |-----          |

|151|   MIDI Channel            | 00~0F |1~16           |

|152|   Velocity Window Top     | 01~7F |001~127        |

|153|   Vel Window Bottom       | 01~7F |001~127        |

|154|   Key Window Top          | 00~7F |C-1~G9         |

|155|   Key Window Bottom       | 00~7F |C-1~G9         |

|-------------------------------------------------------|

|156~200         Reserve                                |

|-------------------------------------------------------|

|       ACC FX PARAMETERS when No.29 ACC FX=Backing SEQ |

|-------------------------------------------------------|

|201|   FX 1 Type               | 00~2F |               |[FX 1/2 Type]

|202|   FX 2 Type               | 00~2F |               |0:NoEffect

|203|   FX 1 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |~2F:Delay/RotarySpeaker

|204|   FX 1 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |

|205|   FX 2 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |

|206|   FX 2 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |[Effect Output C/D Pan]

|207|   Effect Output C Pan     | 00~65 |Off,R,01:99~L  |When Placement is Parallel3

|208|   Effect Output D Pan     | 00~65 |Off,R,01:99~L  |No.141(bit7~4) FX 1 Return L : 0~9

|209|   Effect I/O              |       |               |No.141(bit3~0) FX 1 Return R : 0~9

|   |    (b7)...Reserve         | ----- |-----          |No.142(bit7~4) FX 2 Return L : 0~9

|   |    (b6)Placement(1/3)     | 0~5   |               |No.142(bit3~0) FX 2 Return R : 0~9

|   |    (b5)Placement(2/3)     |       |               |

|   |    (b4)Placement(3/3)     |       |               |[Placement]

|   |    (b3)Effect 2(R) On/Off | 0,1   |Off,On         |0:Serial

|   |    (b2)Effect 2(L) On/Off | 0,1   |Off,On         |1:Parallel 1

|   |    (b1)Effect 1(R) On/Off | 0,1   |Off,On         |2:Parallel 2

|   |    (b0)Effect 1(L) On/Off | 0,1   |Off,On         |3:Parallel 3

|210|   FX 1 Parameter(1/8)     |       |               |4:Serial Sub

|211|   FX 1 Parameter(2/8)     |       |               |5:Parallel Sub

|212|   FX 1 Parameter(3/8)     |       |               |

|213|   FX 1 Parameter(4/8)     |       |               |

|214|   FX 1 Parameter(5/8)     |       |               |

|215|   FX 1 Parameter(6/8)     |       |               |

|216|   FX 1 Parameter(7/8)     |       |               |

|217|   FX 1 Parameter(8/8)     |       |               |

|218|   FX 1 D-Mod.Source       | 00~06 |None~VDA EG    |

|219|   FX 1 D-Mod.Amount       | F1~OF |-15~+15        |

|220|   FX 2 Parameter(1/8)     |       |               |

|221|   FX 2 Parameter(2/8)     |       |               |

|222|   FX 2 Parameter(3/8)     |       |               |

|223|   FX 2 Parameter(4/8)     |       |               |

|224|   FX 2 Parameter(5/8)     |       |               |

|225|   FX 2 Parameter(6/8)     |       |               |

|226|   FX 2 Parameter(7/8)     |       |               |

|227|   FX 2 Parameter(8/8)     |       |               |

|228|   FX 2 D-Mod.Source       | 00~06 |None~VDA EG    |

|229|   FX 2 D-Mod.Amount       | F1~OF |-15~+15        |

|-------------------------------------------------------|

|230~459        BSEQ 1 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|460~689        BSEQ 2 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|690~919        BSEQ 3 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|920~1149       BSEQ 4 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|1150~1379      BSEQ 5 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|1380~1609      BSEQ 6 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|1610~1839      BSEQ 7 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|1840~2069      BSEQ 8 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|2070~2299      BSEQ 9 Control Data (Same as BSEQ 0)    |

|-------------------------------------------------------|

|       BSEQ 0 Track 1 (KBD Tr) Data Address            |

|-------------------------------------------------------|

|2300|  LSB                     |       |               |

|2301|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 2 (Control Tr) Data Address        |

|-------------------------------------------------------|

|2302|  LSB                     |       |               |

|2303|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 3 (Chord Tr) Data Address          |

|-------------------------------------------------------|

|2304|  LSB                     |       |               |

|2305|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 4 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2306|  LSB                     |       |               |

|2307|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 5 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2308|  LSB                     |       |               |

|2309|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 6 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2310|  LSB                     |       |               |

|2311|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 7 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2312|  LSB                     |       |               |

|2313|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 8 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2314|  LSB                     |       |               |

|2315|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 9 (Dummy Tr) Data Address          |

|-------------------------------------------------------|

|2316|  LSB                     |       |               |

|2317|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 10 (Dummy Tr) Data Address         |

|-------------------------------------------------------|

|2318|  LSB                     |       |               |

|2319|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 11 (Dummy Tr) Data Address         |

|-------------------------------------------------------|

|2320|  LSB                     |       |               |

|2321|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 12 (E.KBD4 Tr) Data Address        |

|-------------------------------------------------------|

|2322|  LSB                     |       |               |

|2323|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 13 (E.KBD5 Tr) Data Address        |

|-------------------------------------------------------|

|2324|  LSB                     |       |               |

|2325|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 14 (E.KBD6 Tr) Data Address        |

|-------------------------------------------------------|

|2326|  LSB                     |       |               |

|2327|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 15 (E.KBD7 Tr) Data Address        |

|-------------------------------------------------------|

|2328|  LSB                     |       |               |

|2329|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Track 16 (E.KBD8 Tr) Data Address        |

|-------------------------------------------------------|

|2330|  LSB                     |       |               |

|2331|  MSB                     |       |               |

|-------------------------------------------------------|

|       BSEQ 0 Tempo Track Data Address                 |

|-------------------------------------------------------|

|2332|  LSB                     |       |               |

|2333|  MSB                     |       |               |

|-------------------------------------------------------|

|2334~2367      BSEQ 1 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2368~2401      BSEQ 2 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2402~2435      BSEQ 3 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2436~2469      BSEQ 4 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2470~2503      BSEQ 5 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2504~2537      BSEQ 6 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2538~2571      BSEQ 7 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2572~2605      BSEQ 8 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|2606~2639      BSEQ 9 Data Addresses (same as  BSEQ 0) |

|-------------------------------------------------------|

|       End Address                                     |

|-------------------------------------------------------|

|2640|  LSB                     |       |               |

|2641|  MSB                     |       |               |

+-------------------------------------------------------+

BACKING SEQ SEQUENCE DATA

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       Event 1                                         |

|-------------------------------------------------------|

|  0|   1st Data                |       |               |

|  1|   2nd Data                |       |               |

|  2|   3rd Data                |       |               |

|  3|   4th Data                |       |               |

|-------------------------------------------------------|

|4~             Event 2~ (same as Event 1)              |

+-------------------------------------------------------+

SONG PARAMETERS

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       Song 0 Control Data

|-------------------------------------------------------|

|       MIDI Chnannel

|-------------------------------------------------------|

|000|   Track 1                 |0~0F   |1~16           |

|001|   Track 2                 |0~0F   |1~16           |

|002|   Track 3                 |0~0F   |1~16           |

|003|   Track 4                 |0~0F   |1~16           |

|004|   Track 5                 |0~0F   |1~16           |

|005|   Track 6                 |0~0F   |1~16           |

|006|   Track 7                 |0~0F   |1~16           |

|007|   Track 8                 |0~0F   |1~16           |

|008|   Track 9                 |0~0F   |1~16           |

|009|   Track 10                |0~0F   |1~16           |

|010|   Track 11                |0~0F   |1~16           |

|011|   Track 12                |0~0F   |1~16           |

|012|   Track 13                |0~0F   |1~16           |

|013|   Track 14                |0~0F   |1~16           |

|014|   Track 15                |0~0F   |1~16           |

|015|   Track 16                |0~0F   |1~16           |

|-------------------------------------------------------|

|       Out Status                                      |

|-------------------------------------------------------|

|016|   Track 1                 |       |               |

|   |    (b7)...Reserve         | ----- | -----         |

|   |    (b6)...Reserve         | ----- | -----         |

|   |    (b5)...Reserve         | ----- | -----         |

|   |    (b4)...Reserve         | ----- | -----         |

|   |    (b3)...Reserve         | ----- | -----         |

|   |    (b2)Mute/Play          | 0,1   |Play,Mute      |

|   |    (b1)External Out       | 0,1   |Off,On         |

|   |    (b0)Internal Out       | 0,1   |Off,On         |

|017|   Track 2 (same as Track 1)                       |

|018|   Track 3 (same as Track 1)                       |

|019|   Track 4 (same as Track 1)                       |

|020|   Track 5 (same as Track 1)                       |

|021|   Track 6 (same as Track 1)                       |

|022|   Track 7 (same as Track 1)                       |

|023|   Track 8 (same as Track 1)                       |

|024|   Track 9 (same as Track 1)                       |

|025|   Track 10 (same as Track 1)                      |

|026|   Track 11 (same as Track 1)                      |

|027|   Track 12 (same as Track 1)                      |

|028|   Track 13 (same as Track 1)                      |

|029|   Track 14 (same as Track 1)                      |

|030|   Track 15 (same as Track 1)                      |

|031|   Track 16 (same as Track 1)                      |

|-------------------------------------------------------|

|       Pitch Bend Range                                |

|-------------------------------------------------------|

|032|   Track 1                 | F4~0D |-12~+12,Prog   |

|033|   Track 2                 | F4~0D |-12~+12,Prog   |

|034|   Track 3                 | F4~0D |-12~+12,Prog   |

|035|   Track 4                 | F4~0D |-12~+12,Prog   |

|036|   Track 5                 | F4~0D |-12~+12,Prog   |

|037|   Track 6                 | F4~0D |-12~+12,Prog   |

|038|   Track 7                 | F4~0D |-12~+12,Prog   |

|039|   Track 8                 | F4~0D |-12~+12,Prog   |

|040|   Track 9                 | F4~0D |-12~+12,Prog   |

|041|   Track 10                | F4~0D |-12~+12,Prog   |

|042|   Track 11                | F4~0D |-12~+12,Prog   |

|043|   Track 12                | F4~0D |-12~+12,Prog   |

|044|   Track 13                | F4~0D |-12~+12,Prog   |

|045|   Track 14                | F4~0D |-12~+12,Prog   |

|046|   Track 15                | F4~0D |-12~+12,Prog   |

|047|   Track 16                | F4~0D |-12~+12,Prog   |

|-------------------------------------------------------|[Meter]

|048|   Meter                   | 10~3F |               |10~14:1/4~5/4

|049|   Tempo                   | 0A~D2 |40~240         |20~29:1/8~10/8

|050|   ...Reserve(Protect 1~8) | 0     |-----          |30~3F:1/16~16/16

|051|   ...Reserve(Protect 9~16)| 0     |-----          |

|052|   Next Song Number        | 00~09 |Song 0~Song 9  |

|053|   Next Song Chain/A-Start |       |               |

|   |    (b1)Chain              | 0,1   |Off,On         |

|   |    (b0)Auto Start         | 0,1   |Off,On         |

|054|   Song Name(1/16)         | 20~7E |" " ~ "~"      |

|055|   Song Name(2/16)         |       |(ASCII Code)   |

|056|   Song Name(3/16)         |       |               |

|057|   Song Name(4/16)         |       |               |

|058|   Song Name(5/16)         |       |               |

|059|   Song Name(6/16)         |       |               |

|060|   Song Name(7/16)         |       |               |

|061|   Song Name(8/16)         |       |               |

|062|   Song Name(9/16)         |       |               |

|063|   Song Name(10/16)        |       |               |

|064|   Song Name(11/16)        |       |               |

|065|   Song Name(12/16)        |       |               |

|066|   Song Name(13/16)        |       |               |

|067|   Song Name(14/16)        |       |               |

|068|   Song Name(15/16)        |       |               |

|069|   Song Name(16/16)        |       |               |

|070|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|       FX PARAMETERS                                   |

|-------------------------------------------------------|

|071|   FX 1 Type               | 00~2F |               |[FX 1/2 Type]

|072|   FX 2 Type               | 00~2F |               |0:NoEffect

|073|   FX 1 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |~2F:Delay/RotarySpeaker

|074|   FX 1 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |

|075|   FX 2 Dry/Wet Balance(L) | 00~64 |Dry~Wet        |

|076|   FX 2 Dry/Wet Balance(R) | 00~64 |Dry~Wet        |[Effect Output C/D Pan]

|077|   Effect Output C Pan     | 00~65 |Off,R,01:99~L  |When Placement is Parallel3

|078|   Effect Output D Pan     | 00~65 |Off,R,01:99~L  |No.141(bit7~4) FX 1 Return L : 0~9

|079|   Effect I/O              |       |               |No.141(bit3~0) FX 1 Return R : 0~9

|   |    (b7)...Reserve         | ----- |-----          |No.142(bit7~4) FX 2 Return L : 0~9

|   |    (b6)Placement(1/3)     | 0~5   |               |No.142(bit3~0) FX 2 Return R : 0~9

|   |    (b5)Placement(2/3)     |       |               |

|   |    (b4)Placement(3/3)     |       |               |[Placement]

|   |    (b3)Effect 2(R) On/Off | 0,1   |Off,On         |0:Serial

|   |    (b2)Effect 2(L) On/Off | 0,1   |Off,On         |1:Parallel 1

|   |    (b1)Effect 1(R) On/Off | 0,1   |Off,On         |2:Parallel 2

|   |    (b0)Effect 1(L) On/Off | 0,1   |Off,On         |3:Parallel 3

|080|   FX 1 Parameter(1/8)     |       |               |4:Serial Sub

|081|   FX 1 Parameter(2/8)     |       |               |5:Parallel Sub

|082|   FX 1 Parameter(3/8)     |       |               |

|083|   FX 1 Parameter(4/8)     |       |               |

|084|   FX 1 Parameter(5/8)     |       |               |

|085|   FX 1 Parameter(6/8)     |       |               |

|086|   FX 1 Parameter(7/8)     |       |               |

|087|   FX 1 Parameter(8/8)     |       |               |

|088|   FX 1 D-Mod.Source       | 00~06 |None~VDA EG    |

|089|   FX 1 D-Mod.Amount       | F1~OF |-15~+15        |

|090|   FX 2 Parameter(1/8)     |       |               |

|091|   FX 2 Parameter(2/8)     |       |               |

|092|   FX 2 Parameter(3/8)     |       |               |

|093|   FX 2 Parameter(4/8)     |       |               |

|094|   FX 2 Parameter(5/8)     |       |               |

|095|   FX 2 Parameter(6/8)     |       |               |

|096|   FX 2 Parameter(7/8)     |       |               |

|097|   FX 2 Parameter(8/8)     |       |               |

|098|   FX 2 D-Mod.Source       | 00~06 |None~VDA EG    |

|099|   FX 2 D-Mod.Amount       | F1~OF |-15~+15        |

|-------------------------------------------------------|

|       Track 1 PARAMETERS                              |

|-------------------------------------------------------|

|100|   Program No.             | 00~7F |               |

|101|   Program Bank            | 00~04 |               |[Program No.][Program Bank]

|102|   Volume                  | 00~7F |000~127        | Bank=00,No.=00~7F : A11~A88~B88

|103|   Transpose               | E8~18 |-24~+24        | Bank=01,No.=00~7F : C11~C88~D88

|104|   Detune                  | CE~32 |-50~+50        | Bank=02,No.=00~3F : E11~E88

|105|   Pan                     | FF~1F |Off~Prog       | Bank=03,No.=00~7F : F11~F88~G88

|106|   (b7~b4)C Send Level     | 0~A   |0~9,Prog       | Bank=04,No.=00~27 : R11~R58

|   |   (b3~b0)D Send Level     | 0~A   |0~9,Prog       |[Pan]

|107|   Key Window Top          | 00~7F |C-1~G9         |FF : OFF

|108|   Key Window Bottom       | 00~7F |C-1~G9         |00 : L15

|109|   Velocity Window Top     | 01~7F |001~127        |    |

|110|   Vel Window Bottom       | 01~7F |001~127        |0F : CNT

|111|   ...Reserve(Ctrl Filter) |       |               |    |

|   |    (b7)...Reserve         | ----- |-----          |1E : R15

|   |    (b6)...Reserve         | ----- |-----          |1F : PROG

|   |    (b5)...Reserve         | ----- |-----          |

|   |    (b4)...Reserve         | ----- |-----          |

|   |    (b3)ControlChange      | 1     |Enable         |

|   |    (b2)Aftertouch 0,1     | 1     |Enable         |

|   |    (b1)Damper     0,1     | 1     |Enable         |

|   |    (b0)Prog.Change        | 1     |Enable         |

|112|   MIDI Channel            | 00~0F |1~16           |

|-------------------------------------------------------|

|113~125        Track 2 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|126~138        Track 3 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|139~151        Track 4 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|152~164        Track 5 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|165~177        Track 6 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|178~190        Track 7 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|191~203        Track 8 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|204~216        Track 9 PARAMETERS (same as Track 1)    |

|-------------------------------------------------------|

|217~229        Track 10 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|230~242        Track 11 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|243~255        Track 12 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|256~268        Track 13 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|269~281        Track 14 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|282~294        Track 15 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|295~307        Track 16 PARAMETERS (same as Track 1)   |

|-------------------------------------------------------|

|308|   ...Reserve              | ----- |-----          |

|309|   ...Reserve              | ----- |-----          |

|310|   ...Reserve              | ----- |-----          |

|311|   ...Reserve              | ----- |-----          |

|312|   ...Reserve              | ----- |-----          |

|313|   ...Reserve              | ----- |-----          |

|314|   ...Reserve              | ----- |-----          |

|315|   ...Reserve              | ----- |-----          |

|316|   ...Reserve              | ----- |-----          |

|317|   ...Reserve              | ----- |-----          |

|-------------------------------------------------------|

|318~635        Song 1 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|636~953        Song 2 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|954~1271       Song 3 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|1272~1589      Song 4 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|1590~1907      Song 5 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|1908~2225      Song 6 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|2226~2543      Song 7 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|2544~2861      Song 8 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|2862~3179      Song 9 Control Data (same as Song 0)    |

|-------------------------------------------------------|

|       Pattern 00, Control Data                        |

|-------------------------------------------------------|[Meter]

|3180|  Meter                   | 10~3F |               |10~14:1/4~5/4

|3181|  Length                  | 01~63 |1~99           |20~29:1/8~10/8

|-------------------------------------------------------|30~3F:1/16~16/16

|3182~3379      Pattern 01~99, Control Data(same as 00) |

|-------------------------------------------------------|

|       Song 0 Track 1 Data Address                     |

|-------------------------------------------------------|

|3380|  LSB                     |       |               |

|3381|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 2 Data Address (same as Track 1)   |

|-------------------------------------------------------|

|3382|  LSB                     |       |               |

|3383|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 3 Data Address                     |

|-------------------------------------------------------|

|3384|  LSB                     |       |               |

|3385|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 4 Data Address                     |

|-------------------------------------------------------|

|3386|  LSB                     |       |               |

|3387|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 5 Data Address                     |

|-------------------------------------------------------|

|3388|  LSB                     |       |               |

|3389|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 6 Data Address                     |

|-------------------------------------------------------|

|3390|  LSB                     |       |               |

|3391|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 7 Data Address                     |

|-------------------------------------------------------|

|3392|  LSB                     |       |               |

|3393|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 8 Data Address                     |

|-------------------------------------------------------|

|3394|  LSB                     |       |               |

|3395|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 9 Data Address                     |

|-------------------------------------------------------|

|3396|  LSB                     |       |               |

|3397|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 10 Data Address                    |

|-------------------------------------------------------|

|3398|  LSB                     |       |               |

|3399|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 11 Data Address                    |

|-------------------------------------------------------|

|3400|  LSB                     |       |               |

|3401|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 12 Data Address                    |

|-------------------------------------------------------|

|3402|  LSB                     |       |               |

|3403|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 13 Data Address                    |

|-------------------------------------------------------|

|3404|  LSB                     |       |               |

|3405|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 14 Data Address                    |

|-------------------------------------------------------|

|3406|  LSB                     |       |               |

|3407|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 15 Data Address                    |

|-------------------------------------------------------|

|3408|  LSB                     |       |               |

|3409|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Track 16 Data Address                    |

|-------------------------------------------------------|

|3410|  LSB                     |       |               |

|3411|  MSB                     |       |               |

|-------------------------------------------------------|

|       Song 0 Tempo Track Data Address                 |

|-------------------------------------------------------|

|3412|  LSB                     |       |               |

|3413|  MSB                     |       |               |

|-------------------------------------------------------|

|3414~3447              Song 1 Data Addresses           |

|-------------------------------------------------------|

|3448~3481              Song 2 Data Addresses           |

|-------------------------------------------------------|

|3482~3515              Song 3 Data Addresses           |

|-------------------------------------------------------|

|3516~3549              Song 4 Data Addresses           |

|-------------------------------------------------------|

|3550~3583              Song 5 Data Addresses           |

|-------------------------------------------------------|

|3584~3617              Song 6 Data Addresses           |

|-------------------------------------------------------|

|3618~3651              Song 7 Data Addresses           |

|-------------------------------------------------------|

|3652~3685              Song 8 Data Addresses           |

|-------------------------------------------------------|

|3686~3719              Song 9 Data Addresses           |

|-------------------------------------------------------|

|       Pattern 00, Data Address                        |

|-------------------------------------------------------|

|3720|  LSB                     |       |               |

|3721|  MSB                     |       |               |

|-------------------------------------------------------|

|3722~3919      Pattern 01~99, Data Addresses           |

|                       (same as 00)                    |

|-------------------------------------------------------|

|       End Pattern Address                             |

|-------------------------------------------------------|

|3920|  LSB                     |       |               |

|3921|  MSB                     |       |               |

+-------------------------------------------------------+

SONG SEQUENCE DATA

+-------------------------------------------------------+

|No.|   Parameter               |Data(h)|Value          |

|-------------------------------------------------------|

|       Event 1                                         |

|-------------------------------------------------------|

|  0|   1st Data                |       |               |

|  1|   2nd Data                |       |               |

|  2|   3rd Data                |       |               |

|  3|   4th Data                |       |               |

|-------------------------------------------------------|

|4~             Event 2~ (same as Event 1)              |

+-------------------------------------------------------+

EFFECT PARAMETERS        (8Byte) 47 Types

+-------------------------------------------------------+

|Offset|Parameter               |Data(h)|Value          |

+-------------------------------------------------------+

1~6:Reverb (Hall,Room)

+-------------------------------------------------------+

|(00)|Reverb Time               | 00~61 |0.2~9.9(~4.9)s |

|(01)|( NUL )                   | 00    |               |

|(02)|High Damp                 | 00~63 |0~99%          |

|(03)|Pre Delay                 | 00~C8 |00~200ms       |

|(04)|E.R. Level                | 00~63 |0~99           |

|(05)|( NUL )                   | 00    |               |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

NUL not listed from here on, Value must be 00

7~9:Reverb (Plate,Spring)

+-------------------------------------------------------+

|(00)|Pre Delay                 | 00~C8 |0~200ms        |

|(02)|E.R. Level                | 01~0A |1~10           |

|(03)|Reverb Time               | 00~63 |0~99           |

|(04)|High Damp                 | 00~63 |0~99%          |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

10~12:Early Reflections 1,2,3

+-------------------------------------------------------+

|(00)|E.R. Time                 | 00~46 |100~800ms      |

|(01)|Pre Delay                 | 00~C8 |0~200ms        |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

13:Stereo Delay, 14:Cross Delay

+-------------------------------------------------------+

|(00)|Delay Time L (LSB)        | 0000  |0~500ms        |

|(01)|Deray Time L (MSB)        | ~01F4 |               |

|(02)|Feedback                  | 9D~63 |-99~+99%       |

|(03)|High Damp                 | 00~63 |0~99%          |

|(04)|Delay Time R (LSB)        | 0000  |0~500ms        |

|(05)|Delay Time R (MSB)        | ~01F4 |               |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

15:Dual Mono Delay

+-------------------------------------------------------+

|(00)|Delay Time L (LSB)        | 0000  |0~500ms        |

|(01)|Deray Time L (MSB)        | ~01F4 |               |

|(02)|Feedback  L               | 9D~63 |-99~+99%       |

|(03)|High Damp  L              | 00~63 |0~99%          |

|(04)|Delay Time R (LSB)        | 0000  |0~500ms        |

|(05)|Delay Time R (MSB)        | ~01F4 |               |

|(06)|Feedback  R               | 9D~63 |-99~+99%       |

|(07)|High Damp  R              | 00~63 |0~99%          |

+-------------------------------------------------------+

16~18:St. Multitap Delay 1,2,3

+-------------------------------------------------------+

|(00)|Delay Time A (LSB)        | 0000  |0~500ms        |

|(01)|Delay Time A (MSB)        | ~01F4 |               |

|(02)|Delay Time B (LSB)        | 0000  |0~500ms        |

|(03)|Delay Time B (MSB)        | ~01F4 |               |

|(04)|Feedback                  | 9D~63 |-99~+99%       |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

19,20:Stereo Chorus 1,2

+-------------------------------------------------------+

|(00)|Mod Depth                 | 00~63 |0~99           |[Mod Speed]

|(01)|Mod Speed                 | 00~D8 |               |00~63: 0.0~3.00Hz (0.03step)

|(02)|MG Status                 |       |               |34~C7: 3.1~13.0Hz (0.1 step)

|    | (b2)Wave Shape           | 0     |Normal         |C8~D8: 14 ~30Hz   (1   step)

|    | (b1)Phase                | 1     |180deg         |

|    | (b0)LFO Waveform         | 0,1   |Sin,Triangle   |

|(04)|Delay Time                | 00~C8 |0~200ms        |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

21,22:St. Quadrature Chorus,St. Crossover Chorus

+-------------------------------------------------------+

|(00)|Delay Time L              | 00~FA |0~250ms        |

|(01)|Delay Time R              | 00~FA |0~250ms        |

|(02)|Mod Speed                 | 01~63 |1~99           |

|(03)|Mod Depth                 | 00~63 |0~99           |[Mod Waveform]

|(04)|Mod Waveform              | EB~14 |               |EB~FF: Tri+10~Tri-10

|(06)|EQ High                   | F4~0C |-12~+12        |00~14: Sin-10~Sin+10

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

23:Harmonic Chorus

+-------------------------------------------------------+

|(00)|Delay Time A (LSB)        | 0000  |0~500ms        |

|(01)|Delay Time A (MSB)        | ~01F4 |               |

|(02)|Delay Time B (LSB)        | 0000  |0~500ms        |

|(03)|Delay Time B (MSB)        | ~01F4 |               |

|(04)|Mod Speed                 | 01~63 |1~99           |

|(05)|Mod Depth                 | 00~63 |0~99           |

|(06)|Filter Split Point        | 00~12 |160Hz~10.0kHz  |

+-------------------------------------------------------+

24:Symphonic Ensemble

+-------------------------------------------------------+

|(00)|Mod Depth                 | 01~63 |1~99           |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

25~27:Stereo Flanger1,2, St. Crossover Flanger

+-------------------------------------------------------+

|(00)|Delay Time                | 00~C8 |0~200          |

|(01)|Mod Depth                 | 00~63 |0~99           |

|(02)|Mod Speed                 | 01~63 |1~99           |

|(03)|Feedback                  | 9D~63 |-99~+99%       |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

28:Stereo Exciter

+-------------------------------------------------------+

|(00)|Harmonic density          | 9D~63 |-99~+99        |

|(01)|Hot Spot                  | 00~09 |1~10           |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

29:Enhancer

+-------------------------------------------------------+

|(00)|Harmonic Density          | 01~63 |1~99           |

|(01)|Hot Spot                  | 01~14 |1~20           |

|(02)|Stereo Width              | 00~63 |0~99           |

|(03)|Delay Time                | 01~63 |1~99           |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

30:Distortion, 31:Over Drive

+-------------------------------------------------------+

|(00)|Drive                     | 01~6F |1~111          |

|(01)|Hot Spot                  | 00~63 |0~99           |

|(02)|Resonance                 | 00~63 |0~99           |

|(03)|Distortion Level          | 00~63 |0~99           |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

32,33:Stereo Phaser 1,(2)

+-------------------------------------------------------+

|(00)|Mod Depth                 | 00~63 |0~99           |[Mod Speed]

|(01)|Mod Speed                 | 00~D8 |               |00~63: 0.0~3.00Hz (0.03step)

|(02)|MG Status                 |       |               |34~C7: 3.1~13.0Hz (0.1 step)

|    | (b2)Wave Shape           | 0     |Normal         |C8~D8: 14 ~30Hz   (1   step)

|    | (b1)Phase                | 1,(0) |180deg,(0deg)  |

|    | (b0)LFO Waveform         | 0,1   |Sin,Triangle   |

|(03)|Feedback                  | 9D~63 |-99~+99%       |

|(04)|Hot Spot                  | 00~63 |0~99           |

+-------------------------------------------------------+

34:Rotary Speaker

+-------------------------------------------------------+

|(00)|Vibrato Depth             | 00~0F |0~15           |

|(01)|Acceleration              | 01~0F |1~15           |

|(02)|Slow Speed                | 01~63 |1~99           |

|(03)|Fast Speed                | 01~63 |1~99           |

+-------------------------------------------------------+

35~36:Auto Pan, (Tremolo)

+-------------------------------------------------------+

|(00)|Mod Depth                 | 00~63 |0~99           |[Mod Speed]

|(01)|Mod Speed                 | 00~D8 |               |00~63: 0.0~3.00Hz (0.03step)

|(02)|MG Status                 |       |               |34~C7: 3.1~13.0Hz (0.1 step)

|    | (b2)Wave Shape           | 0     |Normal         |C8~D8: 14 ~30Hz   (1   step)

|    | (b1)Phase                | 1,(0) |180deg,(0deg)  |

|    | (b0)LFO Waveform         | 0,1   |Sin,Triangle   |

|(03)|LFO Shape                 | 9D~63 |-99~+99        |

|(06)|EQ High                   | F4~0C |-12~+12        |

|(07)|EQ Low                    | F4~0C |-12~+12        |

+-------------------------------------------------------+

37:Parametric EQ

+-------------------------------------------------------+

|(00)|Low Frequency             | 00~1D |0~29           |

|(01)|Low Gain                  | F4~0C |-12~+12dB      |

|(02)|Mid Frequency             | 00~63 |0~99           |

|(03)|Mid Gain                  | F4~0C |-12~+12dB      |

|(04)|Mid Width                 | 00~63 |0~99           |

|(05)|High Frequency            | 00~1D |0~29           |

|(06)|High Gain                 | F4~0C |-12~+12dB      |

+-------------------------------------------------------+

38:Chorus-Delay

+-------------------------------------------------------+

|(00)|Chorus Delay Time         | 00~32 |0~50ms         |

|(01)|Mod Speed                 | 01~63 |1~99           |

|(02)|Mod Depth                 | 00~63 |0~99           |

|(03)|Chorus Feedback           | 9D~63 |-99~+99%       |

|(04)|Delay Time                | 00~E1 |0~450ms        |

|(05)|Delay Feedback            | 9D~63 |-99~+99%       |

+-------------------------------------------------------+

39:Flanger-Delay

+-------------------------------------------------------+

|(00)|Flanger Delay Time        | 00~32 |0~50ms         |

|(01)|Mod Speed                 | 01~63 |1~99           |

|(02)|Mod Depth                 | 00~63 |0~99           |

|(03)|Flanger Feedback          | 9D~63 |-99~+99%       |

|(04)|Delay Time                | 00~E1 |0~450ms        |

|(05)|Delay Feedback            | 9D~63 |-99~+99%       |

+-------------------------------------------------------+

40:Delay/Hall Reverb

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Delay High Damp           | 00~63 |0~99           |

|(04)|Reverb Time               | 00~61 |0.2~9.9s       |

|(06)|Reverb High Damp          | 00~63 |0~99           |

|(07)|Reverb Pre Delay          | 00~96 |0~150ms        |

+-------------------------------------------------------+

41:Delay/Room Reverb

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Delay High Damp           | 00~63 |0~99           |

|(04)|Reverb Time               | 00~2F |0.2~4.9s       |

|(06)|Reverb High Damp          | 00~63 |0~99           |

|(07)|Reverb Pre Delay          | 00~96 |0~150ms        |

+-------------------------------------------------------+

42:Delay/Chorus

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Delay High Damp           | 00~63 |0~99           |

|(04)|Mod Depth                 | 00~63 |0~99           |[Mod Speed]

|(05)|Mod Speed                 | 00~D8 |               |00~63: 0.0~3.00Hz (0.03step)

|(06)|MG Status                 |       |               |34~C7: 3.1~13.0Hz (0.1 step)

|    | (b2)Wave Shape           | 0     |Normal         |C8~D8: 14 ~30Hz   (1   step)

|    | (b1)Phase                | 0     |0deg           |

|    | (b0)LFO Waveform         | 0,1   |Sin,Triangle   |

+-------------------------------------------------------+

43:Delay/Flanger

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Delay High Damp           | 00~63 |0~99           |

|(04)|Mod Depth                 | 00~63 |0~99           |[Mod Speed]

|(05)|Mod Speed                 | 00~D8 |               |00~63: 0.0~3.00Hz (0.03step)

|(06)|MG Status                 |       |               |34~C7: 3.1~13.0Hz (0.1 step)

|    | (b2)Wave Shape           | 1     |for Flanger    |C8~D8: 14 ~30Hz   (1   step)

|    | (b1)Phase                | 0     |0deg           |

|    | (b0)LFO Waveform         | 0     |Sin            |

|(07)|Flanger Feedback          | 9D~63 |-99~+99%       |

+-------------------------------------------------------+

44~45:Delay/Distortion, Delay/Over Drive

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Drive                     | 01~6F |1~111          |

|(04)|Hot Spot                  | 01~63 |1~99           |

|(05)|Resonance                 | 00~63 |0~99           |

|(06)|Distortion Level          | 01~63 |1~99           |

+-------------------------------------------------------+

46:Delay / Phaser

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Delay High Damp           | 00~63 |0~99           |[Mod Speed]

|(04)|Mod Depth                 | 00~63 |0~99           |00~63: 0.0~3.00Hz (0.03step)

|(05)|Mod Speed                 | 00~D8 |               |34~C7: 3.1~13.0Hz (0.1 step)

|(06)|Phaser Feedback           | 9D~63 |-99~+99        |C8~D8: 14 ~30Hz   (1   step)

+-------------------------------------------------------+

47:Delay / Rotary Speaker

+-------------------------------------------------------+

|(00)|Delay Time (LSB)          | 0000  |0~500ms        |

|(01)|Delay Time (MSB)          | ~01F4 |               |

|(02)|Delay Feedback            | 9D~63 |-99~+99%       |

|(03)|Acceleration              | 01~0F |1~15           |

|(04)|Slow Speed                | 01~63 |1~99           |

|(05)|Fast Speed                | 01~63 |1~99           |

+-------------------------------------------------------+

SEQUENCE DATA FORMAT

STYLE SEQUENCE DATA

BACKING SEQ SEQUENCE DATA (except "Control Track","Chord Track","Tempo Track")

SONG SEQUENCE DATA (except "Tempo Track")

*1 NOTE ON/OFF

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|1vvv vvv t|tttt tttt |kkkk kkk g|gggg gggg |

+----------+----------+----------+----------+

Velocity  Location   Key No.   Length

t :1/96th quater note unit,  t = 1FE(h) : Tie from previous bar

g :1/96th quater note unit,  g = 1FE(h) : Tie to next bar

*2 PITCH BEND

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0001 000 t|tttt tttt |0 vvv vvvv|0 vvv vvvv|

+----------+----------+----------+----------+

Location       Value(H)   Value(L)

*3 AFTER TOUCH

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0010 000 t|tttt tttt | 0000 0000|0 vvv vvvv|

+----------+----------+----------+----------+

Location                  Value

*4 PROGRAM CHANGE

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0011 000 t|tttt tttt |0000 00 bb|0 ppp pppp|

+----------+----------+----------+----------+

Location            Bank  Program No.

b = 00~04

p = 00~7F

*5 CONTROL CHANGE

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0100 000 t|tttt tttt |0 vvv vvvv|0 ccc cccc|

+----------+----------+----------+----------+

Location       Value      Control No.

*6 POLY KEY PRESSURE

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0101 000 t|tttt tttt |0 vvv vvvv|0 kkk kkkk|

+----------+----------+----------+----------+

Location       Value       Key No.

*7 BAR

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0110 00 bb|bbbb bbbb |xx ss ssss|0ppp pppp |

+----------+----------+----------+----------+

Bar No.     Type  Beat    Pattern No.

x = 00 : Pattern not used

= 10 : Pattern continued

= 11 : Pattern start

s = 10~14 : 1/4~5/4

= 20~29 : 1/8~10/8

= 30~3F : 1/16~16/16

*8 END OF TRACK

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|0111 000 t|tttt tttt |0000 00bb |bbbb bbbb |

+----------+----------+----------+----------+

Location             Last Bar No.

BACKING SEQ SEQUENCE DATA ("Control Track")

*9 BACKING CONTROL EVENT

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|10ii iii t|tttt tttt |vvvv vvvv |vvvv vvvv |

+----------+----------+----------+----------+

Event ID  Location    Value L    Value H

+----------------------------------------------------+

|No.|   Event ID       |3rd Byte|4th Byte|Value      |

|----------------------------------------------------|

|  0|Arrangement       | 00~7F  | 00,01  |A11~D88    |

|  1|Style             | 00~7F  | 00,01  |A11~C68    |[Style]

|  2|Style Element     | FF~0A  | 00     |           |-3rd- -4th-

|  3|Full KBD Play     | 00,01  | 00     |Off,On     |00~7F  00  :A11~B88

|  4|Chord Scanning    | 00~03  | 00     |           |00~2F  01  :C11~C68

|  5|Chord Memory      | 00,01  | 00     |Off,On     |

|  6|Bass Inversion    | 00,01  | 00     |Off,On     |[Style Element]

|  7|Transepose        | F5~0B  | 00     |-12~+12    |FF   :Off

|  8|Drums Mute        | 00,01  | 00     |Play,Mute  |00~03:Variation 1~4

|  9|Perc Mute         | 00,01  | 00     |Play,Mute  |04,05:Intro 1,2

| 10|ACC1 Mute         | 00,01  | 00     |Play,Mute  |06,07:Ending 1,2

| 11|ACC2 Mute         | 00,01  | 00     |Play,Mute  |08,09:Fill 1,2

| 12|ACC3 Mute         | 00,01  | 00     |Play,Mute  |0A   :Break

| 13|Main KBD Mute     | 00,01  | 00     |Play,Mute  |

| 14|Sub KBD Mute      | 00,01  | 00     |Play,Mute  |[Chord Scanning]

| 15|Lower KBD Mute    | 00,01  | 00     |Play,Mute  |00:Off    02:Upper

| 16|Main KBD Program  | 00~7F  | 00~04  |A11~R58    |01:Lower  03:FULL

| 17|Sub KBD Program   | 00~7F  | 00~04  |A11~R58    |

| 18|Lower KBD Program | 00~7F  | 00~04  |A11~R58    |[Program]

| 19|Main KBD Octave   | FE~02  | 00     |-2,~+2     |-3rd- -4th-

| 20|Sub KBD Octave    | FE~02  | 00     |-2,~+2     |00~7F  00  :A11~B88

| 21|Lower KBD Octave  | FE~02  | 00     |-2,~+2     |00~7F  01  :C11~D88

| 22|KBD Set           | 00~3F  | 00     |11~88      |00~3F  02  :E11~E88

| 23|Chord Sound       | 00,01  | 00     |Off,On     |00~7F  03  :F11~G88

| 24|Ensemble          | 00,01  | 00     |Off,On     |00~27  04  :R11~R58

| 25|KBD Lock          | 00,01  | 00     |Off,On     |

| 26|Split Point       | 24~60  | 00     |C2~C7      |

+----------------------------------------------------+

BACKING SEQ SEQUENCE DATA ("Chord Track")

*10 CHORD EVENT

DATA(1st)  DATA(2nd)  DATA(3rd)  DATA(4th)

+----------+----------+----------+----------+

|11ii iii t|tttt tttt |nnnn nnnn |bbbb rrrr |

+----------+----------+----------+----------+

Chord ID  Location  Tension Note  Bass  Root

i = 00:No Chord     n = 0000 0001 :b9

01:Major            0000 0010 :9

02:M6               0000 0100 :#9

03:M7               0000 1000 :11

04:M7b5             0001 0000 :#11

05:sus4             0010 0000 :b13

06:sus2             0100 0000 :13

07:M7sus4

08:m

09:m6           b = 0x~Bx :C~B

0A:m7           r = x0~xB :C~B

0B:m7b5

0C:mM7

0D:7

0E:7b5

0F:7sus4

10:dim

11:dimM7

12:aug

13:aug7

14:augM7