KORGLOAD.PRG INSTRUCTIONS  (Ver 1.0)

COMPUTER REQUIREMENT : Any Atari STE, STFM, STF or MEGA.

Mono(Hi Res) or Colour(Med Res) monitor.

Double sided disc drive only.

(Single sided version available on request).

TO RELOAD DATA

==============

1. Connect the MIDI OUT socket on the Atari ST to the MIDI IN

socket on the instrument.

2. Switch on instrument and set basic MIDI channel to 1.

3. Switch on Atari ST. If you are using either a colour monitor

or a colour T.V. set the Atari ST to Med resolution mode.

4. Load KORGLOAD.PRG by double clicking on its icon.

5. When loaded the program will display a Dialogue box.

6. Click on load to display the GEM file selector.

7. Load the desired SYS-EX file by double clicking on it.

WS1PRELD.BNK - Factory preload for Wavestation & Wavestation EX

WSADPL.BNK   - Factory preload for Wavestation A/D Rack

wssrpl.BNK   - Factory preload for Wavestation SR Rack

8. To download the data to the instrument click on transfer

and wait while the MIDI SYS-EX data is transferred.

9. If all goes well click on the EXIT button to return to the

desktop.

10. If you are experiencing a problem try clicking on MIDI TEST.

11. When the MIDI TEST dialogue is displayed click on TEST.

This sends 2 notes a major 3rd apart on channel 1, and 2 notes

a minor 2nd apart on channels 2 to 16.

12. You can also click on the channel numbers to momentarily

output a middle C on that channel.