MIDICARD.CFG - Info file

Both SEND.EXE and ASK.EXE use MIDICARD.CFG file as their configuration

file. File can be configured with Libra's 'Settings' option.

The file contents are:

1     MIDI Card type: 0=dumb MPU UART, 1=smart MPU, 2=GUS

330   MIDI Base address in Hex. 330 should be the default for MPU.

For GUS, if card base address is 220, the midi address

would be 320.

5     IRQ Number for GUS MIDI Port (MPU routines doesn't use IRQ)

0     Count for delay loop in MPU send routines.

Command lines: ([] must contain something, while <> can be left out)

SEND.EXE [SysEx file name] <-q>

ASK.EXE [midi channel (0..15)] [RAM bank (0..11)] [SysEx filename]

RAM Banks are:

0 RAM1

1 RAM2

2 ROM (same as ROM11)

3 CARD

4 RAM3

5 ROM4

6 ROM5

7 ROM6

8 ROM7

9 ROM8

10 ROM9

11 ROM10