list

This file should be called "Xpand/M12 SysEx" on your computer.

Matrix12/ Xpander MIDI Spec

Remote Editing

The Xpander/Matrix-12 remote editing system is designed so that

virtually every action on the front panel is echoed to MIDI, and conversely

any action which can be performed at the front panel can also be

performed remotely from MIDI.

APPENDIX A: MIDI IMPLEMENTATION SUMMARY

Transmitted Data P Channel Voice Messages

Status  Data Bytes      Description

1100 xxxx       0nnn nnnn       Program select. (If enabled).

0nnn nnnn = 0 through 99

Recognized Receive Data P Channel Voice Messages

1000 xxxx       0kkk kkkk       Note Off (See notes 1 & 2)

0vvv vvvv       0vvv vvvv = Note Off Velocity

1001 xxxx       0kkk kkkk       Note On (See notes 1 & 2)

0vvv vvvv       0vvv vvvv = 0: Note Off

0vvv vvvv > 0: Note On Velocity

1011 xxxx       0ccc cccc       Control change (If enabled)

0nnn nnnn       0ccc cccc = Control number

0nnn nnnn = Control value

1100 xxxx       0nnn nnnn       Program select (If enabled).

0nnn nnnn = 0 through 99

1110 xxxx       0nnn nnnn       Pitch Bend change LSB

0nnn nnnn       Pitch Bend change MSB

Recognized Receive Data P Channel Mode Messages

1011 xxxx       7BH     All Notes Off command.

00H     The Xpander turns off all notes that were turned on by

MIDI.

1011 xxxx       7CH     OMNI Mode off.

00H     The Xpander turns off all notes that were turned on by

MIDI.

1011 xxxx       7DH     OMNI Mode on.

00H     The Xpander turns off all notes that were turned on by

MIDI.

1011 xxxx       7EH     MONO Mode on.

00H     The Xpander turns off all notes that were turned on by

MIDI.

1011 xxxx       7FH     POLY Mode on.

00H     The Xpander turns off all notes that were turned on by

MIDI.

Recognized Receive Data P System messages

1111 0110               System Common Message: Tune Request

APPENDIX B: SYSTEM EXCLUSIVE INFORMATION

Transmitted Data P Channel Voice Messages

Status  Data Bytes      Description

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number:

Xpander = 02H  (See note 3)

Matrix-12 = 04H  (Multi Patch dump only)

01H     Command byte 1: Program data dump follows.

0ccc cccc       Command byte 2: Program type

0ccc cccc = 0: Voice data

0ccc cccc = 1: Multi Patch data

0ppp pppp       Command byte 3: Program number

<data>  Program data

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

04H     Command byte 1: Copy

0xxx xxxx       Data:   Lower 7 bits first followed by

0000 000x               most significant bit.

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

07H     Command byte 1: Store

0xxx xxxx       Data: program number.

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

0AH     Command byte 1: Page edit follows

00H

<data>

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

0BH     Command byte 1: Page and subpage select

0ppp pppp       Page number

0ppp pppp       Sub-page number

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

0DH     Command byte 1: Programmer Mode switches

0xxx xxxx       Data:   Lower 7 bits first followed by

0000 000x               most significant bit.

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

0EH     Command byte 1: Up/Down

0000 0xx0       Data: Format =  Set bit 3 = + key

Set bit 2 = P key

F7H     End of System Exclusive status byte (EOX)

Status  Data Bytes      Description

1111 0000       10H     System Exclusive. Oberheim I.D. number

02H     Device number: Xpander = 2

0FH     Command byte 1: Modulation Edit follows

data

F7H     End of System Exclusive status byte (EOX)

Recognized Receive Data

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H (See note 3)

00H     Command byte 1: Program data dump request

0ccc cccc       Command byte 2: Program type

0ccc cccc = 0: Voice data

0ccc cccc > 0: Multi Patch data

0ppp pppp       Command byte 3: Program number

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number:

Xpander = 02H (See note 3)

Matrix-12 = 04H (Multi Patch dump only)

01H     Command byte 1: Program data dump follows.

0ccc cccc       Command byte 2: Program type

0ccc cccc = 0: Voice data

0ccc cccc = 1: Multi Patch data

0ppp pppp       Command byte 3 : Program number

data    Program data

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

02H     Command byte 1: All data dump request

00H     Command byte 2: Voice data

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

02H     Command byte 1: All data dump request

01H     Command byte 2: Multi Patch data (Matrix-12)

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number : Xpander = 02H

04H     Command byte 1: Copy

0xxx xxxx       Data:   Lower 7 bits first followed by

0000 000x               most significant bit.

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0ccc cccc       Command byte 1: Display control command

05H = Xpander, 06H = Matrix-12

<data>  00H = Display control off

01H = 80 bytes of ASCII data follows

02H = Display control on

F7H     End of System Exclusive status byte (EOX)

Status  Data Bytes      Description

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

07H     Command byte 1: Store

0xxx xxxx       Data : Program number

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0AH     Command byte 1: Page edit follows

00H

<data>

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0BH     Command byte 1: Page and Sub-page select

0ppp pppp       Page number

0ppp pppp       Sub-page number

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0DH     Command byte 1: Programmer mode switches

0xxx xxxx       Data:   Lower 7 bits first followed by

0000 000x               most significant bit.

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0EH     Command byte 1: Up/Down

0000 0xx0       Data Format:    Set bit 3 = + key

Set bit 2 = P key

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

0FH     Command byte 1: Modulation edit follows

data

F7H     End of System Exclusive status byte (EOX)

1111 0000       10H     System Exclusive. Oberheim I.D. number

0ddd dddd       Device number: Xpander = 02H

10H     Command byte 1: Card select (Matrix-12 only)

0nnn nnnn       Card number (0,1)

F7H     End of System Exclusive status byte (EOX)

Notes:

1.      xxxx: Basic Channel number minus one (i.e., 0000 is Ch. 1, 0001 is

Ch.2, etc.).

2.       kkkkkkk = Note number.

3.      The Xpander and Matrix -12 both use System Exclusive Device

Number 02H except during Multi Patch data dumps when the Matrix-12

uses 04H.

J

MIDI Implementation Summary

Standard Header Format

F0H

10H

05H

Data Transmission Format

Bulk Data Sizes

Implemented Messages

System Exclusive Messages

00H Q Program Dump Data Request

F0H 10H 02H 00H <patch_type> <program_num> F7H

<patch_type>    = 0 for single patch data

> 0 for multi patch data

<program_num>   program number (0 to 99)

On receipt of this message, an Xpander or Matrix-12 will respond by

transmitting a Program Data Dump Follows message for the requested

patch.

This does not affect the contents of the edit buffer.

01H Q Program Data Dump Follows

F0H 10H <device> 01H <patch_type> <program_num> <data> F7H

<device>                = 2 for all Xpander and Matrix-12 single

= 4 for Matrix-12 multi

<patch_type>    = 0 for single patch data

> 0 for multi patch data

<program_num>   Program number (0 to 99)

<data>          See section on Bulk Data Format

On receipt of this message the data is stored into patch memory at the

specified

location.  This does not affect the contents of the edit buffer.

02H Q All Data Dump Request

F0H 10H 02H 02H 00H F7H

Receipt of this message causes the entire contents of the Xpander  or

Matrix-12 to be dumped in a series of Program Data Dump Follows

messages.  The patches are dumped in sequential ascending order, first

single patches, then multi patches.  Each patch is sent as a separate

complete SysX message bracketed by F0HIF7H.  The contents of the edit

buffer are not affected.

04H Q Copy Voice Data

F0H 10H 02H 04H <id_lo> <id_hi> F7H

<id_lo>         See Definitions for Programmer Mode (0DH),

<id_hi>            but low order 2 bits of id_lo must be 0.

This message may only be used in multi patch mode when a single voice is

currently selected.  The effect is to copy the contents of that voice's edit

buffer to the edit buffers of the specified voices.  Note that this can not

be used to copy an edit buffer from one bank to the other on a Matrix-12.

05H, 06H Q Display Control

F0H 10H 02H <cmd> <disposition> [text] F7H

<cmd>           = 5 for Xpander

= 6 for Matrix-12

<disposition>   = 0   Resume normal display

= 1   Display the following data

[text]          Only present if <disposition> is 1

This message will cas the receiving unit to display up to 80 characters of

ASCII in the center two displays.  All characters must be in the range

20HI5FH ( T U to T_U).

The data is written into a separate buffer which is then displayed.  Each

message re-writes the buffer starting at the first character so that if

fewer than 80 characters are written, data from a previous message may

appear in the display.  If no text is sent before the EOX in a

<disposition>=1 message, the entire previous contents of the message

buffer will be displayed.

<disposition>=0 reverts to normal display.

07H Q Store

F0H 10H 02H 07H <program_num> F7H

<program_num>   program number (0 to 99)

Stores the current edit buffer (single or multi) into the specified patch

location.

In single patch mode this will always store the current edit buffer into a

single patch location.

In multi mode, the effect depends on what is actively selected:  If a multi

patch is selected, then the multi patch edit buffer will be stored.  But if a

single voice within a multi-patch is selected, that voice's edit buffer will

be stored as a single patch.

0AH Q Page Edit Follows

F0H 10H 02H 0AH 00H [ <id> 00 <rot_lo> <rot_hi> <val_lo> <val_hi> ] F7H

<id>            = 00HI05H for actions on with top buttons.

= 08HI0DH for actions on with bottom buttons.

= 18HI1DH for actions on rotary encoders.

<rot> (8 bit value in two bytes, MS bit in d0 of second byte)

Amount that a rotary switch has been turned as an

8 bit, 2's complement number.

<val> (same as <rot>)

Value for controller as 8 bit 2Us complement

number.  Note that flags(e.g. VCO2 Sync) have a

value

of 0 or 1.

In the special case when a modulation is cleared from the modulation

routings page on the Matrix-12, <id> is 0AH, and <val> is 3FH.

0BH Q Page and Subpage Select

F0H 10H 02H 0BH <page> <subpage> F7H

<page>  See section on Page Definitions

<subpage>

This message selects the specified page and sub page.  If this page is not

defined in the current mode, the command will be ignored.

0CH Q Transpose

F0H 10H 02H 0CH <xpose_lo> <xpose_hi> F7

<xpose_lo>              These bytes represent a 8 bit, 2Us complement

<xpose_hi>              number in the range of x24I+24

This sets the Master Transpose of the unit.

0DH Q Programmer Mode Switches

F0H 10H 02H 0DH <id_lo> <id_hi> F7H

<id> (8 bit quantity in two bytes) as follows:

SINGLE  01H (01H 00H)

MULTI   02H (02H 00H)

1/7             04H (04H 00H)

2/8             08H (08H 00H)

3/9             10H (10H 00H)

4/10            20H (20H 00H)

5/11            40H (40H 00H)

6/12            80H (00H 01H)

Note that pressing several buttons simultaneously  produces an <id> that

is the inclusive OR of the appropriate bits.

(Note: if you wish, that changing from SINGLE to MULTI mode or vice-versa

generates a report of the pedal and lever controller values.  Power-On

generates reports of the pedal positions only.  Invoking the "OBERHEIM"

patch via STORE-CLEAR generates controller values and a jump to the

VCO1 page.)

0EH Q Up / Down Switches

F0H 10H 02H 0EH <code> F7H

<code>  = 04 for the T+U key

= 08 for the T-U key

Note that these values have been corrected since the first printing of this

specification.

Changing program numbers with the T+U and T-U keys generates the system

exclusive message above; It does not generate a normal MIDI patch change

message.  Changing program numbers by typing the two digits generates a

MIDI program change message, but not the system exclusive message.

The messages generated are identical in SINGLE, MULTI, and Rselecting

single voices within a MULTI patchS mode.

0FH Q Modulation Edit Follows

F0H 10H 02H 0FH 00H <id> 00 <action> <val_lo> <val_hi> F7H

<id>    = 0-5, where 0  is the first modulation for the page, 1 is the second,

etc.

<action>        specifies               val (8 bits in 2 bytes)

00      Add Modulation  Source: ID of source range 0-26

01      Delete Modulation       N/A

02      Change Source   New source ID range 0-26

03      Set value       Value (unsigned)

04      Dial Value      Amount of change

05      Set Quantize    0 = off, 1 = quantize

06      Toggle Quantize N/A

07      T+U / T-U       0 = T+U, 1 = T-U

10H Q Select Voice Bank (Matrix-12 Only)

F0H 10H 02H 10H <bank> F7H

<bank>  = 0 to select voices 1-6

= 1 to select voices 7 - 12

Bulk Data Format

Because MIDI requires that the most significant bit of bytes transmitted

within a system exclusive message be 0, 8 bit quantities must be

transmitted in two bytes.  In fact, all quantities are transmitted as a pair

of bytes (denoted below as OBWORD), even though most only require the

first byte, and the second byte is thus generally zero.

There are only a few cases where both bytes are used.

Unused bits are sometimes set in uploads, so it is wise to mask out the

unused bits before interpreting the bits from an upload.

There are three different ways used to represent negative numbers: as a

seven-bit twoUs-complement number (all MULTI patch parameters),  as a

six-bit twoUs-complement number (SINGLE patch detune), and as a sign-

magnitude number where bit 6 is the sign and bits 0-5 are the magnitude.

The following assumes that the least significant bit is numbered 0, that

fields are packed left-to-right (K&R p196), and that fields are allocated

into consecutive bytes.

All data structures are shown packed.

typedef struct {

:1 ;                            /* high bit unused

*/

byte val_low : 7 ;      /* low order 7 bits                     */

: 7 ;                   /* only ls bit is used          */

byte val_hi P: 1 ;      /* high order bit of val in d0  */

} OBWORD ;

#define repack(a) a.val_hi<<7 + a.val_lo

Single Patch

struct singlePatch      /* 196 bytes packed, 392 bytes unpacked*/

{

struct vco

{

u63     freq ;  /* Frequency                            */

s31     detune ;        /* Detune

*/

u63     pw ;            /* Pulse Width

*/

u63     vol ;   /* Volume

*/

modfT   mod ;   /* Standard Modulation Flags    */

vcowT   wave ;  /* VCO Wave Shape flags         */

} vco [2] ;

struct vcf

{

u127    freq ;  /* Frequency                            */

u63             res ;   /* Resonance

*/

vcfT    mode ;  /* Filter Mode Code                     */

u63             vca1 ;  /* VCA 1 level

*/

u63             vca2 ;  /* VCA 2 level

*/

modfT   mod ;   /* Standard Modulation Flags    */

} vcf ;

struct fm_lag

{

u63             fm_amp ;        /* FM Amplitude

*/

fmT             fm_dest ;       /* FM destination code

*/

modsT   lag_in ;        /* Lag Input code

*/

u63             lag_rate;       /* Lag Rate

*/

lagT    lag_mode;       /* Lag Mode

*/

} fm_lag ;

struct lfo

{

u63             speed ; /* LFO Speed

*/

trigT   retrig_mode ; /* Retrigger mode         */

lagT    lag ;   /* Lag Flag                             */

waveT   wave ;  /* Wave Shape code                      */

u63             retrig ;        /* Retrigger Point

*/

modsT   sample ;        /* LFO Sample Source            */

u63             amp ;   /* LFO Amplitude

*/

} lfo[5] ;

struct env

{

envfT   flags ; /* Miscellaneous Flags          */

ltrigT  lfotrig ;       /* Triggering LFO code          */

u63             delay ; /* Delay Time

*/

u63             attack ;        /* Attack Time

*/

u63             decay ; /* Decay Time

*/

u63             sustain ;       /* Sustain Level

*/

u63             release ;       /* Release Time

*/

u63             amp ;   /* Envelope Amplitude           */

} env [5] ;

struct track

{

modsT   input ; /* Tracking Generator Source    */

u63             point[5];       /* Mapped vals for Input Pts

*/

} track[3] ;

struct ramp

{

u63             rate ;  /* Ramp Time

*/

rampfT  flags ; /* Miscellaneous Flags          */

ltrigT  lfotrig ;       /* LFO Trigger Code

*/

} ramp[4] ;

struct mod

{

modsT   source ;        /* Modulation Source code

*/

qsmT    amount ;        /* Modulation Amount            */

moddT   dest    ;       /* Modulation Destination

*/

} mod[20] ;

struct name

{

char    character[8]; /* ASCII Characters               */

} name ;

} ;

typedef char u63                /* range 0..63                          */

typedef char u127               /* range 0..127                 */

typedef char s31                /* range -31..+31,6 bit 2Us comp */

typedef char s63                /* range -63..+63                       */

typedef char bool;              /* boolean bit value            */

typedef struct {

:4 ;

bool VIB        :1 ;

bool LEV_1      :1 ;

bool LAG        :1 ;

bool KEYBD      :1 ;

} modfT ;

typedef struct {

:3 ;

bool NOISE      :1 ;

bool SYNC       :1 ;

bool PULSE      :1 ;

bool SAW        :1 ;

bool TRI        :1 ;

} vcowT ;

typedef enum {

LOW_1, LOW_2, LOW_3, LOW_4,

HIGH_1, HIGH_2, HIGH_3,

BAND_2, BAND_4,

NOTCH_2,

PHASE_3,

HIGH_2L, HIGH_3L,

NOTCH_2L,

PHASE_3L

} vcfT ;

typedef enum {FM_VCO, FM_VCF} fmT ;

typedef enum {

KBD, LAG, VEL, RVEL, PRES,

TRK1, TRK2, TRK3,

RMP1, RMP2, RMP3, RMP4,

ENV1, ENV2, ENV3, ENV4, ENV5,

PED1, PED2,

LFO1, LFO2, LFO3, LFO4, LFO5,

VIB, LEV1, LEV2

} modsT ;

typedef struct {

:5 ;

bool EQUAL_TIME :1 ;

bool EXPO               :1 ;

bool LEGATO     :1 ;

} ;

typedef enum {OFF, SINGLE, MULTI, EXTRIG} trigT ;

typedef enum {

TRIANGLE, UP_SAW, DOWN_SAW, SQUARE,

RANDOM, NOISE, SAMPLE

} waveT ;

typedef struct {:7 ; bool LAG :1 ;} lagT ;

typedef struct {

bool FREERUN    :1 ;

bool DADR               :1 ;

bool LFOTRIG    :1 ;

bool EXTRIG     :1 ;

bool GATED              :1 ;

bool MULTI              :1 ;

bool RESET              :1 ;

} envfT ;

typedef enum {LFO1, LFO2, LFO3, LFO4, LFO5, VIB} ltrigT ;

typedef struct {

:4 ;

bool MULTI              :1 ;

bool EXTRIG     :1 ;

bool LFOTRIG    :1 ;

bool GATED              :1 ;

} rampfT ;

typedef struct {

bool quant      :1 ;

bool sign       :1 ;

char amt        :5 ;

} qsm ;

typedef enum {

VCO1_FRQ, VCO1_PW, VCO1_VOL,

VCO2_FRQ, VCO2_PW, VCO2_VOL,

VCF_FRQ, VCF_RES, VCA1_VOL, VCA2_VOL,

LFO1_SPD, LFO1_AMP,

LFO2_SPD, LFO2_AMP,

LFO3_SPD, LFO3_AMP,

LFO4_SPD, LFO4_AMP,

LFO5_SPD, LFO5_AMP,

ENV1_DLY, ENV1_ATK, ENV1_DCY, ENV1_REL, ENV1_AMP,

ENV2_DLY, ENV2_ATK, ENV2_DCY, ENV2_REL, ENV2_AMP,

ENV3_DLY, ENV3_ATK, ENV3_DCY, ENV3_REL, ENV3_AMP,

ENV4_DLY, ENV4_ATK, ENV4_DCY, ENV4_REL, ENV4_AMP,

ENV5_DLY, ENV5_ATK, ENV5_DCY, ENV5_REL, ENV5_AMP,

FM_AMP,LAG_SPD

} moddT ;

Multi Patch - Xpander

struct multi M12Patch {

struct bankXp {

transT  trans[6] ;      /* Transpose                    */

u63             volume[6];      /* Voice Volume         */

panT    pan[6] ;                /* Pan Position         */

s31             detune[6] ;     /* Detune Amount

*/

} bankXp ;

struct vib      {

u63             speed ;         /* Vibrato Speed

*/

lagT    lag ;           /* Vibrato Lag Enable   */

waveT   wave ;          /* Vibrato Wave Shape   */

u63             amp ;           /* Vibrato Amplitude    */

vmodT   speedModSource;/* Speed Modulation Source*/

vmodT   ampModSource ;  /* Amp Modulation Source        */

s63             speedModAmt ;   /* Speed Mod Amount

*/

s63             ampModAmt ;     /* Amp Mod Amount

*/

} vib ;

cmzT    cvmidi[6] ;             /* Voice Assign         */

chanT   zoneInput[3] ;  /* Zone Inputs                  */

struct ZoneLimits {

u127 lowerLimit ;               /* Lower Note Limit

*/

u127 upperLimit ;               /* Upper Note Limit

*/

} ZoneLimits[3] ;

nassT   mode[3] ;               /* Note Assign Modes    */

} ;

Multi Patch - M12

struct multiXpPatch {

struct bankM12 {

transT  trans[6] ;      /* Transpose                    */

u63             volume[6];      /* Voice Volume         */

panT    pan[6] ;                /* Pan Position         */

s31             detune[6] ;     /* Detune Amount

*/

vassT   vassign[6] ;    /* Voice Assignment             */

} bankM12[2] ;

struct vib vib ;

struct zone

{

chanT   channel ;               /* MIDI I/O channel

*/

u127    lowerLimit ;    /* Lower Note Limit             */

u127    upperLimit ;    /* Upper Note Limit             */

nassT   mode ;          /* Note Assignment Mode */

zonefT  flags ;         /* Zone Enables         */

} zone[6] ;

struct name

{

char    character[8] ;  /* ASCII Name                   */

} name ;

} ;

typedef enum {

LEFT, LF2, LF1, MID, RT1, RT2, RIGHT, OFF

} panT ;

typedef enum {

ZONE1, ZONE2, ZONE3, ZONE4, ZONE5, ZONE6,

CHAN1, CHAN2, CHAN3, CHAN4, CHAN6, CHAN6, CHAN7, CHAN8,

CHAN9, CHAN10, CHAN11, CHAN12, CHAN13, CHAN14, CHAN15,

CHAN16

} vassT ;

typedef enum {OFF, LEV2, PED2} vmodT ;

typedef enum {

CHAN1, CHAN2, CHAN3, CHAN4, CHAN5, CHAN6, CHAN7, CHAN8,

CHAN9, CHAN10, CHAN11, CHAN12, CHAN13, CHAN14, CHAN15,

CHAN16, OMNI

} chanT ;

typedef enum {

ROTATE, REASSIGN, RESET, UNI_LOW, UNI_HIGH, UNI_LAST

} nassT ;

typedef struct {

:3 ;

bool CONTROLLERS        :1 ;

bool KEYBOARD           :1 ;

bool VOICE_ROB          :1 ;

bool MIDI_OUT           :1 ;

bool MIDI_IN            :1 ;

} zonefT ;

Page Definitions

#       Name    Page 2

0       Master  x

1       Tune    x

2       Chain

3       Master Misc (M12)

3       Gate (Xp)       x

4       Voice On/Off

5       MIDI

6       Cassette

7       Send Cassette

8       Check Cassette

9       From Cassette

15      Load All

16      Load One

17      Load Single

18      Load Multi

19      Load Chain

20      Start Load

21      Load Global

22      MIDI Basic Channel

23      MIDI Controllers

24      MIDI Enables

25      MIDI Send Patch

26      MIDI Reset

27      MIDI Mute

29      Version x

32      VCO 1   x

33      VCO 2   x

34      VCF / VCA       x

35      FM / Lag        x

40      Env 1   x

41      Env 2   x

42      Env 3   x

43      Env 4   x

44      Env 5   x

48      LFO 1   x

49      LFO 2   x

50      LFO 3   x

51      LFO 4   x

52      LFO 5   x

56      Track 1

57      Track 2

58      Track 3

64      Ramp 1  x

65      Ramp 2  x

66      Ramp 3  x

67      Ramp 4  x

80      Zone 1(M12)     x

81      Zone 2(M12)     x

82      Zone 3(M12)     x

83      Zone 4(M12)     x

84      Zone 5(M12)     x

88      Misc Name(M12)

89      Name(Xp)

96      Transpose

97      Pan

98      VASSIGN / CVMIDI

99      Zone (Xp)

100     Zone Input(Xp)

101     Zone Limits(Xp)

102     Zone Mode(Xp)

103     Volume

104     VIB

105     Name(M12)

106     Detune

107     Mod Routings

thatUs all folks, hope you enjoyed it...

ACTION> (Next, Down, Xm, List)