PC1600 Patch for editing ES1-Software-Synthesizer from Logic

from Thomas Strunden, Seeheim(Germany)

Fader 1:		Switches to OSC-Pulse varying width

Fader 2:		balancing level from OSC and Sub-OSC

Fader 3:		changes LFO-Rate

Fader 4:		Changes filter-cutoff-frequency

Fader 5:		Changes filter-resonance

Fader 6:		Changes filter-drive

Fader 7:		Changes filter-keyfollow

Fader 8:		Changes low-level of ADSR via Vel

Fader 9:		Changes high-level of ADSR via Vel

Fader 10:	Changes envelope-attack

Fader 11:	Changes envelope-decay

Fader 12:	Changes envelope-sustain

Fader 13:	Changes envelope-release

Fader 14:	Changes envelope-form

Fader 15:	Changes low-level of envelope-moulation via velocity

Fader 16:	Changes high-level of envelope-moulation via velocity

Button 1:	Switches between OSC-Wave saw and triangle

Button 2:	Filtermodus 12db

Button 3:	Filtermodus 18db

Button 4:	Filtermodus 24db classic

Button 5:	Filtermodus 24db fat

Button 6:	AGateR

Button 7:	ADSR

Button 8:	GateR

Button 9:	LFO-wave tri

Button 10:	LFO-wave saw down

Button 11:	LFO-wave saw up

Button 12:	LFO-wave square

Button 13:	LFO-wave S&H

Button 14:	LFO-wave random

Button 15:	LFO ext

Button 16:	none

Following functions of ES1 are not supported by this patch: waveforms of Sub-OSC, glide, Int via Wheel (LFO), all functions of the router matrix and the levels of amplifier and also functions only available in controls-appearance. My experience is, that these are functions not often modified when programming ES1-patches.

If somebody is interested to programm these functions, here the controlermap:

glide		#65

Sub-OSC	#81

fusslage		#82

router env	#88

router LFO	#89

int via wheel low	#91

int via wheel high #92

analog feel	#93

bender range	#94

Volume		#95

Number of voices #96

All controler-values are starting with zero, for most sliders the high-level-value is 100, in some cases 127, try it.

If somebody is interested, i also could make an environment for ES1 by switching all knobs by program-changes, so that the routers can be included in one patch ... for example.