Peavey 1600x Presets for Kawai K5000S Synthesizer

0 X-FADES

1 Volume with mute

2 Volume in groups

3 Mod Squad

4 Notes w/Fade Vel

5 NoteTgl/Fade Vel

6 WvSt Crossfaders

7 Prog Change x16

8 Sampling DJ

9 LXP-5 Dly/Pitch

10 LXP-5 Patch Asg

11 Controllers47-62

12 FxCtlScSw+FS1&2    --------	(Effects Control Sources, Destinations & Depths; -

Switch 1&2 Destinations; Footswitch 1&2 Destinations)

13 User 1-4 Assign  	--------	(Assigns User-control Knobs to up-to 2 functions each)

14 K5K Controllers  	--------	(Controller-number outputs for various tasks)

15 K5K 'Knobs & Switches'  -----	(Controller-number outputs of the K5000S Knobs and Switches)

16 K5K Patch Namers

17 K5 dv Gen & DCO

18 Morph Mode

19 K5K MultiFader1

20 K5K MultiFader2

21 K5K Reverb+FX1dv

22 K5K Effects 2&3dv

23 K5K Effects4&EQdv

24 K5 S1-6Vol+LFOdv

25 K5 S1+2Envelopes

26 K5 S3+4Envelopes

27 K5 S5+6Envelopes

28 K5 dv Filter

29 K5 HarmLv  1-16

30 K5 HarmLv 17-32

31 K5 HarmLv 33-48

32 K5 HarmLv 49-64

33 K5 FormLv  1-16

34 K5 FormLv 17-32

35 K5 FormLv 33-48

36 K5 FormLv 49-64

37 K5 FormLv 65-80

38 K5 FormLv 81-96

39 K5 FormLv 97-112

40 K5 FormLv113-128

41 K5 FormantCommon

Sysex for Mute/Unmutes

0 X-FADES

1=Ch12=Ch9 reversed

3=Ch3 reversed

4-16=respective channel numbers

----------------------------------------------------------------------

1 Volumes with mutes

Respective buttons mute/unmute fader above them.

----------------------------------------------------------------------

# 2 Volume in groups

1-12=Volumes with mutes.

13=master of 1-4;

14=master of 5-8;

15=master of 9-12;

16=master of 13, 14 & 15 (thus 1-12).

----------------------------------------------------------------------

3 Mod Squad

faders		1=volume

2=pan

3=mod wheel

4=breath

5=expression

6=foot

7=porttime

8=aftertouch

9=pitch up

10=pitch down

11=GP1 C16

12=GP2 C17

13=GP3 C18

14=GP4 C19

15=bank select

16=prog change (M)   button	16 sends program change

buttons		1=vol max/fader

2=pan center/fader

3-10=mute/unmute fader above

11=sustain

12=sostenuto

13=soft

14=hold 2

15=legato on/off

16=prog change (sends fader 16).

4 Notes w/Fade Vel

buttons 	1-13 notes C4-C514=C315=C216=C1 -- press on, release off

faders	Faders control respective velocity values.

----------------------------------------------------------------------

5 NoteTgl/Fade Vel

faders		Faders control respective velocity values.

buttons 	1-13 notes C4-C514=C315=C216=C1 -- except notes toggle on/off

----------------------------------------------------------------------

6 WvSt Crossfaders

another crossfader experiment using master faders

----------------------------------------------------------------------

7 Prog Change x16

faders		1=bank change in Wavestation

2=channel-1 program change

3-16=channels 3-16 program changes  (channel 2 skipped)

----------------------------------------------------------------------

8 Sampling DJ

faders		1=Master Vol

2=volume(C-out)

3=Expression

4=pan

5=modulation

6=chan press

7=foot

8=reverb depth

9=chorus depth

10=NRPN Sel Lo

11=NRPN Sel Hi

12=data entry

13=prog change

14=glide time

15=pitchbend sens.

16=pitchbend

8 Sampling DJ     Continued

buttons		1=C1 note36

2=D1 note38

3=D#1 note39

4=E1 note40

5=F1 note41

6=G1 note43

7=A1 note45

8=A#1 note46

9=B1 note47

10=Maj7 chord

11=Min7 chord

12=Dom9 chord

13=Dim chord

14=glide on/off

15=chord hold

16=pbend zero

----------------------------------------------------------------------

9 LXP-5 Dly/Pitch

faders		1=delay1 coarse

2=delay1 fine

3=feedback1

4=delay2 coarse

5=delay2 fine

6=feedback2

7=delay3 coarse

8=decay time

9=treble decay

10=bass mult.

11=size

12=diffusion

13=hicut filter

14=low cut filter

15=LFO rate

16=pitch interval

9 LXP-5 Dly/Pitch    continued

buttons		1=delay/reverb

2=pitch/delay

3=pitch base Default

4=pitch base-2

5=pitch base-1

6=pitch base unison

7=pitch match

8-10=none

11=size fader send

12-13=none

14=request current

15=request all

16=bypass

----------------------------------------------------------------------

10 LXP-5 Patch Asg

1,5,9+13=patch 1234 source

2,6,10+14=patch 1234 threshold

3,7,11+15=patch 1234 destination

4,8,12+16=patch 1234 scale

----------------------------------------------------------------------

11 Controllers47-62 (with fader-sending buttons)

general-purpose controllers 47-62 to general (gc) channel

----------------------------------------------------------------------

** 	In the K5K series following, note that only the EFFECTS controls will UPDATE your K5000S screen "live" with new values. You'll have to press a button for the screen to update with the new settings.  (the section button twice, or the more + back buttons work for me...) Your changes -ARE- audible, however, just not updated on the display screen.

The references to "page," "common," etc. refer to the particular K5000s Screen where your changes can be observed (after the button presses in all but Effects).

----------------------------------------------------------------------

12  FxCtlScSw+FS1&2

(Effects Control Sources, Destinations & Depths; Switch 1&2 Destinations;

Footswitch 1&2 Destinations)

faders		1=CtrlSource1

2=CtrlSrc1DEST

3=CtrlSrc1Dpth

4=CtrlSource2

5=CtrlSrc2DEST

6=CtrlSrc2Dpth

7=Switch1DEST

8=Switch2DEST

9=Footsw1DEST

10=Footsw2DEST

11-16=Empty

buttons		none

----------------------------------------------------------------------

13 User 1-4 Assign  	(Assigns User-control Knobs to up-to 2 functions each)

faders		1=User1 DEST1

2=User1 Depth1

3=User1 Dest2

4=User1 Depth2

5=User2 DEST1

6=User2 Depth1

7=User2 Dest2

8=User2 Depth2

9=User3 DEST1

10=User3 Depth1

11=User3 Dest2

12=User3 Depth2

13=User4 DEST1

14=User4 Depth1

15=User4 Dest2

16=User4 Depth2

----------------------------------------------------------------------

14 K5K Controllers  	(Controller-number outputs for various tasks)

(-gc=general channel used for channel number for items 1-12)

fader		1=Volume-gc 7

button		1=FULL VOLUME

fader		2=Mod Wheel 1

button		2=Zero Mod Wheel

fader		3=Pressure

button		3=Zero Pressure

fader		4=Pitch Bend

button		4=Center Pitch Bend

fader		5=Portamento Speed

button		5=Portamento on/off

fader		6=A-Bank Programs Select

button		6=Send A-Bank Program Change

fader		7=D-Bank Programs Select

button		7=Send D-Bank Program Change

fader		8=Multi-Bank Programs Select

button		8=Send Multi-Bank Program Change

fader		9=Expression 11  (Always top this one!!)

button		9=Full Volume Level for Expression

fader		10=Pitch Bend Range 0-24

button		10=Pitch Bend Range = 2

fader		11+12=Empty

button		11=Hold1 Off/On

button		12=Hold2 Off/On

fader		13-16=Volume Channels 1-4 respectively

button		13-16=Full Volume Channels 1-4 respectively

----------------------------------------------------------------------

15 K5K 'Knobs & Switches'	(Controller # outputs of the K5000S Knobs and Switches)

(-gc=general channel used for channel # for items 1-12)

fader		1=Volume-gc 7

button		1=FULL VOLUME

fader		2=Mod Wheel 1

button		2=Zero Mod Wheel

fader		3=Pressure

button		3=Zero Pressure

fader		4=Pitch Bend

button		4=Center Pitch Bend

button		5-12=Empty

15 K5K 'Knobs & Switches'   continued

fader		5=Even/Odd

6=Attack

7=Decay

8=Release

9=Velocity

10=Filter Cutoff

11=Filter Resonance

12=Formant Filter Depth

13-16=User Knobs 1-4 respectively

button		13=Panel Switch 1

14=Panel Switch 2

15=Foot Switch 1

16=Foot Switch 2

----------------------------------------------------------------------

16 K5K Patch Namers

1-8=singles-patch name's letters

9-16=multi-patch name's letters

----------------------------------------------------------------------

17 K5 dv General* & DCO

note: 'fixed key' and several controller settings not included.

faders

Common:

page 2

1=section volume

2=# of sources to use

3=source1-6 mutes/unmutes*

*see bottom for mutes/unmute numbers to use...

4=AM off/src 2-6

Control:

5=bender pitch

6=bend cutoff

7=pan normal value

17 K5 dv General* & DCO   continued

DCO:

page 1

8=PCM-A Wave#

9=DCO coarse tuning

10=DCO fine tuning

page 2

11=pitch envelope start

12=pitch envelope attack time

13=pitch envelope attack level

14=pitch envelope decay time

15=pitch envelope velocity time

16=pitch envelope velocity level

buttons

Common:

page 2

1=poly mode

2=solo1 mode

3=solo2 mode

Control:

4=pan normal

5=pan KS positive

6=pan KS negative

7=pan random

DCO:

page 1

8=ADD Wave Edit mode

9=KS Pitch zero cents

10=KS pitch 25 cents

11=KS pitch 33 cents

12=KS pitch 50 cents

Control:

13=Effects path 1

14=effects path 2

15=effects path 3

16=effects path 4

18 K5K Morf mode

Additive/Morph mode:

faders		1,3,5,9=P1234 source+patch (both on one fader)

2,4,6,8=P1234 Envelope time

9=total gain

10=KS-to-Gain

buttons		1=morph loop off

2=morph loop1 on

3=morph loop2 (undocumented) on

4=harmonic group lo

5=harmonic group hi

6-9=empty

10=KS-to-Gain=zero

11-13=empty

14=Morph EXECUTE!

15=morph off

16=morph on

----------------------------------------------------------------------

19 K5K MultiFader1

faders

Multi mode edit/page 1:

1-4=sections 1-4   volume

5-8=sections 1-4   tone#

9-12=sections 1-4  pan

13-16=sections 1-4 receive midi channel

buttons		1-4=section 1   effects paths 1-4

5-8=section 2   effects paths 1-4

9-12=section 3  effects paths 1-4

13-16=section 4 effects paths 1-4

----------------------------------------------------------------------

20 K5K MultiFader2

faders

Multi mode edit/page 1:

1-4=sections 1-4   transpose

Multi mode edit/page 2:

5-8=sections 1-4   zone low

9-12=sections 1-4  zone high

13-16=sections 1-4 velocity switch value

buttons		1-4=section 1   velocity switch off

5-8=section 2   velocity switch loud

9-12=section 3  velocity switch soft

13-16=none

----------------------------------------------------------------------

21 K5K Reverb+FX1S1

faders		1=Reverb type

2=Reverb dry/wet

3-6=reverb parameters 1-4

7=Effect1 type

8=Effect1 dry/wet

9-12=effect1 parameters 1-4

13=algorithm select

14-16=none

----------------------------------------------------------------------

22 K5K Effects 2&3S1

faders		1=Effect 2 type

2=Effect2 dry/wet

3-6=Effect2 parameters 1-4

7=Effect3 type

8=Effect3 dry/wet

9-12=Effect3 parameters 1-4

13=common volume

14-16=none

buttons		13-16=common mutes/unmute sections 1-4

----------------------------------------------------------------------

23 K5K Effects4&EQS1

faders		1=Effect4 type

2=Effect4 dry/wet

3-6=Effect4 parameters 1-4

7-13=graphic EQ frequency bands 1-7

14-16=none

----------------------------------------------------------------------

24 K5 S1-6Vol+LFOS1

faders		1-6=sections 1-6 volumes mixer

LFO/Copy:

page 1

7=LFO speed

8=LFO delay on

9=LFO fade-in time

10=LFO fade upto speed

page 2

11=LFO pitch depth

12=LFO pitch KS

13=LFO DCF Depth

14=LFO DCF KS

15=LFO Tremelo depth

16=LFO Tremelo KS

buttons		page 1

1-6=none

7=LFO Triangle

8=LFO Square

9=LFO Saw

10=LFO Sine

11=LFO Random

12-16=none

----------------------------------------------------------------------

25 K5 Section1+2Envelopes

26 K5 Section3+4Envelopes

# 27 K5 Section5+6Envelopes

DCA:

page 1

1+8=Velocity curve

2+9=Attack time

3+10=decay1 time

4+11=decay1 level

l5+12=decay2 time

6+13=decay2 level

7+14=release time

15=section mutes/unmute*

16=none

*see bottom for mute/unmute numbers to use...

----------------------------------------------------------------------

28 K5 Section 1 Filter

faders

DCF:

page 1

1=velocity curve

2=resonance

3=filter level

4=filter cutoff

5=common KS depth

6=common velocity depth

page 2

7=envelope attack time

8=envelope decay1 time

9=envelope decay1 level

10=envelope decay2 time

11=envelope decay 2 level

12=envelope release time

page 3

13=velocity-to-envelope depth

14=velocity-to-envelope attack time

15=velocity-to-envelope decay time

16=empty

28 K5 Section 1 Filter    Continued

buttons		page 1

1=filter=hipass

2=filter=lopass

3-14=none

15=filter active

16=filter bypass

note: 'KS to attack' and 'KS to decay' not included...

----------------------------------------------------------------------

29 K5 HarmLv  1-16S1

30 K5 HarmLv 17-32S1

31 K5 HarmLv 33-48S1

32 K5 HarmLv 49-64S1

Additive/Harmonic Level/Harmonic Level Edit page:

**		Four presets control volume levels of harmonics 1-64,

one fader per harmonic..

press section buttons twice to update display screen.

----------------------------------------------------------------------

33 K5 FormLv  1-16S1

34 K5 FormLv 17-32S1

35 K5 FormLv 33-48S1

36 K5 FormLv 49-64S1

37 K5 FormLv 65-80S1

38 K5 FormLv 81-96S1

39 K5 FormLv 97-112S1

40 K5 FormLv113-128S1

Additive/Formant Level/Formant Filter Level Edit page:

**		Eight presets control levels of formant-filter bands 1-128,

one band per fader..

press section buttons twice to update display screen.

----------------------------------------------------------------------

41 K5 FormantCommon

faders

Additive/Formant Level/Formant Filter Level Edit page:

1=formant bias

Additive/Formant ENV/DFE Envelope Edit page:

2=formant envelope depth

3=formant envelope attack rate

4=formant envelope attack level

5=formant envelope decay1 rate

6=formant envelope decay1 level

7=formant envelope decay2 rate

8=formant envelope decay2 level

9=formant envelope release rate

10=formant envelope release level

11=velocity-to-envelope depth

12=KS-to-envelope depth

Additive/Formant ENV/LFO Edit page:

13=LFO speed

14=LFO depth

15-16=empty

buttons		1-8=empty

Additive/Formant ENV/DFE Envelope Edit page:

9=formant envelope mode on (??)

10=formant loop mode off

11=formant loop 1 mode on

12=formant loop 2 mode on

Additive/Formant ENV/LFO Edit page:

13=formant LFO mode on (??)

14=formant LFO=triangle

15=formant LFO=sawtooth

16=formant LFO=random

*press section buttons twice to update display screen.

Mutes/unmutes numbers to use to mute/unmute desired sections 1-6:

0=0000000 none on

1=0000001 1 on

2=0000010 2 on

3=0000011 1&2 on

4=0000100 3 on

5=0000101 1&3 on

6=0000110 2&3

7=0000111 1,2&3

8=0001000 4

9=0001001 1&4

10=0001010 2&4

11=0001011 1,2&4

12=0001100 3&4

13=0001101 1,3&4

14=0001110 2,3&4

15=0001111 1,2,3&4

16=0010000 5

17=0010001 1&5

18=0010010 2&5

19=0010011 1,2&5

20=0010100 3&5

21=0010101 1,3&5

22=0010110 2,3&5

23=0010111 1,2,3&5

24=0011000 4&5

25=0011001 1,4&5

26=0011010 2,4&5

27=0011011 1,2,4&5

28=0011100 3,4&5

29=0011101 1,3,4&5

30=0011110 2,3,4&5

31=0011111 1,2,3,4&5

32=0100000 6

33-63= figure it out...!

----------------------------------------------------------------------