MKS-50/Editor/Utility

By K.MEIERE

If there are any MKS-50 owners out there, here is a program I know you can use.

It is a tone and patch editor program with some features I have seen missing

from other MKS-50 editors.

FEATURES

---------------------------------------------------------------------------

*  Point, click, & drag envelope settings and Sound Testing on same screen

*  Tone editor and Sound Testing on same screen

*  Patch editor for all MIDI functions and Sound Testing on same screen

*  Randomizer with editable mask settings

*  Load and Save randomizer mask files for future use

*  Sound Testing of entire MKS-50 key range from C0 to C8 w/velocities

*  All editing done with mouse

*  Increment/Decrement value changing and wrap-around counters

*  Tone or patch parameter listing to printer

*  User selectable screen colors

*  It's free

-----------------------------------------------------------------------------

I wrote this program because I was tired of spending time pushing buttons for

an hour just to create one tone and also because I'm constantly searching for

new sounds for this great sounding synth.  Editing on any rackmount synth is

not much fun ........... unless you have a computer.  The program is entirely

mouse driven except when using the keyboard to enter new names for tones and

patches.  Help screens are available from the main menu and everything is

fairly straight forward.

TO MAKE IT WORK

---------------

Two files are required to run this program.

MKS_50.PRG & MKS_50.RSC

Hook up TWO MIDI cables since communication will go both ways.

Only works in Medium Resolution Mode

THINGS TO REMEMBER

------------------

1) Before running the program you must set the MKS-50 into the following mode:

----

MIDI CHANNEL =  1     |

MIDI OMNI    = ON     |

MIDI PROG.CG = ON     |

PCH ASG MODE = ON     |------> Accessed Via The TUNE/MIDI Button on MKS-50

TX C.M. APR  = ON     |

TX PATCH APR = ON     |

TX TONE APR  = ON     |

MEM. PROTECT = OFF    |

----

MIDI EXCL    = ON      ------> Accessed Via The PARAM Button in Patch Mode

This program will not work unless these settings are correct!

After setting the MKS-50 into these modes, run the program, the title screen is

displayed, and the tone called I-Beam should appear in the MKS-50 display

window.  You are ready to roll if it looks like -->     T-a11*  I-Beam

2) Sending tones and patches to the editor is easy.  Every time a two digit

number is pressed on the MKS-50 front panel, a series of SysEx numbers are sent

to the editor.  Be sure that when you are editing or sending tones that the

display has the letter 'T' in front of the name like; T-a68 TwiliteZne and when

you are editing or sending patches, the display should have the letter 'P' in

front of the name like; P-A68 TwiliteZne.

3) When editing a tone and you want to rename it, place the mouse pointer over

the words 'TONE NAME' and press the mouse button.  Enter a 10 character name

from the keyboard and press the Return key.

4) This program will do just about everything except permanently WRITE a file

into the MKS-50 tone and patch memory areas.  That is your job and when you get

a tone or patch tweaked just right, remember to press the WRITE button before

starting on another sound or editing a patch.  You will notice that an asterisk

* appears next to a tone or patch name once you start to edit and the display

looks something like, T-a68*TwiliteZne.  This is an indicator to tell you that

a SysEx command has been sent to the MKS-50.  You can use this as a reminder

that the tone or patch in the temporary memory area has been altered but not

saved in memory.  The asterisk will disappear when you select a new tone or

patch to edit.  (Roland did not incorporate a SysEx Write command for

individual tones or patches on the MKS-50 (bummer).  Roland also failed to

include a way to save the MIDI parameter settings inside the MKS-50 (so what

else is new?).  Every time you turn off the synth, factory settings will return

on power-up and you will need to set the MKS-50 according to the list shown

above in step 1).

5) This is a 'real-time' editor which means that when you are in the tone,

patch, or envelope edit mode, every time you press the mouse button to change a

parameter setting, the tone or patch in the temporary memory area is altered to

match what is shown on the ST monitor display.  However, it is still impossible

for you to accidentally over write any existing tones or patches in the MKS-50

permanent memory area using this program.  The only way to over write is by

manually pressing the WRITE button on the MKS-50 front panel.  (Note: When in

the tone edit mode and a tone name is showing in the MKS-50 display window,

pressing the PARAM button will enable you to see the edit as it happens.  There

is no real significance to this quirk but I noticed it one day while using the

program).

6) The degree to which a tone can be randomized is entirely up to you.  I find

it easier to find the 'right' sound by setting various desired random modes and

then turning the VCA ENV (waveform) OFF and also turning T4 OFF.  When in the

tone edit mode, by setting the VCA ENV (waveform) to D|\ (envelope with

dynamics) and T4 to 127, I can quickly step through as many as 40 different

sounds by repetitively clicking on the RANDOM box before the sounds fade away.

The Randomizer Mask files included are just some samples and they demonstrate

how to use the randomizer feature to tweak an existing tone.  Don't always

stick to these samples though.  Experiment with some of your own.

ENVLIZER.RND  -  Generates random envelope settings

WAVEIZER.RND  -  Randomly chooses from the 14 basic waveforms in the MKS-50

VAPRIZER.RND  -  Vaporizers, Plazmoids, UFO's and (?) everything else

(The VCA LEVEL of the Randomizer Mask defaults to the OFF mode but it can be

turned on IF YOU DARE!  Use this one with caution because a first time randomly

generated VCA LEVEL of only 18 can easily turn into a randomly generated VCA

LEVEL of 127 on the second pass.  It's the same effect as turning the volume

knob all the way up if you don't monitor your volume settings.  I know from

experience!  Also, when in patch edit mode, the VOLUME parameter is another

setting to watch out for.)

6) Fail-Safe: If you accidently replace a tone in the temporary edit area of

the MKS-50 by using TX! or by pressing a two digit tone number, all is not

lost.  You can send all of the parameter settings shown on the screen to the

MKS-50 by going into the Edit Randomizer Mask mode and turning all the settings

to OFF.  When you return to the tone edit screen, clicking on the RANDOM box

will send every setting on the screen back to the MKS-50 temporary edit area

and your 'lost' tone is restored.

This is a FreeWare program.  Unlike ShareWare, the FreeWare concept requires

you to send no money, no blank disks, etc....., but you are required to upload

the sounds you create in 'GenPatch', 'Cassette Buster', or any other type of

format!  Thanks.