MANUAL

for

Yamaha DX7 Synthesizer (Mark 1)

This Manual written by:

Clyde Jones

email - jonescnt@ats.com.au

internet - http://www.ats.com.au/~jonescnt

as a service to DX7 owners who either don't have a manual or can't understand the (not always clear) manual supplied by the manufacturer.  Any comments/suggestions welcome as long as they ain't rude.

MIDI

The DX7 seems to basically transmit on MIDI Channel 1 but can be set to receive  playing data on any one of 16 MIDI channels.  Use FUNCTION button and then button 8 (press again and again until the display reads "FUNCTION CONTROL - MIDI CH=   " then use the data entry slider (or YES/NO button) to select the receiving channel.

The DX7 will ONLY receive and transmit voice and parameter data if the CH=1 so set that now!!!

The DX7 will send out the following information via MIDI:

1.	Notes ON/OFF  Pitch Velocity

2.	Pitchbend, Modulation Wheel, Sustain Pedal

3.	Voice number 1 - 32 (internal mode) 33 - 64 (cartridge mode)

IMPORTANT:

While the unit is in the FUNCTION mode, button 8 can be pressed once, twice, three times.  Each time you press it a new function displays... FUNCTION CONTROL - SYS INFO UNAVAIL... FUNCTION CONTROL - MIDI CH=  ... FUNCTION CONTROL - MIDI TRANSMIT?.  Each of these functions can be adjusted using the YES/NO buttons (or the data slider)

Selecting the Receiving Channel

In FUNCTION mode press button 8 till display reads FUNCTION CONTROL - MIDI CH= then enter the MIDI channel number you want to receive on... CH=1 is a really good idea!  If you are using a sequencer and other multi-timbrel synthesisers make the DX7 your MIDI channel 1 instrument = believe me (already) it saves a lot of bother because the DX7 will only receive your sysex commands on MIDI channel 1 anyway.  The MIDI in the DX7 is very basic as MIDI was brand new when the DX7 came out.

System Information (sysex)

Certain types of data can be transmitted between certain groups of instruments of the same manufacturer.  The following types of data can be exchanged using the Yamaha DX7.

1.	Data for one voice or for all voices

2.	The data for a single parameter within a certain voice

3.	The data for a single parameter within the FUNCTIONS

Transmit System Information

A.  To Transmit Single Voice Data

1.	Switch to SYS INFO AVAIL  (Function + 8 + YES)

2.	Press either INTERNAL or CARTRIDGE

3.	Get your sequencer/software ready to receive the data

3.	Press the VOICE NUMBER of the voice you want to send and the data will 	travel from MIDI out to your software.  (a single voice usually arrives as 163         	bytes)

B.	To Receive Single Voice Data

1.	Switch to SYS INFO AVAIL  (Function - 8 - YES)

2.	Set INTERNAL MEMORY PROTECT to OFF (Memory Protect - Internal - 	OFF)

3.	Make sure the MIDI receive channel = 1 (Function - 8 - Data slider)

4.	Select the Voice Number where you want the voice to end up (optional)

5.	Send the voice data from your software

6.	When the single voice data is received the DX7 panel displays the message 	"INTERNAL VOICE" and the received voice name ...and... the first letter of 	the voice name will flash

7.	If you want to keep the voice press and hold STORE and press the number (1-    	32) where you want the voice located and the voice will be saved there

C.	Transmit Voice Data for All 32 Voices at Once

1.	Switch to SYS INFO AVAIL  (Function + 8 + YES)

2.	Press button 8 till the display says "FUNCTION CONTROL - MIDI 	TRANSMIT?"

3.	Get your sequencer/software ready to receive the data

4.	Press the YES button and the data will transmit  (32 voices usually arrive at 	your software as 4104 bytes)

D.	Receive the Voice Data for All 32 Voices at Once

1.	Switch to SYS INFO AVAIL  (Function - 8 - YES)

2.	Set INTERNAL MEMORY PROTECT to OFF (Memory Protect - Internal - 	OFF)

3.	Make sure the MIDI receive channel = 1 (Function - 8 - Data slider)

4.	Send the data from your software

5.	The DX7 panel will display "FUNCTION CONTROL - MIDI RECEIVED"

6.	All 32 voices previously in memory will be lost to the new voices

E.	Transmit Voice or Function Parameters

1.	Switch to SYS INFO AVAIL  (Function + 8 + YES)

2.	Press EDIT or FUNCTION (depending on what type of information you want 	to send)

3.	Press the button that corresponds to the parameter you want to send and the 	data will travel to your software just like the voice data did (much of this 	data will be only 7 bytes long)

F.	Receive Voice or Function Parameters

1.	Switch to SYS INFO AVAIL  (Function - 8 - YES)

2.	Send the parameter data from your software

3.	When receiving this data the DX7 will vary the data for that parameter

There are a few good shareware programs around that are very useful if you have an IBM compatible computer with a MIDI interface and a DX7 - software that will help you create, store, data base and organise voices.  See my web page (Clyde's Midi Bus Stop at http://www.ats.com.au/~jonescnt)to find them and download them.