Hi folks,

In this directory you'll find the IBM PC executable file, DX2SY.EXE.

I got this from Compuserve (via Scott Amspoker - thanks Scott).  It is

apparently distributed by Yamaha.

The program converts complete DX7 32 voice bulk dumps to SY77 voice

files.  At the moment, I haven't had time to completely test the

program, but I have used it to convert a DX7 dump to a file that looks

like the SY77 might be able to use it.

Anyways, here's the recipe for use:

Ingredients:      1 SY77

1 IBM PC or compatible with 3.4" disc drive.

1 DX7 32 voice bulk dump (see * below)

1 3.5" disc

1 DX2SY.EXE (from this directory)

Method:

1.  Get the DX2SY.EXE file onto your PC.

2.  Get the DX7 bulk voice dump onto your PC.

3.  Run DX2SY.  It'll prompt for input and output filenames.

4.  Wait while the program munges the file.  If it says

something like 'illegal format', refer to (*) below.

5.  Copy the file onto your SY77 disc, using a file name that

the machine will recognise as a 'Synth All' file (I think

the extension is something like .T00, .T01, but I'm not sure.

This is the bit I haven't had time to do yet.)

6.  Put the disc in your SY77, and try to retrieve the file.

(*) The problem I had here was the lack of a header on the 4096 byte

long files I got from the DX7 archive on ucsd.edu.  So, you'll find a

little program in this directory (addit.c) that you can compile up to

add the headers onto these bulk dumps.  Feed the resultant file into

DX2SY, and everything should be okay.  [I've not included a .EXE of

addit.c, 'cos the one I have says 'stack fault' when I run it.  The

program works fine on my HP-UX system, and the code looks okay.  If

anyone compiles up a .EXE that works, you could place it here for

everyone else to use.]

I you have any problems, feel free to get in touch with me.

Mike Prudence (mjp@hplb.hpl.hp.com)

A satisfied SY77 owner.