Installation of TG33 Editor ver 1.20

February 15, 1997

Matt Arend

Windows 95 Installation

To install TG33 editor simply use Explorer create a directory and then copy the files from the diskt to your new directory.  Either drag the TG33.EXE icon to the desktop or add it to the "start" menu.

Setup

To use the program, first make sure that your MIDI SETUP parameters are correct by setting your MIDI card in the Midi/Setup Dialog box.  Don't forget to set the DEVICE ID to the same value for both the TG33 editor and the TG33 (or SY33/35).  You need to select either SY or TG for the Midi Setup dialog box to set the correct file transmit and save format correctly.  Note:  errors may result during transmit if this is set incorrectly.

SY22/35 and TG33 Differences and Notes

There are important differences between the SY and TG series of Yamaha instruments.  Please read the README.WRI file of us the TG33 on-line help to read about these differences.  ]

The TG33 Editor does NOT download from the TG33 (or SY22/35).  If you have sounds stored in your unit, you will have to use either your sequencer or another program to first get them from your instrument to your computer in a "bulk dump" format.  TG33 Editor will read standard buld dump format from either the SY22/35 or TG33 (single voice or voice bank).  The editor will save to file either SY or TG format depending on what option is set in the Midi/Setup dialog box.  SY voices with multi information is transmitted to the SY and re-saved IF the SY option is set in the Midi/Setup dialog box otherwise the multi information will be discarded (the TG33 does not combine multi information with voice information).

SY22/35 series do NOT respond to edit changes in real-time (this is a Yamaha design limitation).  To see the changes, the user must press the SY22/35 >> buttons.  TG33 series DOES respond in realtime edit except for the FM Modulator Envelope and FM Mod/Carrier Waves (for these parameters the user must press the SY22/35 >> buttons for the changes to take effect).  This is also a Yamaha design limitation.

Disclaimer

Users of TG33 Editor  must accept this disclaimer of warranty: TG33 Editor is supplied as is. I disclaim all warranties, expressed or implied,  including, without limitation, the warranties of merchantability and of fitness for any purpose.  I assume no liability for damages, direct or consequential, which may result from the use of TG33 Editor.

Distribution

Individuals are granted permission to copy this package for their  own use, or for backup purposes only. These files are:

TG33.EXE	 	 TG33 Editor Program

TG33.HLP		 Windows Help File for TG33 Editor

AWMPARAMS.DAT	 Preset data for the AWM Waves

FMPARAMS.DAT	 Preset data for the FM Waves

MUSCROLL.DLL	 Custom Spin Control

CTRL3D.DLL		 3D Control DLL

README.WRI		 Latest changes, etc.